3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415634

# Hefei, Anhui, China, 14th – 18th October, 2024

Title: 	Discussion on 8Rx performance requirements with MMSE-IRC

Source: 	Huawei, HiSilicon

Agenda item:	6.16.2

Document for:	Discussion

Background

In last meeting, WF R4-2413570 [1] was approved, one objective is related to 8Rx requirements under interference scenario, this contribution provides our views.

- Discussions

General

Test scope

One open issue is whether to define CQI reporting requirements with ICI. Based on our understanding, CQI requirements with ICI has been covered in the WI scope and RAN4 should work with reference to the WI. Furthermore, CQI reporting is a key part of baseband processing, which should be verified. One point is that there is no CQI requirements under fading channel in the current spec, to guarantee the completeness of spec, we propose to define that requirement in Rel-19. However, considering the timeline limitation, RAN4 could focus on inter-cell interference as 1st priority and go back to fading channel if times allows.

Proposal 1: RAN4 to define two different CQI requirements in Rel-19:

CQI requirements with inter-cell interference (1st priority)

CQI requirements under fading channel (If time allows)

8Rx UE MMSE-IRC receiver assumption

- Two different receiver assumptions have been defined in the current spec, therefore, it is unreasonable that no test cases are defined for simplified receiver in Rel-19. From performance perspective, the performance gap between baseline receiver and simplified receiver is very large for some cases based on our simulation results in Table 2-1(ICI) and Table 2-2(MU-MIMO), that means two set requirements should be defined to distinguish them. RAN4 should cover all main implementations when defining performance requirements.

Proposal 2: RAN4 to define PDSCH requirements for both baseline receiver and simplified receiver under ICI and MU-MIMO scenario

Antenna correlation

- In Rel-18 8Rx WI, Medium B was applied for rank2 test cases. However, unlike white noise scenario, increasing antenna correlation will reduce spatial freedom degree to mitigate colour interference and reduce the performance gain of 8Rx over 4Rx. Hence, it’s proposed to only consider low correlation for all Rel-19 test.

Proposal 3: RAN4 to only consider low correlation for all tests.

PDSCH mapping type

Some companies proposed to cover PDSCH mapping type B for requirements definition, based on our understanding, PDSCH mapping type B is introduced for URLLC scenario specifically which has never been considered for demodulation requirements before. We don’t see any motivation to introduce this feature in this WI.

Proposal 4: RAN4 to only consider PDSCH mapping type A.

PDSCH requirements with inter cell interference

Interference modelling and power levels

For interference modelling, some companies propose to reuse 2Rx/4Rx interference modelling, which cover both HomNet and HetNet scenarios. However, according to our simulation results in Table 2-1, sufficient performance gain can be observed for HomNet scenario (2 interference cells), so test purpose is can be satisfied. Covering HetNet scenario will bring additional test cost without any benefit.

Proposal 5: RAN4 to only consider HomNet scenario for PDSCH requirements definition.

Propagation condition

Two traditional channel models are to be selected: TDLA30-10 and TDL300-100, MMSE-IRC receiver is vulnerable to high selective channel and based on our simulation results, the MMSE-IRC performance gain is slight larger for TDLA30-10 compared to TDLC300-100, we prefer to consider TDLA30-10 for requirements definition.

Proposal 6: RAN4 to only consider TDLA30-10.

MIMO layers, MCS and antenna configurations

We propose the following logic: Define one set of test setup for both receiver types. The performance gain over MMSE-IRC should be guaranteed and the final SINR should be in reasonable range. Additionally, it’s proposed to choose rank4 considering 8Rx have high spatial freedom degree. Based on our simulation results, Rank4+MCS13+TDLA30-10 is a good choice, for this case, the performance gain for baseline receiver is 5.1dB and 2.0dB for simplified receiver, which is enough.

Proposal 7: RAN4 to consider following test setup for inter-cell interference scenario:

Rank4

MCS13

TDLA30-10

CQI requirements with inter cell interference

Rank

We have stated our views in [4], it’s proposed to consider Rank 2 instead of Rank1 and Rank4. Firstly, we don’t think Rank1 is proper for high-freedom 8Rx UE in the real deployment, so higher rank should be considered. We provided the simulation results for rank2 and rank4 respectively in [4], it can be observed that case of rank2 has more SINR points with considerable gain, which looks more reasonable than rank4.

Proposal 8: RAN4 to consider Rank2 for CQI requirements with inter cell interference.

CQI calculation algorithm

Same as demodulation processing, there could be two different CQI calculation assumptions:

- Baseline CQI calculation: UE perform CQI calculation with 8Rx joint processing

- Simplified CQI calculation: UE perform CQI calculation with separate 4Rx processing

Based on our understanding, CQI calculation algorithm should be consistent with demodulation algorithm, otherwise, performance degradation may occur. For example, if demodulation is based on 8Rx joint processing but CSI calculation is based on two separate 4Rx joint processing, CQI derived by CSI-RS estimation may be underestimated and absolute throughput may be lower than it should be. Therefore, CQI calculation algorithm is expected to match to demodulation algorithm. We did some simulations for both two receiver types: Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing. Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing. It’s observed that in most SNR points γfor baseline receiver is higher than simplified receiver. The reason is that baseline receiver has higher interference suppression capability, given the same total power of interference and white noise, baseline receiver can cancel more interference. Therefore, we propose to define CSI requirements for such two receiver types.

Figure 2-1 CQI test results for baseline receiver and simplified receiver.

Proposal 9: RAN4 to define CQI requirements for two receiver types:

Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing.

Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing.

PDSCH requirements with MU-MIMO

As proposed above, we prefer to define one set of test setup for both baseline receiver and simplified receiver. However, simplified receiver supports up to 4 layers. Hence, it’s proposed to only consider Rank2+2. Based on our initial simulation results in Appendix, for MCS19, 70% of max throughput is unachievable, which can better verify the performance gain of MMSE-IRC. Hence, it’s proposed to consider MCS19. For other test setup, we propose to reuse Rel-17 MU-MIMO test setup.

Proposal 10: RAN4 to consider following test setup for PDSCH requirements with MU-MIMO:

Number of UE: 1 serving UE + 1 interference UE

Rank allocation: Rank 2+2

MCS: MCS19

Propagation conditions: TDLA30-10

Orthogonal PMI for serving UE and interference UE

Applicability rule and release independent

Test applicability

We have following options for test applicability rules:

- Candidate options: - Option 1: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule - Option 2: Applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined

We agree that MMSE-IRC is baseline feature which could be mandatory for 8Rx UE without additional test applicability rule. Additionally, if 8Rx UE passes the 8Rx test, 2Rx/4Rx test can be skipped regardless the test configurations since it tends to agree higher rank for ICI and MU-MIMO test and reuse interference model. Furthermore, 8Rx Ruu calculation is much complicate than 4Rx/2Rx, UE doesn’t need to pass test cases with simper implementation if has passed cases with complicate implementation.

Proposal 11: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule. Applicability rule to skip 2/4Rx requirements if 8Rx tests are passed regardless the concrete test setup.

Release independent

One open issue is about release independent.

- Candidate options: - Option 1: The new requirements for 8Rx should be release independent from Rel-18 - Other options are not precluded

Noted that supporting 8Rx is introduced in Rel-18 but release independent from Rel-17. Considering there is short period between Rel-18 and Rel-19 and MMSE-IRC receiver is basic feature which is always be assumed as baseline receiver for demodulation requirements. It’s reasonable to define the new requirements release independent from Rel-18. To be conservative, the requirements could be optional for Rel-17.

Proposal 12:  The new requirements for 8Rx should be mandatory and release independent from Rel-18 and optional and release independent from Rel-17.

The performance requirements for 8Rx should be release independent from Rel-17, but should be mandatory for Rel-18 and optional for Rel-17.

- Conclusion

- In this paper, we provide our views on 8Rx requirements with interference. The proposals are:

- General:

Proposal 1: RAN4 to define two different CQI requirements in Rel-19:

CQI requirements with inter-cell interference (1st priority)

CQI requirements under fading channel (If time allows)

Proposal 2: RAN4 to define PDSCH requirements for both baseline receiver and simplified receiver under ICI and MU-MIMO scenario

Proposal 3: RAN4 to only consider low correlation for all tests.

Proposal 4: RAN4 to only consider PDSCH mapping type A.

PDSCH requirements with inter cell interference:

Proposal 5: RAN4 to only consider HomNet scenario for PDSCH requirements definition.

Proposal 6: RAN4 to only consider TDLA30-10.

Proposal 7: RAN4 to consider following test setup for inter-cell interference scenario:

Rank4

MCS13

TDLA30-10

Proposal 8: RAN4 to consider Rank2 for CQI requirements with inter cell interference.

Proposal 9: RAN4 to define CQI requirements for two receiver types:

Baseline receiver: UE performs demodulation and CSI calculation with 8Rx joint processing.

Simplified receiver: UE performs demodulation and CSI calculation with two separate 4Rx processing.

PDSCH requirements with MU-MIMO

Proposal 10: RAN4 to consider following test setup for PDSCH requirements with MU-MIMO:

Number of UE: 1 serving UE + 1 interference UE

Rank allocation: Rank 2+2

MCS: MCS19

Propagation conditions: TDLA30-10

Orthogonal PMI for serving UE and interference UE

Applicability rules:

Proposal 11: The new requirements for 8Rx MMSE-IRC receiver should be mandatory requirements for all 8Rx UEs without additional test applicability rule. Applicability rule to skip 2/4Rx requirements if 8Rx tests are passed regardless the concrete test setup.

Proposal 12:  The new requirements for 8Rx should be mandatory and release independent from Rel-18 and optional and release independent from Rel-17.

- Appendix

Table 4-1 and Figure 4-1 provides simulation results for 8Rx with inter-cell interference.

- Table 4-1: Summary of simulation results for 8Rx with inter-cell interference

| Propagation conditions | Rank | MCS |  | Baseline receiver |  |  | Simplified receiver |  |  | Performance difference (Baseline receiver vs Simplified receiver) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | MMSE-IRC | MMSE-MRC | Gain | MMSE-IRC | MMSE-MRC | Gain |  |
| TDLA30-10 | Rank2 |  | MCS13 | 4.2 | 9.3 | 5.1 | 6.5 | 10.0 | 3.5 | 2.3 |
|  |  |  | MCS19 | 8.9 | 13.8 | 4.9 | 11.6 | 14.9 | 3.3 | 2.7 |
|  | Rank4 |  | MCS13 | 8.6 | 13.7 | 5.1 | 14.2 | 16.2 | 2.0 | 5.6 |
|  |  |  | MCS17 | 12.1 | 17.0 | 4.9 | 19.6 | 21.1 | 1.5 | 7.5 |
|  |  |  | MCS19 | 13.6 | 18.5 | 4.9 | 21.8 | 23.0 | 1.2 | 8.2 |
| TDLC300-100 | Rank2 |  | MCS13 | 6.1 | 11.1 | 5.0 | 8.2 | 11.6 | 3.4 | 2.1 |
|  |  |  | MCS19 | 11.2 | 15.8 | 4.6 | 13.8 | 16.9 | 3.1 | 2.6 |
|  | Rank4 |  | MCS13 | 10.8 | 15.5 | 4.7 | 16.5 | 18.3 | 1.8 | 5.7 |
|  |  |  | MCS17 | 14.6 | 19.2 | 4.6 | 22.4 | 23.5 | 1.1 | 7.8 |
|  |  |  | MCS19 | 16.4 | 20.8 | 4.4 | 24.7 | 25.5 | 0.8 | 8.3 |

Figure 4-1: Simulation results for PDSCH requirements with inter cell interference

Table 4-2 and Figure 4-2 provides simulation results for 8Rx with MU-MIMO.

- Table 4-2: Summary of simulation results for 8Rx with MU-MIMO

|  |  | Baseline receiver |  |  | Simplified receiver |  |  | Performance loss |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MMSE-IRC | MMSE-MRC | Gain | MMSE-IRC | MMSE-MRC | Gain |  |
| Rank2+2 | MCS13 | 6.0 | 11.7 | 5.7 | 7.9 | 17.4 | 6.5 | 1.9 |
|  | MCS19 | 10.8 | Inf | Inf | 14.2 | Inf | Inf | 3.4 |

Figure 4-2: Simulation results for PDSCH requirements with MU-MIMO

- Reference

[1]  R4-2413570 WF on NR demodulation performance: Phase 5.

[2]	 RWS-210003 eMBB PHY Enhancements for Rel-18, Qualcomm

[3]	 RP-231928 MIMO evolution in Rel-19, Huawei, HiSilicon

[4]  R4-2412763 Views on 8Rx requirements with interference, Huawei, HiSilicon