3GPP TSG-RAN WG4 Meeting#112-bis		R4-2415640

Hefei, China, Oct 14th -18th, 2024

Agenda item:	6.1.1.3.1

Source: 	Spreadtrum Communications

Title: 	Discussion on reference sensitivity requirements for 6RX UE

Document for:	Discussion

Introduction

In RAN4#112 meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

General considerations for specifying ΔRIB,6R value

Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case Agreement: Include n104 in the high band (n77, n78, n79) category. Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Agreement: Different value for handheld UE and FWA. Issue 1-1-3: Verification of 6Rx receiver requirements Way Forward: Companies encouraged to provide their views on the ZTE proposal in Proposal 5 in R4-2411883 and any further positions on verification of 6Rx receiver requirements at future meetings given that ΔRIB,6R values are now defined.

ZTE’s proposal is that when the UE is equipped with six Rx antenna ports: For single carrier RFESENS requirements, additional requirement for four RX antenna ports and six Rx antenna ports shall be verified in operating bands; For Rx requirements other than single carrier REFSENS, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands. In our understanding, it is reasonable to use the same method with 4Rx/8Rx for 6Rx. At the same time, it could decrease test burden and save test time.

Proposal 1: Support ZTE’s proposal about verification of 6Rx receiver requirements.

- For single carrier RFESENS requirements, additional requirement for four RX antenna ports and six Rx antenna ports shall be verified in operating bands.

- For Rx requirements other than single carrier REFSENS, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands

ΔRIB,6R value for handheld UE and FWA

Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Agreement: Based on the outcome of Issue 1-1-2, RAN4 to specify ΔRIB,6R values for handheld UE and FWA as follows. - Adopt the average value of ΔRIB,6R for handheld UE from company proposals (updated to include company proposals from RAN4#111) and specify it for handheld UE devices. Table: Averaged values for handheld UE based on proposals Operating Bands; ΔRIB,6R (dB). MediaTek; Band n41; -3.4. Band n77, n78, n79, n104; -3.1. Xiaomi; Band n41; -3.2. Band n77, n78, n79, n104; -3.0. LGE; Band n41; -3.5. Band n77, n78, n79, n104; -3.0. Spreadtrum; Band n41; -3.3. Band n77, n78, n79, n104; -3.0. Meta; Band n41; -3.5. Band n77, n78, n79, n104; -3.1. vivo #1; Band n41; -3.0. Band n77, n78, n79, n104; -3.0. vivo #2; Band n41; -3.2. Band n77, n78, n79, n104; -3.0. ZTE; Band n41; -3.5. Band n77, n78, n79, n104; -3.0. Ericsson; Band n41; -3.6. Band n77, n78, n79, n104; -3.2. Google; Band n41; -3.4. Band n77, n78, n79; -3.0. Huawei; Band n41; -3.3. Band n77, n78, n79, n104; -3.0. Qualcomm; Band n41; -3.4. Band n77, n78, n79, n104; -3.0. Apple (R4-2407071); Band n41; -3.3. Band n77, n78, n79, n104; -3.0. OPPO (R4-2408759); Band n41; -3.6. Band n77, n78, n79, n104; -3.2. Average; Band n41; -3.4. Band n77, n78, n79, n104; -3.0. - Adopt the average value of ΔRIB,6R for FWA UE from company proposals indicating different FWA UE values from handheld UE values and specify it for FWA UE devices. Table: Averaged values for FWA UE based on proposals Operating Bands; ΔRIB,6R (dB). LGE; Band n41; -3.7. Band n77, n78, n79, n104; -3.4. Spreadtrum; Band n41; -3.6. Band n77, n78, n79, n104; -3.2. vivo; Band n41; -3.4. Band n77, n78, n79, n104; -3.2. Google; Band n41; -3.7. Band n77, n78, n79; -3.3. Nokia; Band n41; -4.0. Band n77, n78, n79, n104; -3.4. Average; Band n41; -3.7. Band n77, n78, n79, n104; -3.3.

The number of antenna for 6RX is three times that of 2RX, thus the diversity gain of 6RX is 10log3=4.77 dB higher than for 2RX. In theory, the diversity gain of 6RX is 10log1.5=1.76dB for 4RX and the diversity gain of 8RX is 10log1.33=1.25dB for 6RX. But practically, we should consider the impact of coupling and antenna correlation, which will be more severe than 4RX but less severe than 8RX. Based on the current specification in TS 38.101-1 table 7.3.2-2 and Table 7.3.2-2a, in our understanding, ΔRIB,6R  of FWA/CPE/vehicle/industrial devices is -3.2dB for n77/n78/n79/n104.But n41 has lower frequencies than n77/n78/n79, we could separately consider this question. ΔRIB,6R is -3.6dB for n41. In addition, we think that handheld UE could be lower 0.3dB compared with CPE/FWA/vehicle/industrial devices for n41 and could be lower 0.2dB compared with CPE/FWA/vehicle/industrial devices for n77/n78/n79/n104. In view of the above, we could take the table 1 into consideration.

Proposal 2: Adopt the value of ΔRIB,6R  in Table1 for n41/n77/n78/n79/n104 for handheld UE and FWA.

Table1: Six antenna port reference sensitivity allowance ΔRIB,6R

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.61,-3.32 |
| n77, n78, n79,n104 | -3.21,-3.02 |
| NOTE 1: When 6 Rx operation is supported by FWA form factor. NOTE 2: When 6 Rx operation is supported by handheld UE. |  |

Proposal 3: We accept that adopt the average value of ΔRIB,6R from company proposals for handheld UE and FWA.

Conclusions

In this paper, we provide our views on 6RX UE RF requirements, we have made the following proposals.

Proposal 1: Support ZTE’s proposal about verification of 6Rx receiver requirements.

- For single carrier RFESENS requirements, additional requirement for four RX antenna ports and six Rx antenna ports shall be verified in operating bands.

- For Rx requirements other than single carrier REFSENS, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands

Proposal 2: Adopt the value of ΔRIB,6R  in Table1 for n41/n77/n78/n79/n104 for handheld UE and FWA.

Table1: Six antenna port reference sensitivity allowance ΔRIB,6R

| Operating band | ΔRIB,6R (dB) |
| --- | --- |
| n41 | -3.61,-3.32 |
| n77, n78, n79,n104 | -3.21,-3.02 |
| NOTE 1: When 6 Rx operation is supported by FWA form factor. NOTE 2: When 6 Rx operation is supported by handheld UE. |  |

Proposal 3: We accept that adopt the average value of ΔRIB,6R from company proposals for handheld UE and FWA.

- Reference

- R4-2414285, WF on requirements for 6RX, AT&T.TSG-RAN4#112 meeting.