3GPP TSG-RAN WG4 Meeting#112bis							         		R4-2415644

Hefei, China, 14 – 18 October 2024

Title: 	Preliminary input on 2Tx PC1.5 contiguous ULCA MPR based on measurements

Source: 	Skyworks Solutions, Inc.

Agenda Item:	6.1.1.1.2 Intra-band contiguous and non-contiguous UL CA with PC1.5 [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN#112, we discussed PC1.5 intra-band contiguous ULCA in [1] and the emissions issues related to PSD imbalance for ULCA in [2] and our proposal to evaluate MPR for up to 6dB PSD imbalance was captured in way forward [3]. In this contribution, we provide some preliminary measurements results that compare 2Tx PC1.5 contiguous ULCA required backoff with the 1Tx PC2 case, while also assessing the impact of supporting some level of PSD imbalance as proposed in [4].

# Discussion

## 2.1 Way forward inputs

The way forward [2] captured our input on PSD imbalance in [1] in the following agreement related to contiguous ULCA in italics.

Online agreement:

- MPR and CANS_04 A-MPR studies for PC1.5 contiguous intra-band ULCA focusses on TxD architecture and may account for PSD imbalance with up to 6dB.

  - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB

- PC1.5 contiguous intra-band ULCA based on dualPA architecture with two LOs is not specified in R19

Furthermore, it also discussed ULCA PCmax aspects similar to what was done in previous meetings within the following agreements in italic.

Adhoc agreement:

- For intra-band UL contiguous CA with TxD, the upper bound of Pcmax,c is 29dBm per CC

  - Whether or how to reflect the above agreement in spec is FFS

Online agreement with adhoc modification:

- For intra-band contiguous ULCA with 2Tx-TxD, the upper bound of Pcmax is 29dBm

In this contribution, we further evaluate the PC1.5 contiguous ULCA MPR based on measurements and the above agreements:

- Evaluate 2Tx architecture only and compare it to the closest already specified case of 1Tx PC2.

- Evaluate the impact of up to 6dB PSD imbalance.

## 2.2 Measurement description.

Note that this evaluation is not yet meant to provide MPR value proposals as:

- It has a limited set of waveforms: only DFT-s-OFDM QPSK are evaluated with a set of corner waveforms.

- It does not account for limitations of ET or APT implementations at large back-offs (these would require an assessment at lower voltages as the back-off increases resulting in a “slower” improvement of the IMDs with back-off

- The measurements focus on assessing two aspects to provide some insight on the MPR evolution:

  - How 2Tx PC1.5 using two PC2 PAs compares with 1Tx PC2 using one PC2 PA

  - How PSD imbalance impacts MPR.

Measurement parameters:

- 2Tx PC1.5 measurements:

  - Uses two band n77 PAs calibrated MPR1 at 31dB ACLR for 20MHz QPSK DFT-s-OFDM 100RB0 waveform.

  - Coupled at 10dB and with 3dB post PA losses.

  - The measured output of each PA is summed mathematically for PC1.5 power capability.

- 1Tx PC2 measurements:

  - Uses the same two band n77 PAs calibrated MPR1 at 31dB ACLR for 20MHz QPSK DFT-s-OFDM 100RB0 waveform.

  - Coupled at 50dB and with 3dB post PA losses such that it corresponds to two 1Tx PC2 measurement.

  - The measured output of each PA is evaluated separately, and the worst case is kept.

- Configurations and power

  - 2x80MHz contiguous ULCA is configured.

  - Only 30kHz SCS DFT-S-OFDM waveforms are measured.

  - Each CC is allocated with permutations of 1, 15, 108 and 216RBs

  - RB positions are such that the following scenarios are tested:

    - Contiguous inner allocation

    - Contiguous outer allocation

    - Non-contiguous inner allocation

    - Non-contiguous outer1 allocation

    - Non-contiguous outer2 allocation

    - Resulting in 25 allocation permutations.

  - Different power balance cases are tested:

    - For 1Tx PC2 and 2Tx PC1.5: Equal PSD (=equal power when the same allocation is used in both)

    - For 2Tx PC1.5 only: 6dB PSD imbalance (2 cases depending on which CC has the lower PSD, in some cases the 6dB PSD imbalance results in equal power)

    - For 2Tx PC1.5 only: Some equal power corner cases are evaluated to further evaluate the impact of PSD imbalances.

  - Measurements only at the calibration voltage (does not account for ET APT) with back offs up to 8dB.

  - Measured parameters:

    - Total power, power per CC

    - ACLR

    - SEM

    - IMD3 and IMD5.

- Overall, 844 configurations and power levels were tested with close to 15000 measurements resulting in 6 days of continuous automated testing.

Observation:

- Even with such a limited waveform set the test time is prohibitive.

- To perform a fully representative set of waveforms, CA configurations, allocation types and accounting for PSD imbalances and APT/ET operation at large backoff will require weeks of measurements:

- Hopefully the set may be reduced based on selecting corner cases from simulations.

- Otherwise, patience will be needed for a complete MPR proposal, or it is evaluated as a delta versus PC2 1Tx with a limited set of waveforms.

## 2.3 2Tx PC1.5 contiguous ULCA MPR versus existing specification.

To have a first insights on the 2Tx PC1.5 MPR, it can be compared with the existing 1Tx and 2Tx PC2 cases, which MPR is copied here for QPSK and both contiguous on non-contiguous allocations:

Table 1: PC2 MPR for bandwidth class C

| MPR (dB) |  | Contiguous allocations |  |  |  | Non-ontiguous allocations |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation |  | 1Tx PC2 |  | 2Tx PC2 |  | 1Tx PC2 |  |  | 2Tx PC2 |  |  |
|  |  | inner | outer | inner | outer | inner | Outer11 | Outer22 | inner | Outer11 | Outer22 |
| DFT-s-OFDM | QPSK | 2.5 | 7 | 3.5 | 8 | 33 | 7.5 | 13.5 | 43 | 8.5 | 14.5 |
| CP-OFDM | QPSK | 3.5 | 8 | 4.0 | 8.5 | 3.53 | 8 | 14.5 | 4.53 | 9 | 15.5 |
| NOTE 1: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 2: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 3: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. |  |  |  |  |  |  |  |  |  |  |  |

The main PA power capabilities and linearity differences between 1Tx PC2, 2Tx PC2 and 2Tx PC1.5 are summarized in Table 2:

Table 2: PA configuration comparison

| Power class configuration | PA configuration | ACLR @ 1dB MPR | RIMD impact | PCmax /PA | CA PCmax |
| --- | --- | --- | --- | --- | --- |
| 1Tx PC2 | one PC2 PA | 31dB | no | 26 dBm | 26 dBm |
| 2Tx PC2 | two PC3 PAs | 30dB | yes | 23 dBm | 26 dBm |
| 2Tx PC1.5 | two PC2 PAs | 31dB | yes | 26 dBm | 29 dBm |

From the above table it may be legitimate to compare PC1.5 MPR with both cases as it uses the two PAs that are the same than the one used in 1Tx PC2, but has 2Tx like for 2Tx PC2 and thus is equally subject to RIMD. In our measurements, we chose to compare to 1Tx PC2, as it means the PAs are measured under the same linearity and voltage (31dB ACLR MPR1) and the only two differences are the addition of RIMD and a 3dB higher maximum power reference.

These differences will impact MPR in a different way, depending on whether the emission limitations are:

- From a relative measurement like IBE, EVM or ACLR => only impact is RIMD (0.5-1dB impact)

- From an absolute measurement like SEM or spurious emissions => impacted by 3dB higher power (at least 1dB impact) and RIMD (additional 0.5-1dB impact)

In order to assess this, a set of waveforms were tested for 2Tx PC1.5 and 1Tx PC2 using the same PAs and linearity conditions. We provide here the results for the most relevant cases in Table 3 that excludes contiguous inner allocations cases as they are not OOB emissions limited (proper EVM/IBE evaluation will be needed for those).

Table 3: 2Tx PC1.5 and 1Tx PC2 results for relevant allocations.

| DFT allocation parameters |  |  | back-off |  | emission limitation |  |
| --- | --- | --- | --- | --- | --- | --- |
| type | #RB1 | #RB2 | PC2 | PC1.5 | PC2 | PC1.5 |
| ContOuter | 108 | 108 | 1.4 | 1.6 | ACLR | ACLR |
| ContOuter | 216 | 1 | 1.2 | 1.3 | ACLR | ACLR |
| ContOuter | 216 | 15 | 1.5 | 1.8 | ACLR | ACLR |
| ContOuter | 216 | 108 | 1.8 | 2 | ACLR | ACLR |
| ContOuter | 216 | 216 | 3.5 | 3.8 | ACLR | ACLR |
| NCInner | 1 | 1 | 0.3 | 0.5 | -13&-30dBm/MHz SEM | -13&-30dBm/MHz SEM |
| NCInner | 15 | 15 | 0 | 0.1 | -13dBm/MHz SEM | -13dBm/MHz SEM |
| NCouter1 | 1 | 1 | 5.2 | 6.1 | -30dBm/MHz SEM | -30dBm/MHz SEM |
| NCouter1 | 15 | 1 | 2.9 | 3.2 | -13dBm/MHz SEM | -13dBm/MHz SEM |
| NCouter1 | 15 | 15 | 4.8 | 5.1 | ACLR | ACLR |
| NCouter2 | 108 | 15 | 1.7 | 1.9 | ACLR | ACLR |
| NCouter2 | 108 | 108 | 3.3 | 3.5 | ACLR | ACLR |
| NCouter2edge | 1 | 1 | 5.4 | 7.8 | -13dBm/MHz SEM | -13dBm/MHz SEM |
| NCouter2edge | 15 | 1 | 2.3 | 2.9 | -13dBm/0.4MHz SEM | -13dBm/0.4MHz SEM |
| NCouter2edge | 15 | 15 | 5.6 | 6.1 | ACLR | ACLR |
| NCouter2edge | 108 | 1 | 1.5 | 1.7 | ACLR | ACLR |
| NCouter2edge | 108 | 15 | 2.1 | 2.4 | ACLR | ACLR |
| NCouter2edge | 108 | 108 | 3.3 | 3.6 | ACLR | ACLR |

Observations: The predicted behavior is confirmed with:

- ACLR (relative measurements) related backoff slightly higher for 2Tx PC1.5 than 1Tx PC2 due to RIMD impact of ~0.3dB.

- SEM (absolute measurement) related backoff are more impacted due to higher power and RIMD by up to 2.4dB in one case.

- Backoff above 4-5dB will need to be re-assessed at lower voltages to account for APT/ET PA behavior.

- Note that these differences may be further impacted by allowing PSD imbalance as discussed in next chapter.

Based on the above observations, we can already formulate an anticipated range for the PC1.5 MPR increase versus 1Tx PC2 for the different allocation types and accounting for the fact that ET PAs are more sensitive to RIMD:

- Contiguous inner and outer are all limited by relative measurements like ACLR, EVM and IBE and thus will see the MPR increase by 0.5-1dB.

- Non-contiguous allocations are limited by ACLR for large total allocation and will see MPR increase by 0.5-1dB. However, the worst smallest allocations are SEM limited, thus will see 2.5-3dB increase.

## 2.4 2Tx PC1.5 contiguous ULCA MPR versus PSD imbalance.

In a similar way that we could compare 2Tx PC1.5 backoff versus 1Tx PC2, we made similar comparison for different PSD imbalance and especially for 6dB imbalance like we proposed in [4]. The relevant results are presented in Table 4:

- Equal PSD case is denoted “=PSD” (this is also equal power for cases with same allocation BW in each CC)

- “+6dB” denotes 6dB PSD imbalance with increased PSD on CC1 vs CC2

- “+6dB” denotes 6dB PSD imbalance with increased PSD on CC2 vs CC1

- Equal Poweer case is denoted “=PWR”

- XXdBm/YYM denotes a SEM limitation at XX dBm in YY MHz.

Table 4: 2Tx PC1.5 results for relevant allocations versus PSD imbalance

| DFT allocation parameters |  |  | back-off |  |  |  | emission limitation |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| type | #RB1 | #RB2 | =PSD | +6dB | -6dB | =PWR | =PSD | +6dB | -6dB | =PWR |
| ContOuter | 108 | 108 | 1.6 | 0.6 | 1.7 | na | ACLR |  |  | na |
| ContOuter | 216 | 1 | 1.3 | 1.5 | 1.6 | 1.4 | ACLR |  |  |  |
| ContOuter | 216 | 15 | 1.8 | 1.7 | 2.6 | 1.2 | ACLR |  |  |  |
| ContOuter | 216 | 108 | 2 | 1.9 | 2.5 | 2.5 | ACLR |  |  |  |
| ContOuter | 216 | 216 | 3.8 | 2.3 | 3.7 | na | ACLR |  |  | na |
| NCInner | 1 | 1 | 0.5 | 0 | 0.6 | na | -13dBm/M |  |  | na |
| NCInner | 15 | 1 | 0 | 0 | 0 | 0.3 | none |  |  | -13dBm/M |
| NCInner | 15 | 15 | 0.1 | 0 | 0 | na | -13dBm/M | none |  | na |
| NCouter1 | 1 | 1 | 6.1 | 4.6 | 6.3 | na | -30dBm/M | -13dBm/M | -30dBm/M | na |
| NCouter1 | 15 | 1 | 3.2 | 1.2 | 5.4 | 6.7 | -13dBm/M |  | ACLR |  |
| NCouter1 | 15 | 15 | 5.1 | 3.5 | 5.3 | na | ACLR |  |  | na |
| NCouter2 | 108 | 1 | 0 | 0 | 0.5 | 4.2 | none |  | ACLR |  |
| NCouter2 | 108 | 15 | 1.9 | 0.6 | 4.1 | 4.4 | ACLR |  |  |  |
| NCouter2 | 108 | 108 | 3.5 | 2.3 | 3.6 | na | ACLR |  |  | na |
| NCouter2edge | 1 | 1 | 7.8 | 8.2 | 7.5 | na | -13dBm/M |  |  | na |
| NCouter2edge | 15 | 1 | 2.9 | 3 | 4.2 | 4.9 | -13dBm/0.4M |  | ACLR |  |
| NCouter2edge | 15 | 15 | 6.1 | 4.3 | 5.9 | na | ACLR |  |  | na |
| NCouter2edge | 108 | 1 | 1.7 | 2.3 | 2.5 | 3.3 | ACLR |  |  |  |
| NCouter2edge | 108 | 15 | 2.4 | 2.3 | 4.2 | 4.4 | ACLR |  |  |  |
| NCouter2edge | 108 | 108 | 3.3 | 3 | 4.8 | na | ACLR |  |  | na |

Observations:

- In most cases, the limitation comes from the same emission requirement versus PSD imbalance, but there are cases where different limitation occurs; and, in a few cases, the limitation changes from an absolute power measurement (SEM) to a relative measurement (ACLR)

- The worst back-off occurs for the equal power case for large BW imbalance between the two CCs, in some cases it can be more than 3dB higher than the equal PSD case (red highlight).

- The worst back-off between equal PSD and 6dB imbalance (bold) is in most cases for the 6dB imbalance with:

  - Up to 0.8dB impact for contiguous outer

    - For non-contiguous outer worst-case allocations based on 1RB+1RB, the impact is less important as the the backoff for equal PSD is already important.

  - Up to 2.2dB impact for non-contiguous outer.

  - Limited impact for inner (may depend on further EVM/IBE evaluation).

The overall conclusion is that accounting for 6dB PSD imbalance in the MPR is viable as, if it has an impact, it is limited for the worst-case allocations. However, supporting equal power has a significant impact for MPR for little value as it further impacts the CC with the larger allocated RB BW.

Proposal on power imbalance:

- Equal power is not supported by the MPR and is not evaluated.

- 6dB PSD imbalance is supported by MPR and evaluated (as proposed in [4]).

## 2.5 Proposals for 2Tx PC1.5 contiguous ULCA MPR evaluation

Based on the above observations we make a detailed proposal for the MPR evaluation plan.

Proposal on 2Tx PC1.5 contiguous ULCA MPR evaluation: Given the complexity of the number of configurations to be measured, the evaluation is performed via:

- Simulations to identify corner allocation cases and waveforms for cases that are:

  - EVM, IBE or SEM limited (RIMD impact only).

  - SEM or spurious emission limited (RIMD+maximum power impact) .

  - By comparison with either 1Tx PC2 or 2Tx PC2 cases.

- Measurement of corner cases in 2Tx setup with cross-coupled PAs

- Measurements and simulations should account for APT/ET PA behavior at larger back-off.

- Both BW class B and class C are evaluated.

- Both Equal PSD and worst case 6dB imbalance are evaluated.

- As a start, QPSK DFT-s-OFDM and CP-OFDM are evaluated for:

  - Contiguous inner and outer allocations.

  - Non-contiguous inner, outer1 and outer2 allocations.

# Conclusion

In this contribution, based on our previous work on the topic we performed back-off measurements for 2Tx PC1.5 and 1Tx PC2 cases and various PSD imbalance to assess their differences and make the following proposals for future evaluations.

Proposal on power imbalance:

- Equal power is not supported by the MPR and is not evaluated.

- 6dB PSD imbalance is supported by MPR and evaluated (as proposed in [4]).

Proposal on 2Tx PC1.5 contiguous ULCA MPR evaluation: Given the complexity of the number of configurations to be measured, the evaluation is performed via:

- Simulations to identify corner allocation cases and waveforms for cases that are:

  - EVM, IBE or SEM limited (RIMD impact only).

  - SEM or spurious emission limited (RIMD+maximum power impact) .

  - By comparison with either 1Tx PC2 or 2Tx PC2 cases.

- Measurement of corner cases in 2Tx setup with cross-coupled PAs

- Measurements and simulations should account for APT/ET PA behavior at larger back-off.

- Both BW class B and class C are evaluated.

- Both Equal PSD and worst case 6dB imbalance are evaluated.

- As a start, QPSK DFT-s-OFDM and CP-OFDM are evaluated for:

  - Contiguous inner and outer allocations.

  - Non-contiguous inner, outer1 and outer2 allocations.

# References

[1] R4-2411646 On PC1.5 intra-band contiguous ULCA with 2Tx, Skyworks Solutions Inc., RAN4#112

[2] R4-2411302 On equal PSD vs equal power spectral regrowth, Skyworks Solutions Inc., RAN4#112

[3] R4-2414277 WF on HPUE for CA in TN, Samsung, RAN4#112

[4] R4-2415275 On PC1.5 ULCA PCmax and UE behavior under large PSD imbalance, Skyworks Solutions Inc., RAN4#112b