# 3GPP TSG RAN WG4 Meeting#112-bis	R4-2415646

# Hefei, CN, Oct 14 – Oct 18, 2024

Source: 	Qualcomm Incorporated

Title: 	Discussion on NTN testing for NGSO

Agenda item:	6.8.4

Document for:	Discussion

# 1. Introduction

Currently NTN testing for NGSO assumes fixed Doppler or fixed delay shifts which are randomly selected before the test. However, it does not reflect the dynamic conditions that the UE supporting NGSO devices may experience in deployment scenarios. To this end, a more realistic channel model with varying Doppler and delay shifts that matches propagation trajectory of the NGSO satellites may be needed to ensure appropriate test coverage for these devices under dynamic conditions.

To this end, RAN4 has introduced a work item in Rel-19 to come up with a channel model considering varying Doppler and timing shifts for NR-NTN and IoT-NTN devices [1]. During the last meeting, RAN4 discussed relevant issues, including propagator models and test procedures. In this paper, we share our views on the remaining open issues.

2. TE Emulated Channel Model

2.1 Time Varying Doppler and Delay Shift Modelling

Methodology for Time varying Doppler and Delay shifts modelling

- Agreement

- RAN4 aims to introduce TE-emulated time varying doppler shift and delay shift model for satellite motion with following candidate options:

  - Option 1: Eckstein-Hechler model

  - Option 2: Keplerian model

  - Option 3: Existing model from TR 38.811 5.3.4.3

  - Option 4: Proposal 2 from R4-2412552

  - Option 5: Proposal 2 from R4-2412783

  - Other options not precluded

Note: Companies are encouraged to bring more information for option 1 and option2.

- Predefined files which generated by GMAT can be considered as reference for the comparison purpose of proposed channel model

- FFS on the initial ephemeris information and parameters

During the last meeting, RAN4 discussed several proposals from companies in regard to modelling the time varying Doppler and delay shift for satellite mobility. The relevant way forward is presented below [2].

We note that the Keplerian propagator uses simplified two-body equations of motion, assuming only central gravitational forces. This model is suitable for short-term predictions and simple orbits. On the other hand, the Ekstein-Hechler propagator is specifically designed to integrate the equation of motion of a satellite under various perturbations, such as Earth’s oblateness, atmospheric drag and can handle more complex orbital dynamics.

Observation 1: The complexity of the propagator models needs to be considered as the UE vendors may have to implement the chosen model in their channel emulator.

Considering the implementation complexity, we prefer an analytical propagator model over a numerical integration-based model. In particular, we propose to consider Keplerian model (Option 2) which offers a good trade-off between implementation complexity and performance. To this end, we present our results for Keplerian model below (using ECEF data provided by one of the satellite vendors). The satellite position and velocity error are shown in Fig. 1. Our modeling results suggest that over a prediction window of 5s and 10s, the position errors are 3.4m and 6.7m, respectively. We also study the propagation delay and delay change rate. We found that for a prediction window of 20s, propagation delay error is 0.66us, while the maximum delay change rate is 1.89e-8.

[Semi-major Axis (a), Eccentricity (e), Inclination (i), Right Ascension of the Ascending Node (Iaan), Argument of Periapsis (w), True Anomaly (nu)]

Figure 1: Satellite position and velocity error for a two-body Keplerian-based propagator model

Observation 2: With the aid of ECEF data from one of satellite vendors, the two-body Keplerian-based propagator model leads to following observations

  - For a prediction window of 5s and 10s, position errors are 3.4m and 6.7m, respectively.

  - For a prediction window of 200s

    - The propagation delay error is 0.66us

    - The maximum delay change rate is 1.89e-8

We further evaluate the performance of the two-body Keplerian model against the Simplified General Perturbation Model 4 (SGP4). We note that SGP4 model begins with an idealized Keplerian orbit, but also accounts for non-Keplerian effects, such as gravitational irregularities and drag. We note that SGP4 is supported by General Mission Analysis Tool (GMAT) as well. As presented in Fig. 2, our results suggest that SGP4 and two-body Keplerian model provides very similar maximum delay error. Furthermore, for a prediction window of 20s, propagation delay error is still less than 10ns with the two-body Keplerian model.

Input ephemeris

every 10sec

Keplerian-based interpolated position vectors at 1kHz

- Maximum delay error (b) Keplerian based prediction (input ephemeris sampled at every 10s)

[Semi-major Axis (a), Eccentricity (e), Inclination (i), Right Ascension of the Ascending Node (Iaa n), Argument of Periapsis (w), True Anomaly (nu)]

Figure 2: Comparison of performance for SGP4 and two-body Keplerian-based propagator models

Observation 3: Performance comparison of the two-body Keplerian-based propagator model against SGP4 based propagator model (supported by GMAT) shows that the propagation delay error is less than 10ns for a prediction window of 20s.

The previous two observations confirm that the two-body Keplerian-based propagator mode offers sufficient estimation accuracy with reasonable implementation complexity.

Proposal 1: RAN4 to adopt the two-body Keplerian propagator model to model the time varying Doppler and delay shifts.

2.2 Parameters for TE-emulated Channel Model

Parameters for TE-emulated Channel Model

- Agreement

- RAN4 further discuss the necessary parameters with associated channel model, following parameters list for information purpose:

  - Satellite altitude:

    - Option 1: LEO -600km only

  - Option 2: Both LEO-600km and LEO-1200km Elevation angle range

  - UE position

  - Whether to consider a non-zero time-varying feeder link delay

- Other parameters not precluded

As presented below, one of the several parameters that were discussed during the last RAN4 meeting is satellite altitude [2]. To this end, we prefer to consider only LEO-600 orbit. In our view, LEO-600 is a more challenging scenario in some respects, such as atmospheric drag and space debris. Additionally, demodulation requirements for Rel-17 and Rel-18 work items only considered LEO-600 orbit. Therefore, we think that considering only LEO-600 orbit is sufficient for testing NGSO scenarios.

Proposal 2: Consider only LEO-600 orbit for NTN testing for UE supporting NGSO.

2.3 Test Procedure

During the last RAN4 meeting, test procedure related to TE-emulated channel model was briefly discussed. In this section, we share our views on this. A diagram showing the test procedure is presented below.

Figure 3: Pictorial view of the test procedure for the two-body Keplerian-based propagator model

As seen in Fig. 3, we consider a test time of X seconds containing a total of N orbits. The initial ephemeris for each orbit is generated by either GMAT or any numerical method. While doing so, it is also necessary to make sure that the elevation angle is not very small, as otherwise UE will not be satellite’s view. The subsequent ephemeris data after the initial ephemeris will be generated based on the two-body Keplerian model at a certain interval, at which point the propagation delay and Doppler shift will be derived based on the predicted ephemeris. If the prediction window is larger than the slot length of interest in the test case, e.g., due to test equipment or channel emulator complexity, the propagation delay and Doppler shift at the beginning of each slot can be generated by applying linear interpolation between the propagation delays and Doppler shifts for that interval.

The detailed proposal for the test procedure is provided below.

Proposal 3: The entire N orbits within a test time of X seconds are generated as follow.

- The initial ephemeris for each orbit is generated by either GMAT or any numerical method.

- The initial ephemeris for each orbit should be chosen such that the elevation angle from the UE perspective is not smaller than Z degrees. The value of Z may vary for different test cases. And if multiple satellites (i.e. N > 1) are needed in RRM mobility/measurement test cases, the value of Z may be different for different satellites.

- The ephemeris data after the initial data is generated based on the two-body Keplerian model at every Y sec. The value of Y may differ for NR NTN and IoT NTN.

- The propagation delay and Doppler shift at every Y sec are derived based on the projected ephemeris data and given UE location.

- In case Y is larger than the slot length of interest in the test case due to, e.g., test equipment/channel emulator complexity, the propagation delay and Doppler shift at the beginning of each slot can be generated by applying, e.g., linear interpolation between the propagation delays and Doppler shifts at Y sec intervals.

3. RAN4 Requirements and Tests

3.1 Demodulation Requirements

Demodulation requirements

- Agreement

  - RAN4 further discuss how to apply the varying Doppler and delay shifts to the UE performance requirements and necessary simulation assumption on UE side pending on channel model discussion progress.

    - UE assumption on propagator model for satellite motion for RAN4 simulation purpose

      - Option 1: Keplerian model

      - other options not precluded

  - RAN4 further discuss test applicability if new PDSCH demodulation requirements introduced pending on channel model discussion progress.

During the last meeting, several issues related to RAN4 test and requirements were discussed. The way forward is captured below [2].

To align results across companies during the propagator model evaluation phase, it would be useful to consider a certain propagator model at the UE side. To this end, our preference is to consider Keplerian propagator model as it offers a good trade-off between implementation complexity and performance. We note that this does not prevent the UE from implementing a more accurate propagator model than Keplerian.

Proposal 4: Option 1 (Consider Keplerian model for UE assumption on the propagator model)

4. Conclusions

In this paper, we share our views on the performance requirements for NTN testing for UE supporting NGSO scenarios. The following have been proposed.

Observation 1: The complexity of the propagator models needs to be considered as the UE vendors may have to implement the chosen model in their channel emulator.

Observation 2: With the aid of ECEF data from one of satellite vendors, the two-body Keplerian-based propagator model leads to following observations

  - For a prediction window of 5s and 10s, position errors are 3.4m and 6.7m, respectively.

  - For a prediction window of 200s

    - The propagation delay error is 0.66us

    - The maximum delay change rate is 1.89e-8

Observation 3: Performance comparison of the two-body Keplerian-based propagator model against SGP4 based propagator model (supported by GMAT) shows that the propagation delay error is less than 10ns for a prediction window of 20s.

Proposal 1: RAN4 to adopt the two-body Keplerian propagator model to model the time varying Doppler and delay shifts.

Proposal 2: Consider only LEO-600 orbit for NTN testing for UE supporting NGSO.

Proposal 3: The entire N orbits within a test time of X seconds are generated as follow.

- The initial ephemeris for each orbit is generated by either GMAT or any numerical method.

- The initial ephemeris for each orbit should be chosen such that the elevation angle from the UE perspective is not smaller than Z degrees. The value of Z may vary for different test cases. And if multiple satellites (i.e. N > 1) are needed in RRM mobility/measurement test cases, the value of Z may be different for different satellites.

- The ephemeris data after the initial data is generated based on the two-body Keplerian model at every Y sec. The value of Y may differ for NR NTN and IoT NTN.

- The propagation delay and Doppler shift at every Y sec are derived based on the projected ephemeris data and given UE location.

- In case Y is larger than the slot length of interest in the test case due to, e.g., test equipment/channel emulator complexity, the propagation delay and Doppler shift at the beginning of each slot can be generated by applying, e.g., linear interpolation between the propagation delays and Doppler shifts at Y sec intervals.

Proposal 4: Option 1 (Consider Keplerian model for UE assumption on the propagator model)

5. References

[1] RP-241281, “Revised WID on enhanced requirements and conductive test methodology for NR NTN and IoT NTN,” 3GPP TSG RAN WG Meeting #104, Shanghai, CN June 17 – 20, 2024.

[2] R4-2413516, “WF for NTN testing for NGSO channel model,” 3GPP TSG RAN WG Meeting #112, Maastricht, NL, Aug 19 – 23, 2024.