3GPP TSG RAN WG4 #112bis	R4-2415675

Hefei, China, Oct 14 – 18, 2024

Agenda item:		6.20.2.3

Source:	Nokia

Title:	On existing BS RF RX requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#112 [1] regarding evolution of NR duplex operation. There were many agreements made that have impact on the existing BS RF requirements for SBFD operation. Also, the open items are listed in the WF. The requirement with the biggest impact is the RX in-band blocking. This contribution displays Nokia’s view on the existing BS RF RX requirements for SBFD operation and emphasizes the importance of setting correctly the RX in-band blocking requirement to enable functionality in real-life deployment.

# Impact on BS RX requirements

## In-band blocking simulations

RAN4#112 agreed on the following:

Agreement: - On contributing source for in-band blocking requirement, it is only the DL adjacent channel interference. - RAN4 to not consider any CLI handling scheme effects when defining the in-band blocking requirements. - For co-existence study, - The reference point for the power level which should be before array gain WF: - FFS on the followings: - RAN4 needs to discuss on how in-band blocking requirement are derived: - Option 1: MCL assumption for BS2BS CLI interference - Option 2: Co-existence study - Option 3: Both - For co-existence study, RAN4 need firstly agree on the open parameters for simulation assumption: - FFS grid-shift values should be considered. - Option 1: 10% - Option 2: 20% - Option 3: 50% - Option 4: 100% - 99% of the UL SBFD wideband received power CDF

### MCL vs. coexistence study

During RAN4#112, some companies proposed to use the minimum coupling loss (MCL) analysis to derive the in-band blocking requirements while others propose to rely on coexistence simulations. Our preference is to use the coexistence simulations since companies have already agreed on the simulation methodology. Starting the discussions about MCL now would require time to agree on, for instance, the level of coupling loss that we should consider for each scenario/BS type (which might require additional simulations and/or on-field measurements); note also that the determination of MCL resemble partly the feasibility analysis done during the Rel-18 Study Item phase (e.g. inter-sector interference analysis in TR 38.858) in which it was not possible to reach a consensus among companies. It is also important to note that while MCL would in principle only account for one aggressor base station while the coexistence analysis is able to model the interfering signal from N adjacent base stations. The latter is more in line with the expected conditions in real deployments. Also, how to account for the antenna and beamforming gains as part of the MCL is not clear. Finally, it is worth mentioning that coexistence simulations have been historically used in 3GPP to derive the current in-band blocking requirements [TR 25.942].

Consider only coexistence simulations for the derivation of the in-band blocking requirements.

### Clarification on how the in-band blocking requirement derivation

In the previous RAN4#112 meeting, companies agreed to not account for the SBFD specific co-channel interferences to derive the in-band blocking requirement. In other words, BS self-interference and inter-sector interferences are not considered. However, these interferences also impact the receiver performance since they are also received by the BS LNA. This is an important assumption that should be clearly stated as part of the in-band blocking requirement definition for SBFD-capable BS in order to avoid possible misunderstandings.

RAN4 specifications should include a clear statement on how the in-band blocking requirements for SBFD BS are derived in order to avoid possible misunderstandings.

### Grid shift assumptions

Regarding the grid-shift values for the coexistence simulations, we propose to focus on the 10% case. Among the considered grid-shift values, 10% grid-shift results in the worst-case scenario from an in-band blocking perspective since it results in the highest adjacent channel interference. A SBFD BS that fulfils the in-band blocking requirement based on the 10% grid-shift would be automatically compliant for any of the other considered grid-shift values such as 20%, 50% or 100%. However, if the in-band blocking requirement is derived from, e.g., a grid shift of 50%, the SBFD operation for 10% grid-shift would be compromised since the interfering signal power would be higher than the used to determine the in-band blocking requirement. Note that site-sharing is commonly used when deploying base stations from different operators, and 10% grid shift is what closet resembles the site-sharing scenario.

Only consider 10% grid-shift to derive the interfering signal power for the in-band blocking requirement.

### ACLR and ACS effects

One important aspect for the in-band blocking power derivation is whether effects such as ACLR and ACS effects should be included. As described in TR 38.858, Clause E.2.3.1 the DL adjacent channel interference for the in-band blocking requirement should be measured following the definition of IACI, and it is explicitly mentioned that it does not include the “frequency isolation” aspects. i.e., the ACLR and ACS effects.

RAN4 to agree/clarify that the derivation of the in-band blocking requirement is aligned with the definition of IACI in TR 38.858, Clause E.2.3.1 which does not include ACLR and ACS effects.

### Coexistence results for in-band blocking derivation

A summary of the results can be found in the following table, where the 99th percentile of the in-band blocking power is collected. The simulated case is Case 3 (i.e., SBFD BS as victim and TDD DL as aggressor) and results are shown for all scenarios considered during the study item. For those scenarios where grid shift can be applied, a grid shift of 10% is assumed. The simulation assumptions are aligned with the Annex E TR 38.858. For those parameters with multiple options, baseline and 1st priority values are assumed.

Table 1. In-band blocking power [dBm] per scenario and base station type

| Case 3 scenarios | In-band blocking power at 99th percentile | Current in-band blocking power requirement in TS 38.104 |
| --- | --- | --- |
| Scenario 1 (Urban macro FR1) | -2.5 dBm | Wide Area FR1 BS: -43 dBm |
| Scenario 3 (Urban micro FR1) | -4.3 dBm | Medium Range FR1 BS: -38 dBm |
| Scenario 5 (Indoor FR1) | -36.1 dBm | Local Area FR1 BS: -35 dBm |
| Scenario 6 (Urban macro FR2) | -37.4 dBm | Wide Area FR2-1 BS: [-86, -63] dBm |
| Scenario 9 (Indoor FR2) | -57.7 dBm | Local Area FR2-1 BS: [-76, -53] dBm |

Based on the results above the following is observed:

For FR1 wide area SBFD BS, the in-band blocking power at the 99th percentile is -2.5 dBm, which is approximately 40 dB higher than the current in-band blocking requirement.

For FR1 medium range SBFD BS, the in-band blocking power at the 99th percentile is -4.3 dBm, which is approximately 34 dB higher than the current in-band blocking requirement.

For FR1 local area SBFD BS, the in-band blocking power at the 99th percentile is -36.1 dBm, which is on the same order of magnitude than the existing local area FR1 BS.

For FR2-1 wide area SBFD BS, the in-band blocking power at the 99th percentile is -37.4 dBm, which is approximately 25 dB higher than the current in-band blocking requirement.

For FR2-1 local area SBFD BS, the in-band blocking power at the 99th percentile is -57.7 dBm, which is on the same order of magnitude than the current in-band blocking requirement.

We also have the following proposals based on the results:

For FR1 wide area SBFD BS, define an in-band blocking power requirement of -2 dBm.

For FR1 medium range SBFD BS, define an in-band blocking power requirement of -4 dBm.

For FR2 wide area SBFD BS, define an in-band blocking power requirement of -37 dBm.

For FR1 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements.

For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements.

Our analysis indicates that existing local area BS in-band blocking requirements can be re-used for local area SBFD BS for both FR1 and FR2-1. However, FR1 wide area and medium range BS as well as FR2-1 wide area BS should introduce new in-band blocking requirements. The only BS type not studied is the medium range FR2-1 BS. The FR2-1 medium range SBFD BS requirement has not been studied, and therefore we think it should also be considered.

Consider Scenario 8 (urban dense FR2-1) to derive the FR2 SBFD medium range BS in-band blocking requirements.

## Reference sensitivity level and OTA sensitivity

The following was agreed in RAN4#112:

Agreement: - OTA sensitivity is measured with only self-interference. FFS on the OTA sensitivity degradation within [0.5~1.0] dB.

Even though SBFD operates in non-overlapping mode in frequency, it is still transmitting and receiving simultaneously in full duplex mode in time domain. This makes it fundamentally different from TDD deployment and the differences in the actual system on the field should be taken into careful consideration. It can be argued that the self-interference plays a major role in sensitivity degradation but nevertheless the effect of co-site inter-sector and inter-site interference cannot be neglected. They are much more difficult to cancel than self-interference, which might be easier to cancel up to some extent. As the SBFD technique is all about improving the UL performance, it is of paramount importance to not sacrifice too much sensitivity when deploying SBFD systems. As sectorized setup for co-site inter-sector interference and inter-site interference from other gNB’s is impossible to realize in the OTA testing, the agreement was made to measure OTA sensitivity only with self-interference. But as noted before, the impact of other interferences is not reflected in the test, and it might lead to unacceptable level of desensitization of the receiver when deployed on the field. Therefore, we propose to limit the OTA sensitivity degradation to maximum of 0.5dB.

OTA sensitivity test only accounts for self-interference.

Other interferences still exist in real-life deployments.

SBFD benefits lie in UL improvements and sensitivity should not be overly degraded.

OTA sensitivity degradation should be limited to maximum of 0.5dB.

## Dynamic Range

The following was agreed in RAN4#112:

WF: - FFS on the followings: - Option 1: RAN4 to discuss if the IoT level and wanted signal power level will be derived based on simulation work. - Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations. - Option 3: for receiver dynamic requirement, both uplink signals and BS2BS CLI signal should be considered for IoT levels.

For SBFD, the BS IoT level will be influenced by the RSI capability of the base station as well level of interference from other sectors and neighbouring base stations. Nevertheless, based on our analysis we think 20 dB (Option 2) provides sufficient margin to cover these different sources of interference.

For receiver dynamic range requirement, adopt Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations.

## ACS Requirement

The following was agreed in RAN4#112:

- 1.7. ACS requirement WF: - FFS on the followings: - Option 1: If co-location ACS is not defined, the note in the spec that the ACS requirement is not applied to the co-location scenario is needed. - Option 2: The OTA sensitivity degradation should be taken into account for the baseline REFSENS for ACS requirement.

We don’t see the need for co-location ACS requirement. The same reasons apply as for the co-location ACLR and OBUE requirement. There is no need for 3GPP to define any additional requirements for co-location ACS. If one wants to deploy SBFD system, they should have proprietary customer requirements to satisfy the co-location requirements. We also disagree to add the text in the note proposed in option 1. Rather than stating that ACS requirement is not applied to co-location scenario, it should be stated that current requirements do not enable co-location scenario.

No need for RAN4 to specify additional co-location ACS requirement.

## Necessity of New RX intermodulation requirement with 1 interfering signal

The following was agreed in RAN4#112:

WF: - FFS on the followings: - Option 1: Whether IMD requirement for single interfering signal scenario is needed. Investigate whether such a requirement is implicitly captured by the SBFD RX blocking requirement. - Option 2: If RAN4 want to introduce the additional RX intermodulation requirement (a single input signal placed to cause IM with the RX sub-band), the new intermodulation scenario shall be confirmed firstly with evidence showing the proposed scenario exists in practice.

We think that the proposed new RX intermodulation scenario with 1 interfering signal might be a possible scenario, and it should be studied further. We must be very careful when defining RF requirements and especially reusing existing ones. The differences with legacy TDD and SBFD systems are imminent and might end up causing disastrous effects in the field in the worst case. As the power of the own TX sub-band signal (self-interference) could be significantly higher than what is currently assumed in the general RX IMD requirement (-52dBm for WA BS), this could lead to situation where the current RX IMD requirement is inadequate and doesn’t guarantee the linearity of RX is enough. Additional robustness to RX would be highly welcomed to ensure functionality in this situation. The SBFD RX blocking requirement might not capture this implicitly, at least not if the own TX is not on during the test.

Careful consideration is needed to evaluate requirements.

The power level of own TX signal might be significantly higher than what is currently defined for interfering signals.

RAN4 to study further RX intermodulation scenario with 1 interfering signal.

## Necessity of New RX intermodulation requirement with 2 interfering signals

The following was agreed in RAN4#112:

- FFS on the following: - Option 1: for the receiver intermodulation requirements, BS2BS CLI should be taken into account for power level for interference signal.

In our view this is similar case than in 2.5. If the above case with 1 interfering signal is considered (self-interference signal with 1 interferer causes IMD in the RX sub-band), then we see this will be captured there.

# Conclusion

In this contribution, our views on the SBFD BS RF requirements are presented. There were observations and proposals as listed below.

- For FR1 wide area SBFD BS, the in-band blocking power at the 99th percentile is -2.5 dBm, which is approximately 40 dB higher than the current in-band blocking requirement.

- For FR1 medium range SBFD BS, the in-band blocking power at the 99th percentile is -4.3 dBm, which is approximately 34 dB higher than the current in-band blocking requirement.

- For FR1 local area SBFD BS, the in-band blocking power at the 99th percentile is -36.1 dBm, which is on the same order of magnitude than the existing local area FR1 BS.

- For FR2-1 wide area SBFD BS, the in-band blocking power at the 99th percentile is -37.4 dBm, which is approximately 25 dB higher than the current in-band blocking requirement.

- For FR2-1 local area SBFD BS, the in-band blocking power at the 99th percentile is -57.7 dBm, which is on the same order of magnitude than the current in-band blocking requirement.

- OTA sensitivity test only accounts for self-interference.

- Other interferences still exist in real-life deployments.

- SBFD benefits lie in UL improvements and sensitivity should not be overly degraded.

- Careful consideration is needed to evaluate requirements.

- The power level of own TX signal might be significantly higher than what is currently defined for interfering signals.

- Consider only coexistence simulations for the derivation of the in-band blocking requirements.

RAN4 specifications should include a clear statement on how the in-band blocking requirements for SBFD BS are derived in order to avoid possible misunderstandings.

Only consider 10% grid-shift to derive the interfering signal power for the in-band blocking requirement.

RAN4 to agree/clarify that the derivation of the in-band blocking requirement is aligned with the definition of IACI in TR 38.858, Clause E.2.3.1 which does not include ACLR and ACS effects.

For FR1 wide area SBFD BS, define an in-band blocking power requirement of -2 dBm.

For FR1 medium range SBFD BS, define an in-band blocking power requirement of -4 dBm.

For FR2 wide area SBFD BS, define an in-band blocking power requirement of -37 dBm.

For FR1 local area SBFD BS, re-use existing FR1 local area BS in-band blocking power requirements.

For FR2 local area SBFD BS, re-use existing FR2 local area BS in-band blocking power requirements

Consider Scenario 8 (urban dense FR2-1) to derive the FR2 SBFD medium range BS in-band blocking requirements.

OTA sensitivity degradation should be limited to maximum of 0.5dB.

For receiver dynamic range requirement, adopt Option 2: The existing dynamic range requirement, RAN4 assume 20dB interference over thermal noise, which is enough to cover the co-channel interference from other base stations.

No need for RAN4 to specify additional co-location ACS requirement.

RAN4 to study further RX intermodulation scenario with 1 interfering signal.

# References

- R4-2413515 “Way Forward for [112][308] NR_duplex_evo_BSRF”, RAN4#112, Maastricht, Netherlands, 19th-23rd August, 2024

- TR 38.858 V18.1.0 “Study on Evolution of NR Duplex Operation”

- TR 25.942 V18.0.0 “Radio Frequency (RF) system scenarios”