3GPP TSG RAN WG4 #112bis	R4-2415677

Hefei, China, Oct 14 – 18, 2024

Agenda item:		6.20.2.1

Source:	Nokia

Title:	On potentially new BS RF requirements for SBFD operation

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#112 [1] regarding evolution of NR duplex operation. There were some agreements made that have impact on the potentially new BS RF requirements for SBFD operation. Also, the open items are listed in the WF. This contribution displays Nokia’s view on the potentially new BS RF requirements for SBFD operation.

# Discussion

## BS RF Specification structure for SBFD requirements

The following was agreed in RAN4#112:

#### Way forward: #### RAN4 shall study how to introduce BS RF new requirements for SBFD-capable BS: #### Option 1: Embedding the corresponding SBFD RF requirements in the corresponding section of TS 38.104 (similar as NB-IoT RF requirement in TS36.104) #### Option 2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD #### Option 2a: details (like new suffix or not) can be FFS when RF requirements are stable. #### Option 2b: Set up new sub-clause with suffix-B for modified existing RF requirements and SBFD-dedicated new RF requirements #### Option 3: A new specification for SBFD BS RF requirements

#### How to introduce BS RF new requirements for SBFD-capable BS has also been under discussion during the last meetings. The WF presents three options and we prefer the option 2a. Embedding the requirements in the corresponding section of TS 38.104 as proposed in option 1 would create a complex and messy specification. We do not see the need for a new specification for SBFD BS RF requirements as many of the requirements would be the same and the repetition would be immense. Creating a new separate specification will complex BS specifications and increase efforts to maintenance both core and test specifications, and thus it should be avoided. The reason for choosing option 2a is that SBFD is an important feature that should be explicitly visible in specification. Similar approach is already used in UE core specification TS 38.101-1 or Repeater/NCR in Rel-17/18, or IAB/mIAB in Rel-16/17/18 where specific requirement sub-clauses are used for some new network entities or requirements. Also, in the future releases when some potential enhancements may come, such approach will be beneficial and will allow easier maintenance work. Creation a new separate specification will complex BS specifications and increase efforts to maintenance both core and test specifications, and thus it should be avoided.

The details of setting new sub-clauses with suffix-B/other can be left to decision in the later phase as the RF requirements are stable.

- Separate sub-clauses are the clearest way to introduce new BS RF requirements for SBFD in core specification TS 38.104.

- Creating a new separate specification will complex BS specifications and increase efforts to maintenance both core and test specifications, and thus it should be avoided.

For introduction of new BS RF requirements for SBFD operation, solution with creating new sub-clauses is proposed.

Details (like new suffix or not) of the new sub-clauses can be left to decision when the RF requirements are stable.

# Potentially new requirements for SBFD operation

## In-channel adjacent subband leakage ratio, In-channel adjacent subband blocking and adjacent subband selectivity

The following was agreed in RAN4#112:

- Way forward: - FFS the necessity of in-channel adjacent subband leakage ratio requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband leakage ratio. - Option 2: New requirement for in-channel adjacent subband leakage ratio should be defined. - Define a requirement on TX sub-band ACLR similar to the ACLR requirement and use existing ACLR requirement as baseline.

- Way forward: - FFS the necessity of in-channel adjacent subband selectivity requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband selectivity. - Option 2: New requirement for in-channel adjacent subband selectivity should be defined. - Define a requirement on RX sub-band ACS similar to the ACS requirement and use existing ACS requirement as baseline.

- Way forward: - FFS the necessity of in-channel adjacent subband blocking requirements - Option 1: No necessity to introduce new requirement for in-channel adjacent subband blocking. - Option 2: New requirement for in-channel adjacent subband blocking should be defined.

SBFD introduces new types of interference, some of which are highlighted in the figure below. From the BS perspective, the new cases of interference come from: 1) self-interference, 2) co-site inter-sector interference, and 3) inter-site interference.

Figure 1: Types of interference in SBFD deployment

During meetings, it has been discussed whether new requirements should be defined for the adjacent subbands, such as:

- In-channel adjacent subband blocking and selectivity

- In channel adjacent subband leakage

Below, we repeat the need to introduce the different requirements to ensure the SBFD operation.

In our view, new requirements are needed to ensure interoperability between base stations from different vendors in the same network. They cannot be implicitly guaranteed by OTA sensitivity requirement. It is easier to cancel the effects of interference inside your base station, whereas from other base stations there might be no cancellation options. Inter-site and co-site inter-sector BS interference should be taken into consideration in the definition of the requirements for SBFD operation.

The OTA sensitivity requirement does not capture the effects from co-site inter-sector and inter-site interference.

As RAN4 has not agreed on a reference implementation for the gNB, it is difficult to define additional requirements. Though we agree that depending on the gNB implementation the adjacent channel leakage, for example, can be different, in our view, RAN4 is responsible to define minimum requirements which should be achievable for reasonable implementations to guarantee the minimum performance.

In-channel adjacent subband leakage ratio, in-channel adjacent subband blocking and in-channel adjacent subband selectivity requirements cannot be guaranteed implicitly by the OTA sensitivity requirement, since the methods used for self-interference cancellation, might not be available for cancelling interference from other sectors and gNBs, especially when considering a multi-vendor deployment.

Even though RAN4 has not agreed on a reference implementation for SBFD operation, minimum requirements can still be defined to enable proper operation considering self-interference, co-site inter-sector and inter-site interference.

In-channel adjacent sub-band leakage ratio requirements

For the options listed in the WF, we see that option 2 is needed to be selected here. As shown in Figure 1, the leakage from the transmission in the DL subband can cause self-interference, co-site inter-sector interference and inter-site interference. The effects of the adjacent sub-band leakage into the gNBs own receiver can be cancelled to some extent, and we agree that this requirement can be implicitly guaranteed by the OTA sensitivity when considering self-interference only. However, the OTA sensitivity requirement does not guarantee that the adjacent sub-band leakage to other nodes in the network is sufficiently low, too. Therefore, we believe that a new requirement is needed. The exact value of the requirement needs more discussion, since in our view it would depend on the ratio between the bandwidths of the DL and the UL subbands but the baseline value should be the existing ACLR requirement.

RAN4 to define in-channel adjacent sub-band leakage ratio requirements within SBFD time slots considering co-site inter-sector interference and inter-site interference. Existing ACLR requirements could be used as baseline depending on the ratio between the bandwidths of the DL and the UL subbands.

In-channel adjacent sub-band selectivity and blocking

For the options listed in the WF, we see that option 2 is needed to be selected here. Currently, the adjacent channel selectivity (ACS) is defined as a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an adjacent channel signal with a specified centre frequency offset of the interfering signal to the band edge of a victim system. The in-band blocking is defined as: a measure of the receiver's ability to receive a wanted signal at its assigned channel in the presence of an unwanted interferer, which is an NR signal for general blocking or an NR signal with one resource block for narrowband blocking.

In case of SBFD, the sub-band selectivity and blocking would be measured considering the locations of the UL subband and the DL subbands.

As mentioned before, the gNB operating in SBFD would suffer from self-interference, co-site inter-sector interference and inter-site interference. Therefore, there should be a requirement for the in-channel adjacent sub-band selectivity or blocking. Again, it can be argued that the in-channel adjacent sub-band selectivity can be captured in the OTA sensitivity requirements. However, as in the in-channel adjacent sub-band leakage ratio case, this test does not capture the selectivity in the presence of co-site inter-sector and inter-site interference. Furthermore, there might be base stations from different vendors in the same network. That is why these requirements are needed to ensure interoperability between these base stations.

Regarding whether both the in-channel adjacent sub-band selectivity and blocking requirements should be created, it is important to discuss how the existing adjacent channel and in-band blocking requirements are defined and tested. The current tests differ in terms of:

- the frequency offset between the RF bandwidth edge and the center frequency of the unwanted signal

- the unwanted signal power level

In case of assessment of the in-channel adjacent sub-band selectivity or blocking test, the frequency offset should the same, since it depends on the configuration of the UL and DL sub-bands. Therefore, we do not believe that defining/ testing both requirements is necessary but only one of them would suffice. Our preference is to define in-channel adjacent sub-band selectivity.

RAN4 to define in-channel adjacent sub-band selectivity requirement, with the existing ACS requirement would be used as baseline.

# Conclusion

In this contribution, our views on the potentially new SBFD BS RF requirements are presented. There were observations and proposals as listed below.

- Separate sub-clauses are the clearest way to introduce new BS RF requirements for SBFD in core specification TS 38.104.

- Creating a new separate specification will complex BS specifications and increase efforts to maintenance both core and test specifications, and thus it should be avoided.

- The OTA sensitivity requirement does not capture the effects from co-site inter-sector and inter-site interference.

- In-channel adjacent subband leakage ratio, in-channel adjacent subband blocking and in-channel adjacent subband selectivity requirements cannot be guaranteed implicitly by the OTA sensitivity requirement, since the methods used for self-interference cancellation, might not be available for cancelling interference from other sectors and gNBs, especially when considering a multi-vendor deployment.

- Even though RAN4 has not agreed on a reference implementation for SBFD operation, minimum requirements can still be defined to enable proper operation considering self-interference, co-site inter-sector and inter-site interference.

- For introduction of new BS RF requirements for SBFD operation, solution with creating new sub-clauses is proposed.

- Details (like new suffix or not) of the new sub-clauses can be left to decision when the RF requirements are stable.

- RAN4 to define in-channel adjacent sub-band leakage ratio requirements within SBFD time slots considering co-site inter-sector interference and inter-site interference. Existing ACLR requirements could be used as baseline depending on the ratio between the bandwidths of the DL and the UL subbands.

RAN4 to define in-channel adjacent sub-band selectivity requirement, with the existing ACS requirement would be used as baseline.

# References

- R4-2413514 “Way Forward for [112][307] NR_duplex_evo”, RAN4#112, Maastricht, Netherlands, 19th-23rd August, 2024