3GPP TSG-RAN WG4 Meeting #112bis 			                        R4-2415680
Hefei, Anhui, China, 14th – 18th October, 2024
 
Agenda 		Item:6.9.1
Source:			SKY Perfect JSAT 
Title:			Regulation Analysis for Region 3 Ku-band Band Definitions  
Document for:	Discussion and Decision

- Introduction

The Work Item for Satellite Ku Band was approved at RAN#104[1], and discussions started from the RAN4#112 Meeting. During the previous RAN4#112 meeting, the following agreement was reached regarding the Ku Band definition[2]:

- Issue 1-2: Band definitions Define as a first Ku band for Region 1 (CEPT countries): 10.7 – 12.75 GHz (S-to-E) and 14.0 - 14.5 GHz (E-to-S). Additional band definitions are expected once the applicable regulations have been identified and studied.

It has been decided that discussions regarding Region 3 will be conducted after clarifying the Regulations. This contribution presents the regulations in Japan and clarifies the frequencies that should be included in the Ku-band Band Definitions for Region 3.

- Discussion with respect to band definition for Region 3

Japanese regulatory information

The overview of the allocated bands for satellite communication in the Ku-band in Japan is as follows [3]:

Note: Major uses are shown when a frequency band is used for multiple purposes.

Figure 2.1-1 Frequency Allocation Status of Ku Band in Japan

In Japan, after deciding on the domestic allocation of frequencies according to ITU’s frequency allocation, technical requirements are examined to establish the technical standards for new systems. This leads to the establishment of ministerial ordinances regarding technical standards (such as the Radio Equipment Regulations [4] and the Radio Law Enforcement Regulations [5]), allowing for the acquisition of radio station licenses for the new systems [6].

The frequency allocations regarding satellite communication are outlined below.

- FSS(Fixed Satellite Service) Earth Station, 13.75-14.5GHz (Earth-to-Space) 10.7-12.75*GHz (Space-to-Earth)

*Although the allocation to FSS in the 11.7-12.2 GHz band is omitted in Figure 2.1-1, it may also be used for FSS Space-to-Earth in J204[7] and J205[8].

- ESV (Earth Station on Vessel)  Maritime Mobile Earth Station (Ku-band), Radio Equipment Regulations (Article 49-24, Paragraph 2), 14.0-14.5GHz (Earth-to-Space)

- Aeronautical Earth Station, Radio Equipment Regulations (Article 45-21), 14.0-14.5GHz (Earth-to-Space)

- Ku-band non-geostationary satellite communication system using a satellite constellation in an orbit of 600 km or less altitude VSAT Earth Station, Radio Equipment Regulations (Article 54-3 Paragraph 3), 14.0-14.5GHz (Earth-to-Space) 10.7-12.7GHz (Space-to-Earth)

- Ku-band non-geostationary satellite communication system using a satellite constellation in an orbit of 600 km or less altitude Mobile Earth Station, Radio Equipment Regulations (Article 49-23, Paragraph 5), 14.0-14.5GHz (Earth-to-Space) 10.7-12.7GHz (Space-to-Earth)

- Ku-band non-geostationary satellite communication system using a satellite constellation in an orbit exceeding 1,100 km and up to 1,300 km altitude VSAT Earth Station, Radio Equipment Regulations (Article 54-3, Paragraph 4), 14.0-14.5GHz (Earth-to-Space), 10.7-12.7GHz (Space-to-Earth)

- Ku-band non-geostationary satellite communication system using a satellite constellation in an orbit exceeding 1,100 km and up to 1,300 km altitude Mobile Earth Station, Radio Equipment Regulations (Article 49-23, Paragraph 6), 14.0-14.5GHz (Earth-to-Space), 10.7-12.7GHz (Space-to-Earth)

As stated above, the main frequency bands allocated to satellite communications in the Ku band in Japan are 10.7-12.75 GHz (Space-to-Earth) and 13.75-14.5 GHz (Earth-to-Space).

Observation1: The main frequency bands allocated to satellite communications in the Ku band in Japan are 10.7-12.75 GHz (Space-to-Earth) and 13.75-14.5 GHz (Earth-to-Space).

ITU regulatory information

The frequency allocations for the 13.75-14.0 GHz band as specified in the ITU Radio Regulations are shown below.Table 2.2-1 The frequency allocations related to 13.75-14.0 GHz in the ITU Radio Regulations

There are restrictions regarding antenna size for the 13.75-14 GHz band, and the details of these restrictions are specified in section 5.502.

5.502 In the band 13.75-14 GHz, an earth station of a geostationary fixed-satellite service network shall have a minimum antenna diameter of 1.2 m and an earth station of a non-geostationary fixed-satellite service system shall have a minimum antenna diameter of 4.5 m. In addition, the e.i.r.p., averaged over one second, radiated by a station in the radiolocation or radionavigation services shall not exceed 59 dBW for elevation angles above 2° and 65 dBW at lower angles. Before an administration brings into use an earth station in a geostationary-satellite network in the fixed-satellite service in this band with an antenna diameter smaller than 4.5 m, it shall ensure that the power flux-density produced by this earth station does not exceed: – –115 dB(W/(m2 · 10 MHz)) for more than 1% of the time produced at 36 m above sea level at the low water mark, as officially recognized by the coastal State; – –115 dB(W/(m2 · 10 MHz)) for more than 1% of the time produced 3 m above ground at the border of the territory of an administration deploying or planning to deploy land mobile radars in this band, unless prior agreement has been obtained. For earth stations within the fixed-satellite service having an antenna diameter greater than or equal to 4.5 m, the e.i.r.p. of any emission should be at least 68 dBW and should not exceed 85 dBW. (WRC-03)

Currently, there are restrictions that require an antenna diameter of at least 1.2 m for GSO FSS and at least 4.5 m for non-GSO FSS.

On the other hand, as a result of the discussions at WRC-23, it has been decided to initiate discussions on shared use of smaller diameter antennas for the antenna diameter restrictions, which will be included as Agenda Item 1.2 for the next WRC-27[9].

Agenda for the 2027 world radiocommunication conference 1.2 to consider possible revisions of sharing conditions in the frequency band 13.75-14 GHz to allow the use of uplink fixed-satellite service earth stations with smaller antenna sizes, in accordance with Resolution 129 (WRC-23);

Due to the potential relaxing of antenna size restrictions, the 13.75-14.0 GHz band may also become more accessible for VSAT use in Japan in the future.

Observation2: At the next WRC-27, under Agenda Item 1.2, discussions are planned to explore shared use of smaller diameter antennas to address the restrictions on antenna diameter when using the 13.75 GHz to 14.0 GHz band.

Observation3: There is a possibility of relaxing the ITU's antenna size restrictions, which may become more accessible for the future use of the 13.75-14.0 GHz band for VSAT.

Ku band plan for Region 3

According to the current frequency allocations for Ku-band satellite communications in Japan, the available bands for domestic use are 13.75-14.5 GHz (Earth-to-Space) and 10.7-12.75 GHz (Space-to-Earth). Currently, there are restrictions that require an antenna diameter of at least 1.2 meters for GSO FSS and at least 4.5 meters for non-GSO. However, discussions have been scheduled for the next WRC-27 regarding the shared use of smaller diameter antennas. With the potential relaxation of antenna size restrictions, the 13.75-14.0 GHz band may also become more suitable for VSAT use in Japan in the future. Based on the analysis of Japan’s regulatory framework for Ku-band allocation for satellite communications, it is essential that the Ku-band Band Definitions for Region 3 includes 13.75-14.5 GHz (Earth-to-Space) and 10.7-12.75 GHz (Space-to-Earth).

Proposal1: Based on the analysis of Japan’s regulatory framework for Ku-band allocation for satellite communications, it is essential that the Ku-band Band Definitions for Region 3 includes 13.75-14.5 GHz (Earth-to-Space) and 10.7-12.75 GHz (Space-to-Earth).

- Conclusions

Observation1: The main frequency bands allocated to satellite communications in the Ku band in Japan are 10.7-12.75 GHz (Space-to-Earth) and 13.75-14.5 GHz (Earth-to-Space).

Observation2: At the next WRC-27, under Agenda Item 1.2, discussions are planned to explore shared use of smaller diameter antennas to address the restrictions on antenna diameter when using the 13.75 GHz to 14.0 GHz band.

Observation3: There is a possibility of relaxing the ITU's antenna size restrictions, which may become more accessible for the future use of the 13.75-14.0 GHz band for VSAT.

Proposal1: Based on the analysis of Japan’s regulatory framework for Ku-band allocation for satellite communications, it is essential that the Ku-band Band Definitions for Region 3 includes 13.75-14.5 GHz (Earth-to-Space) and 10.7-12.75 GHz (Space-to-Earth).

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104.

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112

- “MIC The Radio Use Website  Spectrum Charts above 10GHz”, https://www.tele.soumu.go.jp/resource/e/search/myuse/use0303/10g.pdf.

- “Radio Equipment Regulations”, https://www.tele.soumu.go.jp/horei/law_honbun/72081000.html

- “Radio Law Enforcement Regulations”, https://www.tele.soumu.go.jp/horei/law_honbun/72002000.html

- “MIC The Radio Use Website  Process of Frequency Assignment”, https://www.tele.soumu.go.jp/e/adm/freq/process/index.htm

- “MIC The Radio Use Website J204”, https://www.tele.soumu.go.jp/wari/WarikyakServlet?KYAK=J204

- “MIC The Radio Use Website J205”, https://www.tele.soumu.go.jp/wari/WarikyakServlet?KYAK=J205

- “World Radio Communication Conference 2023(WRC-23)Final Acts”, https://www.itu.int/dms_pub/itu-r/opb/act/R-ACT-WRC.16-2024-PDF-E.pdf