3GPP TSG-RAN WG4 Meeting #112bis	R4-2415682

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:	Discussion on spatial channel model for demodulation performance requirements

Agenda item:	6.14.2

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved, which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#108 in June 2025. The objectives are as below:

- Study practical spatial channel modelling methodology for both SU- and MU-MIMO demodulation requirements and CSI reporting requirements: - Identify the limitations of the current (i.e. up to and including Release 18) channel models and corresponding scenarios and how they relate to UE MIMO performance - Consider both Clustered Delay Line (CDL)-based and TDL-based channel modelling approaches - For CDL-based channel modelling, use the tuned repeatable spatial channel model of TR38.827 as the starting point and identify any necessary changes. - Study and verify test methodology feasibility including test complexity and achievable results uncertainty. The test complexity shall not be significantly increased. - The methodology shall include both FR1 (conducted) and FR2 (wireless cable), with first priority for FR1.

In last RAN4#112 meeting, ‘draft pCR’ process, draft of TR skeleton structure and work plan have been agreed [2].  Furthermore, some progress on spatial channel modelling methodology are achieved [2].

In this contribution, we share our views and analysis on the CDL-based model and TDL-based model.

- Discussion

Generally, the channel model with spatial property has been discussed at the beginning of NR, and RAN1 are using geometric based SCM(E) defined in 38.901 for system level simulation since this channel model is more realistic, 38.901 CDL channel model are derived from SCM(E) with some parameters simplified/discarded, and 38.901 TDL are further derived from 38.901 CDL without spatial property [3] as Figure 2-1. RAN4 deceided to use simplified TDL model for NR FR1 demodulation test in May 2018 [6] and simplified TDL model for NR FR2 demodulation test in Aug 2018 [7] since majority companies think TDL is enough for demodulation performance requirements and it is hare to align simulation results using 38.901 CDL at that stage. A New SID [8] on the radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs was approved in June 2018, which targeting for 38.827 approval in Dec 2019 and actually approved in June 2020.

Figure 2-1. Channel Models in 3GPP

  - CDL Based Methodologies

In last RAN4#112 meeting, belows Way forword and agreement are agreed by companies, in which 38.827 Uma-CDL-C is the first priority to be used for initial comparison and alignment.

Cluster Delay Model Way Forward: Prioritise 38.827 UMa-CDL-C for initial comparison and alignment. FFS on parameters. Other options not precluded. UE Antenna Configuration Agreement: UE Antenna Configuration, for DL, for 38.827: Cross Pol (1, n/2, 2), i.e. [ + + + +]. Where ‘n’ is number of Rx with half lambda spacing, with omni antenna element radiation pattern.

The mapping between antenna element and CSI-RS/DMRS ports

Here NLOS fading formulas are used as an example to decsribe the difference between 38.901 CDL and 38.827 CDL.

38.901 CDL

Where u denotes the receive antenna element, s denotes the transmit antenna element, n denotes a cluster and m denotes a ray within cluster n. Hence the 38.901 CDL modelling the channels from transmit antenna elements to the receive antenna elements, which do not modelling the mapping between antenna element and CSI-RS/DMRS ports.

38.827 CDL

Hu,s,nNLOSt=PnMm=1Mne=1Neme=1MeFrx,u,θθn,m,ZOA,φn,m,AOAFrx,u,φθn,m,ZOA,φn,m,AOATexpjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕ

Ftx,s,me,ne,θθn,m,ZOD,φn,m,AODFtx,s,me,ne,φθn,m,ZOD,φn,m,AODαs,me,neexpjΦn,mexpj2πrrx,n,mT.drx,uλ0expj2πrtx,n,mT.dtx,s,me,neλ0expj2πrrx,n,mT.vλ0t

Where u denotes the Rx antenna/beam, s denotes the Tx beam,  n denotes a cluster and m denotes a ray within cluster n, Me denotes the elements per panel in vertical dimension, Ne denotes the elements per panel in horizontal dimension. Therefore, BS beamforming weights αme,nedefined for antenna elements are used and the BS antenna signals are summed for BS beamforming, which means the co-polarized elements of the array are combined to a single RF port (a fix mapping rule between antenna elements and CSI-RS/DMRS ports). Since 38.827 is targeting for OTA test which should include RF, the CDL channel modelling in 38.827 introduce the beamforming weights and combining process accordingly. However, the mapping rule between BS antenna elements and CSI-RS/DMRS ports are actually transparent to UEs, subject to BS algorithm design, and different mapping method may corresponding to different throughput performance. There are no problem for 2T2R scenario using current mapping algorithm defined in 38.827, as one polarization need to support one beam direction. But for 4T4R, one polarization need to support two spatial multiplex layers in higher rank scheme, this would lead the two layers in one polarization share two different beams in multiple CSI-RS ports and the orthogonality between these two layers would be hard to promise. The higher the layers, the worse the performance. Moreover, considering current parameter values of Me and Ne defined in 38.827 do not include the 3 to 1 antenna elements combining, i.e. current mapping algorithm defined in 38.827 only partially modelling the realistic typical product, it seems it’s unreasonable to use this half-baked mapping model for RAN4 demodulation performance requirements.

Observation 1: The mapping rule between BS antenna elements and CSI-RS/DMRS ports are transparent to UEs, subject to BS algorithm design, and different mapping method may corresponding different throughput performance. Current 38.827 defines the co-polarized antenna elements of the array are combined to a single RF port.

Observation 2: Current mapping method for the mapping between antenna elements and CSI-RS/DMRS ports defined in 38.827 is not suitable for 4T4R and 8T8R with high rank modes.

Observation 3: Current parameter values of Me and Ne defined in 38.827 do not include the 3 to 1 antenna elements combining, i.e. current mapping algorithm defined in 38.827 only partially modelling the realistic typical product, it seems it’s unreasonable to use this half-baked mapping model for RAN4 demodulation performance requirements.

Proposal 1: For RAN4 demodulation and CSI reporting simulation, remove the BS beamforming weights αs,me,ne and the antenna array combined to a single RF port process for CDL channel model.

The generation of departure and arrival angles

According current description in 38.827, the departure and arrival angles are generated using below equation

Where n,AOA+ cASAm denotes the arrival angles with ray offsets, and  is the mean angle of the original channel model table. Therefore, the technical effects of above equation is only scaling the ray offsets in addition to the mean angle of the original channel model table, the final scaled angel ϕn,m,AOA,scaled are still fixed due to fluctuating around the mean angle, e.g. Uma CDL-C scaled departure/arrival angles in Figure 2-2. Then the drawbacks mentioned in 38.901 section 7.7.5.1 are still exist, i.e. “The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization”. Hence we prefer to replace the  in the last position by the desired mean angle μϕ,desired.

Figure 2-2. Scaled angles with ray offsets defined for Uma CDL-C in 38.827

Observation 4: The technical effects of current departure and arrival angles generation equation in 38.827 is only scaling the ray offsets in addition to the mean angle of the original channel model table, the final scaled angle ϕn,m,AOA,scaled are still fixed due to fluctuating around the mean angle instead of desired angles.

Proposal 2: Replace the mean angle  in the last position of departure and arrival angles generated equation by the desired angle μϕ,desired as ϕn,m,AOA,scaled=ASdesiredASmodeϕn,AOA+cASAαm-μϕ,model+μϕ,desired

BS antenna configuration

For the convenience of simulation alignments, the antenna configuration of BS side should be defined. Since the agreement is achieved for UE side in last RAN4 meeting as below

UE Antenna Configuration Agreement: UE Antenna Configuration, for DL, for 38.827: Cross Pol (1, n/2, 2), i.e. [ + + + +]. Where ‘n’ is number of Rx with half lambda spacing, with omni antenna element radiation pattern.

The antenna configuration of BS side could select cross Pol with +/- 450 polarization, and the antenna pattern of a single BS antenna element could use the radiation power pattern defined in 38.901 Table 7.3-1 as below

Proposal 3: For BS antenna configuration, select cross Pol with +/- 450 polarization, and the antenna pattern of a single BS antenna element could use the radiation power pattern defined in 38.901 Table 7.3-1.

  - TDL Based Methodologies

TDL parameters for performance comparsion

Considering the legacy TDL models will be used for CDL performance comparision, and CDL model introduces the correlation cofficient for polarizations between BS and UE side as expjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕ, it seems TDLC with ULA Low, which neither include the correlations between polarizations nor the correlation coefficients for different layers (identity matrixs), is an unreasonable and unfair comparison object. Therefore, we prefer to use TDLC X-pol medium correlation as the comparison object for CDL performance comparision.

Observation 5: CDL model introduces the correlation cofficient for polarizations between BS and UE side as expjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕ, while TDLC with ULA Low, which neither include the correlations between polarizations nor the correlation coefficients for different layers (identity matrixs), is an unreasonable and unfair comparison object.

Proposal 4: Prefer to use TDLC X-pol medium correlation as the comparison object for CDL performance comparision.

TDL Model Extension

The motivation of the SID [1] is current TDL model does not have spatial property for RAN4 demodulation/CSI reporting requirements, which are needed for some scenarios e.g. High rank, PMI reporting cases. This target could also be achieved by extending current TDL model through introducing spatial clusters as mentioned in [9] multi-cluster TX-RX beam steering with TDL model.

Observation 6: Multi-cluster TX-RX beam steering with TDL model method could also achieve the target for introducing spatial property, which is easier than CDL model since no antenna patterns and antenna combining weight needed.

  - Common for all Methodologies

Companies agreed to focus on FR1 conducted test as the priority in last RAN4 meeting. About the test cases for SU-MIMO and MU-MIMO, further discussion and analysis are needed. For SU-MIMO, since the two codeword performance issue, 8 layer with 8T8R PDSCH throughput performance should be selected as a starting point. Meanwhile, considering 4 layer with 4T4R is a typical case for PDSCH demodulation test, we think it should be deployed for CDL performance evaluation.

Proposal 5: For CDL evaluation, prefer to use 8 layers with 8T8R and 4 layers with 4T4R as the starting point for PDSCH throughput performance evaluation.

Channel model is a mathematical model of characteristics which reflect the impacts to the transmit signals from the wireless communication environement including the delay, power and spatial properties. Theoretically once the wireless environement is specified, the channel model characteristics are decided, no matther how many UEs are deployed in this environement. The normal process of MU-MIMO modeling is gNB select candidate UEs to be paired according the correlation between them, in which the correlation is highly related with the locations of candidate UEs. For the MU-MIMO modeling in RAN4 demodulation, we usually assume two UEs are already paired, which means the correlation between them are limited, hence the angle desired for paired UEs should be selected carefully. However, the alignment of channel modeling with spatial property for RAN4 is still on the starting stage, it seems too hasty to deploy MU-MIMO simulations since many undetermined factors for the channel modeling with spatial property.

Proposal 6: For CDL evaluation, prioritize the simulation assumption/results alignment of SU-MIMO scenario alignment and defer MU-MIMO scenario.

- Conclusion

In this contribution, we provide analysis and views on SCM topic.

The observations and proposals could be summarized as:

Observation 1: The mapping rule between BS antenna elements and CSI-RS/DMRS ports are transparent to UEs, subject to BS algorithm design, and different mapping method may corresponding different throughput performance. Current 38.827 defines the co-polarized antenna elements of the array are combined to a single RF port.

Observation 2: Current mapping method for the mapping between antenna elements and CSI-RS/DMRS ports defined in 38.827 is not suitable for 4T4R and 8T8R with high rank modes.

Observation 3: Current parameter values of Me and Ne defined in 38.827 do not include the 3 to 1 antenna elements combining, i.e. current mapping algorithm defined in 38.827 only partially modelling the realistic typical product, it seems it’s unreasonable to use this half-baked mapping model for RAN4 demodulation performance requirements.

Proposal 1: For RAN4 demodulation and CSI reporting simulation, remove the BS beamforming weights αs,me,ne and the antenna array combined to a single RF port process for CDL channel model.

Observation 4: The technical effects of current departure and arrival angles generation equation in 38.827 is only scaling the ray offsets in addition to the mean angle of the original channel model table, the final scaled angle ϕn,m,AOA,scaled are still fixed due to fluctuating around the mean angle instead of desired angles.

Proposal 2: Replace the mean angle  in the last position of departure and arrival angles generated equation by the desired angle μϕ,desired as ϕn,m,AOA,scaled=ASdesiredASmodeϕn,AOA+cASAαm-μϕ,model+μϕ,desired

Proposal 3: For BS antenna configuration, select cross Pol with +/- 450 polarization, and the antenna pattern of a single BS antenna element could use the radiation power pattern defined in 38.901 Table 7.3-1.

Observation 5: CDL model introduces the correlation cofficient for polarizations between BS and UE side as expjΦmθθκn,m-1expjΦmθϕκn,m-1expjΦmϕθexpjΦmϕϕ, while TDLC with ULA Low, which neither include the correlations between polarizations nor the correlation coefficients for different layers (identity matrixs), is an unreasonable and unfair comparison object.

Proposal 4: Prefer to use TDLC X-pol medium correlation as the comparison object for CDL performance comparision.

Observation 6: Multi-cluster TX-RX beam steering with TDL model method could also achieve the target for introducing spatial characters, which is easier than CDL model since no antenna patterns and antenna combining weight needed.

Proposal 5: For CDL evaluation, prefer to use 8 layers with 8T8R and 4 layers with 4T4R as the starting point for PDSCH throughput performance evaluation.

Proposal 6: For CDL evaluation, prioritize the simulation assumption/results alignment of SU-MIMO scenario alignment and defer MU-MIMO scenario.

- Reference

[1] RP-241610, New SID: Study on spatial channel model for demodulation performance requirements, RAN#104 meeting, Nokia, BT Plc, AT&T, Bell Mobility, Bouygues Telecom, China Telecom, CMCC, Deutsche Telekom, Ericsson, Intel Corporation, KDDI, Keysight, KT Corp., MediaTek, NTT Docomo, Orange, Qualcomm, Rohde & Schwarz, Samsung, SK Telecom, Spark NZ, Telecom Italia, Telefonica, Telenor, Telia Company, Telstra, T-Mobile USA, Verizon, Vodafone, ZTE Corporation

[2] R4-2413606, Way Forward for [112][327] FS_NR_demod_SCM, RAN4#112 meeting, Nokia

[3] 	3GPP TR 38.901 V18.0.0 (2024-03), Study on channel model for frequencies from 0.5 to 100 GHz

[4] 	3GPP TR 38.827 V16.8.0 (2022-09), Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR User Equipment (UE)

[5] 	3GPP TS 38.101-4 V18.4.0 (2024-06), User Equipment (UE) radio transmission and reception; Part 4: Performance requirements

[6] R4-1808400, NR Test Methods SI ad-hoc meeting notes, RAN4 #87, Intel Corporation

[7] R4-1812001, RAN4#88 Meeting report, RAN4#88

[8] RP-181325, New SID on Study on radiated metrics and test methodology for the verification of multi-antenna reception performance of NR UEs, RAN#80, CATR, OPPO, Samsung

[9] R4-2412328, Discussion on Spatial Channel Modelling, RAN4#112 meeting, MediaTek Inc.