3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415683

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:	Discussion on UE demodulation performance requirements for 8Rx with MMSE-IRC

Agenda item:	6.16.2

Document for:	Discussion

- Introduction

In the RAN#104 meeting, the revised WID [1] for “NR demodulation performance: Phase 5” has been approved, in which additional scenarios for “UE performance requirements for 8Rx” is an important objective for the enhancement of UE demodulation performance parts. The latest objectives of UE demodulation performance enhancements as below:

- Define the following UE performance requirements for 8Rx CPE/FWA/vehicle/industrial devices: - PDSCH demodulation and CQI reporting requirements with inter-cell interference with MMSE-IRC receiver - Reuse the Rel-17 interference model for 2/4Rx UE as a baseline - PDSCH demodulation requirements with intra-cell inter-user interference with MMSE-IRC receiver - Reuse the existing interference model for 2/4Rx UE as a baseline

Generally, the enhancement focus on two scenarios, the first one is the inter-cell interference with MMSE-IRC receiver scenario, the second one is the intra-cell inter-user interference with MMSE-IRC receiver scenario.

Last RAN4#112 meeting is the first meeting for this topic, several high level agreements are achieved [2]. We provide our further analysis and views on the test scopes and simulation assumptions in this contribution.

- Discussion

  - General

Test scope

In last RAN4#112 meeting, for both 8Rx inter-cell interference scenario and 8Rx intra-cell inter-user interference scenario, companies agreed to introduce UE demodulation requirements for both FDD and TDD and not to include PDSCH absolute physical layer throughput requirements. However, for the CQI reporting requirements of inter-cell interference scenario, it’s still under discussion. For the CQI reporting requirements of inter-cell interference with MMSE-IRC receiver, test requirements has been standardized for 2Rx and 4Rx UE antennas, which are using 2x2 and 2x4 antenna configurations seperately and could verify the CQI processing with inter-cell interference enough. For 8Rx UE antennas, it seems no CQI related processing difference with 4Rx cases, hence, we think it’s not necessary to introduce CQI reporting requirements for 8Rx UE antennas with inter-cell interference scenario.

Proposal 1: Prefer not to introduce CQI reporting requirements for 8Rx UE antennas with inter-cell interference with MMSE-IRC receiver scenario since no CQI related processing difference with 4Rx cases.

8Rx UE MMSE-IRC receiver assumption

In Rel-18, the logic for introducing both baseline and simplified SU-MIMO 8Rx receivers is obvious performance difference could be observed. Therefore, for both 8Rx inter-cell interference scenario and 8Rx intra-cell inter-user interference scenario, requirements should cover both baseline and simplified 8Rx receiver once obvious performance difference could be observed.

Proposal 2: For both 8Rx inter-cell interference scenario and 8Rx intra-cell inter-user interference scenario, requirements should cover both baseline and simplified 8Rx receiver once obvious performance difference could be observed.

PDSCH mapping type

For the PDSCH mapping type, considering our target is to define the minimum requirements of 8Rx inter-cell interference scenario and 8Rx intra-cell inter-user interference scenario, rather than to verify the difference between PDSCH mapping type A and type B, hence it seems there is no need to include two PDSCH mapping types at the same time.

Proposal 3: For 8Rx inter-cell interference scenario and 8Rx intra-cell inter-user interference scenario, reuse the same configuration for Rel-17 2/4Rx, i.e., PDSCH mapping typeA only.

  - PDSCH requirements for inter-cell interference with MMSE-IRC receiver scenario

Interference modelling and power levels

For the inter-cell interference with MMSE-IRC receiver scenario, PDSCH demodulation test requirements has been standardized for 2Rx and 4Rx UE antennas, which are using 2x2 and 2x4 antenna configurations seperately. For 8Rx UE antennas, it’s a good starting point to use current interference model for PDSCH requirements. Furthermore, since current requirements are only based on rank 1 for the serving cell, it’s better to check if the enhancement is still focusing on rank 1. At the same time, considering 64QAM and 256QAM are more typical than 16QAM in NR, if it is still reasonable to use 16QAM based randomly modulated data for each interfering cell over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel may need more discussion.

Proposal 4: For 8Rx inter-cell interference scenario, clarify the modulation order based randomly modulated data for each interfering cell over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel, whether 16QAM is still suitable or not.

Network Type and SCS/CBW

For the network type and SCS/CBW, we assume it’s reasonable to reuse the configuration in Rel-17.

Proposal 5: For 8Rx inter-cell interference scenario, reuse the same configuration defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U)

Propagation condition

For the propagation condition, we assume it’s reasonable to reuse the configuration in Rel-17.

Proposal 6: For 8Rx inter-cell interference scenario, reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

Rank

Current requirements for 2Rx and 4Rx are only based on rank 1 for the serving cell, however, since 8Rx could support higher rank values, it is reasonable to consider higher rank for 8Rx.

Proposal 7: For 8Rx inter-cell interference scenario, consider higher rank for 8Rx, RAN4 to investigate rank 1, 2, 4 and choose a reasonable requirement value.

MCS

For Inter-cell interference with MMSE-IRC receiver scenario, current PDSCH demodulation requirements for 2Rx and 4Rx are all based on MCS13. Take the better performance of 8Rx compared to 2/4 Rx into account, MCS values may need to be updated.

Proposal 8: For 8Rx inter-cell interference scenario, select MCS values together with rank values, e.g., higher MCS value corresponding lower rank value, lower MCS value corresponding higher rank value.

Antenna and MIMO correlation

For the antenna configuration, it’s no doubt the number of UE receive antennas should be 8. For the number of BS transmit antennas, on the one hand, from the test coverage point of view, 2, 4 and 8 should be all covered, on the other hand, the test effort should be considered.

For the MIMO correlation, we suppose the simulation assumptions for Rel-18 8Rx have been deeply discussed. Therefore, use the same MIMO correlation configuration as the Rel-18 8Rx are suitable, i.e. ULA medium B for 2T8R, ULA low for 4T8R and 8T8R.

Proposal 9: For 8Rx inter-cell interference scenario, select one or two antenna configurations from 2T8R, 4T8R and 8T8R, and reuse the same MIMO correlation configuration as the Rel-18 8Rx, i.e., ULA medium B for 2T8R, ULA low for 4T8R and 8T8R.

  - PDSCH requirements for intra-cell inter-user interference with MMSE-IRC receiver scenario

For the intra-cell interference with MMSE-IRC receiver scenario, PDSCH demodulation test requirements has been standardized for 2Rx and 4Rx UE antennas with multiple UE pairing, which are using 2x2 and 4x4 antenna configurations seperately.

For 8Rx UE antennas, it’s a good starting point to use current test assumptions for MU-MIMO PDSCH demodulation requirements.

Network Type and SCS/CBW

For the network type and SCS/CBW, we assume it’s reasonable to reuse the configuration in Rel-17.

Proposal 10: For 8Rx intra-cell inter-user interference scenario, reuse the same configuration defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U)

Interference modelling

For the MU-MIMO beamforming model, we suppose it’s still workable to reuse the same MU-MIMO beamforming model for Rel-17 in 28.101-4 B.4.2. For the number of co-UE, it seems 1-co-UE is enough to verify the performance of 8Rx intra-cell inter-user interference scenario.

Proposal 11: For 8Rx intra-cell inter-user interference scenario, reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4, and consider 1 target UE + 1 co-UE configuration.

Propagation condition

For the propagation condition, reuse the same configuration as Rel-17 is a suitable selection.

Proposal 12: For 8Rx intra-cell inter-user interference scenario, reuse the same propagation condition as Rel-17 is a suitable selection, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2.

Precoding method for the co-scheduled UE

Considering the MU-MIMO pairing scheduling strategy in the network is highly related with the long term beam weights but not with the short term weight i.e. PMIs, it seems it is too idealization to use orthogonal PMI selection for co-scheduled UE with the target UE. Thus, it is better to use the randomized precoder which is not equal to any column of the precoder matrix of target UE. However, due to Rel-17 MU-MIMO requirements are based on both random and orthogonal PMI selection, we think it’s an acceptable way to proceed.

Proposal 13: For 8Rx intra-cell inter-user interference scenario, reuse the same precoding method as Rel-17 MMSE-IRC, i.e., random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests.

Antenna, rank (target UE + co-scheduled UEs) configuration and MIMO correlation

For the ranks in 2Rx antennas deployment, 1+ 1 for the target UE and co-scheduled UE, while for the ranks in 4Rx antennas deployment, 2+ 2 for the target UE and co-scheduled UE. The number of Tx antennas and ranks should be specified for the simulation campaign for 8Rx with intra-cell inter-user interference. Rank 1+1 for 2T8R, Rank 2+2 for 4T8R and Rank 4+4 for 8T8R should be an appropriate configuration.

Proposal 14: For 8Rx intra-cell inter-user interference scenario, the antenna and rank assumptions could be Rank 1+1 for 2T8R, Rank 2+2 for 4T8R and Rank 4+4 for 8T8R.

For the MIMO correlation assumption, we suppose the simulation assumptions for Rel-18 8Rx have been deeply discussed. Therefore, use the same MIMO correlation configuration as the Rel-18 8Rx are suitable, i.e. ULA medium B for 2T8R, ULA low for 4T8R and 8T8R.

Proposal 15: For 8Rx intra-cell inter-user interference scenario, the antenna correlation assumptions could be as same as Rel-18 8Rx, i.e., ULA medium B for 2T8R, ULA low for 4T8R and 8T8R.

MCS

For MCS value of the target UE, we prefer to align with Rel-17 MU-MIMO requirements i.e., MCS 13.

Proposal 16: For 8Rx intra-cell inter-user interference scenario, Same MCS value as Rel-17 2/4Rx requirements for MMSE-IRC, i.e., MCS 13.

  - Wideband CQI reporting requirements for inter-cell interference with MMSE-IRC receiver scenario

CQI calculation algorithm

For the CQI calculation algorithm, since the test metric is the ratio γ of the throughput instead of absolute throughput value, it seems it’s not necessary to define two CQI calculation assumptions to match two 8Rx receiver assumptions.

Proposal 17: For 8Rx inter-cell interference scenario, it’s not necessary to define two CQI calculation assumptions to match two 8Rx receiver assumptions.

Test metric

For the CQI test metric, below legacy test metric should be workable.

a) the ratio of the throughput obtained when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR and that obtained when transmitting the transport format indicated by each reported wideband CQI index subject to a white Gaussian noise source shall be ≥  b) when transmitting the transport format indicated by each reported wideband CQI index subject to an interference source with specified INR, the average BLER for the indicated transport formats shall be greater than or equal to X%.

Proposal 18: For 8Rx inter-cell interference scenario, reuse legacy CQI test metric of Rel-17 2/4Rx.

Propagation condition

For the propagation condition, reuse the same configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell should be a suitable starting point.

Proposal 19: For 8Rx inter-cell interference scenario, reuse the same propagation configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

Interference power level

For the interference power level, we assume reuse the same configuration of Rel-17 2/4Rx is enough.

Proposal 20: For 8Rx inter-cell interference scenario, reuse the same interference power level for Rel-17 2/4Rx, i.e., INR=10.04dB with rank 1.

Antenna configuration and MIMO correlation for the serving cell

For the antenna configuration and MIMO correlation for the serving cell, we propose the similar configuration as legacy 2Rx and 4Rx, i.e. 2T8R with ULA Low.

Proposal 21: For 8Rx inter-cell interference scenario, similar antenna and MIMO correlation configuration for the serving cell as legacy Rel-17 2/4Rx could be used, i.e., 2T8R with ULA Low.

Rank

For the rank value, we assume reuse the same configuration of Rel-17 2/4Rx is enough.

Proposal 22: For 8Rx inter-cell interference scenario, reuse the same rank configuration for Rel-17 2/4Rx, i.e., rank 1.

Tested SINR value, throughput ratio and BLER X%

For the tested SINR value, throughput ratio and BLER X%, we suppose they should be decided based on further simulation results.

Proposal 23: For 8Rx inter-cell interference scenario, the tested SINR value, throughput ratio and BLER X% should be decided based on further simulation results.

CSI-IM and NZP CSI-RS configurations

For the CSI-IM and NZP CSI-RS configurations, we suppose the same configuration as Rel-17 2/4Rx should be used.

Proposal 24: For 8Rx inter-cell interference scenario, the CSI-IM and NZP CSI-RS configurations should be configured as same as Rel-17 2/4Rx cases.

  - Applicability rule and release independent

Test applicability

In order to balance the test coverage and test workload, we prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined.

Proposal 25: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined.

Release independent

Since 8Rx is introduced in Rel-18, we think it is reasonable to define the new requirements for 8Rx should be release independent from Rel-18.

Proposal 26: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18.

- Conclusion

In this contribution, we provide analysis and views on NR demodulation performance: Phase 5.

The observations and proposals could be summarized as:

Proposal 1: Prefer not to introduce CQI reporting requirements for 8Rx UE antennas with inter-cell interference with MMSE-IRC receiver scenario since no CQI related processing difference with 4Rx cases.

Proposal 2: For both 8Rx inter-cell interference scenario and 8Rx intra-cell inter-user interference scenario, requirements should cover both baseline and simplified 8Rx receiver once obvious performance difference could be observed.

Proposal 3: For 8Rx inter-cell interference scenario and 8Rx intra-cell inter-user interference scenario, reuse the same configuration for Rel-17 2/4Rx, i.e., PDSCH mapping typeA only.

Proposal 4: For 8Rx inter-cell interference scenario, clarify the modulation order based randomly modulated data for each interfering cell over the entire PDSCH region and over the full transmission bandwidth of the specified reference measurement channel, whether 16QAM is still suitable or not.

Proposal 5: For 8Rx inter-cell interference scenario, reuse the same configuration defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U)

Proposal 6: For 8Rx inter-cell interference scenario, reuse the same propagation condition defined for Rel-17 2/4Rx, i.e., TDLC300-100 for 2 interfering cells (HomNet) and TDLA30-10 for 1 interfering cell (HetNet)

Proposal 7: For 8Rx inter-cell interference scenario, consider higher rank for 8Rx, RAN4 to investigate rank 1, 2, 4 and choose a reasonable requirement value.

Proposal 8: For 8Rx inter-cell interference scenario, select MCS values together with rank values, e.g., higher MCS value corresponding lower rank value, lower MCS value corresponding higher rank value.

Proposal 9: For 8Rx inter-cell interference scenario, select one or two antenna configurations from 2T8R, 4T8R and 8T8R, and reuse the same MIMO correlation configuration as the Rel-18 8Rx, i.e., ULA medium B for 2T8R, ULA low for 4T8R and 8T8R.

Proposal 10: For 8Rx intra-cell inter-user interference scenario, reuse the same configuration defined for Rel-17 2/4Rx, i.e., 15kHz/10MHz for FDD, and 30kHz/40MHz for TDD with 7D1S2U(S=6D+4G+4U)

Proposal 11: For 8Rx intra-cell inter-user interference scenario, reuse the same MU-MIMO Beamforming Model for Rel-17 2/4Rx in B.4.2 in TS38.101-4, and consider 1 target UE + 1 co-UE configuration.

Proposal 12: For 8Rx intra-cell inter-user interference scenario, reuse the same propagation condition as Rel-17 is a suitable selection, i.e., TDLC300-100 with rank 1+1 and TDLA30-10 with rank 2+2.

Proposal 13: For 8Rx intra-cell inter-user interference scenario, reuse the same precoding method as Rel-17 MMSE-IRC, i.e., random precoding with rank 1+1 and orthogonal precoding for rank 2+2 tests.

Proposal 14: For 8Rx intra-cell inter-user interference scenario, the antenna and rank assumptions could be Rank 1+1 for 2T8R, Rank 2+2 for 4T8R and Rank 4+4 for 8T8R.

Proposal 15: For 8Rx intra-cell inter-user interference scenario, the antenna correlation assumptions could be as same as Rel-18 8Rx, i.e., ULA medium B for 2T8R, ULA low for 4T8R and 8T8R.

Proposal 16: For 8Rx intra-cell inter-user interference scenario, Same MCS value as Rel-17 2/4Rx requirements for MMSE-IRC, i.e., MCS 13.

Proposal 17: For 8Rx inter-cell interference scenario, it’s not necessary to define two CQI calculation assumptions to match two 8Rx receiver assumptions.

Proposal 18: For 8Rx inter-cell interference scenario, reuse legacy CQI test metric of Rel-17 2/4Rx.

Proposal 19: For 8Rx inter-cell interference scenario, reuse the same propagation configuration for Rel-17 2/4Rx, i.e., TDLA30-5 for the serving cell and AWGN for the interference cell.

Proposal 20: For 8Rx inter-cell interference scenario, reuse the same interference power level for Rel-17 2/4Rx, i.e., INR=10.04dB with rank 1.

Proposal 21: For 8Rx inter-cell interference scenario, similar antenna and MIMO correlation configuration for the serving cell as legacy Rel-17 2/4Rx could be used, i.e., 2T8R with ULA Low.

Proposal 22: For 8Rx inter-cell interference scenario, reuse the same rank configuration for Rel-17 2/4Rx, i.e., rank 1.

Proposal 23: For 8Rx inter-cell interference scenario, the tested SINR value, throughput ratio and BLER X% should be decided based on further simulation results.

Proposal 24: For 8Rx inter-cell interference scenario, the CSI-IM and NZP CSI-RS configurations should be configured as same as Rel-17 2/4Rx cases.

Proposal 25: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, prefer to define a test applicability rule to skip 2/4Rx requirements if 8Rx tests are passed, if same test scenarios/configurations as 2RX/4RX are defined.

Proposal 26: For 8Rx inter-cell interference scenario and intra-cell inter-user interference scenario, the new requirements should be release independent from Rel-18.

- Reference

[1] RP-241297, Revised WID on NR demodulation performance: Phase 5, RAN#104 meeting, China Telecom, NTT DOCOMO

[2] R4-2413570, WF on NR demodulation performance: Phase 5, RAN4#112 meeting, China Telcom