3GPP TSG-RAN WG4 Meeting #112-bis	R4-2415684

Hefei, Anhui, China, 14th – 18th October, 2024

Source:	Samsung

Title:	Discussion on RRM requirements for on-demand SSB SCell operation

Agenda item:	6.22.2.1

Document for:	Discussion

- Introduction

The SID [1] for “New SID: Study on spatial channel model for demodulation performance requirements” has been approved in the RAN#104 meeting [2] and updated in the RAN#105 meeting [3], which starts from RAN4#112 in Aug 2024 and targeting to complete by RAN#108 in June 2025. The objectives are as below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In last RAN4#112 meeting, the work plan and several high level issues have been agreed [7] as below.

### Sub-topic 1-1: General OD-SSB requirements Issue 1-1-1: OD-SSB work plan - Agreement: - RAN4 to first focus on the MAC CE based single SCell activation case as starting point. - After RAN4 concludes on the above case, RAN4 may discuss other Scell activation scenarios if needed. Issue 1-1-2: OD-SSB scenarios - Agreement: - RAN4 to discuss OD-SSB based SCell activation requirements based on following RAN1 agreed scenarios. - Case #1 and Scenario #2 - Case #1 and Scenario #2A - Case #2 and Scenario #2 - Case #2 and Scenario #2A - Note 1: Follow RAN1 definition on the scenarios and cases. - Note 2: This list can be updated pending on RAN1 further agreement. Issue 1-1-3: Deactivated SCell measurement requirement - Agreement: - RAN4 to discuss OD-SSB based deactivated Scell measurement and Scell activation requirement. - Includes both FR1 and FR2-1 ### Sub-topic 1-2: OD-SSB requirements (Case 1) Deactivated SCell L3 measurement part Issue 1-2-1: Case 1- OD-SSB based deactivated SCell measurement - Proposals - Option 1: For deactivated SCell measurement requirements, follow the periodicity of configured OD-SSB (not the measurement cycle). - Option 2: For deactivated SCell measurement requirements, follow the measCycleSCell. - Other option is not precluded Issue 2-2-2: Case 1 - OD-SSB based deactivated SCell measurement before OD-SSB indication - Agreement: - In (Case #1 Scenario #2) and (Case #1, Scenario #2A), UE is not expected to perform the SSB based deactivated SCell L3 measurement before OD-SSB is triggered. - Note: How to differentiate OD-SSB SCell and normal SCell depends on other WGs’ discussion. Issue 2-2-10: Case #1 and Scenario #2A - unknown SCell activation - Wayforward: - Option 1: RAN4 to only define unknown SCell activation provided that the OD-SSB is the first OD-SSB triggered by NW for (Case #1, Scenario #2A). - Option 2: RAN4 only specifies requirements assuming the target SCell is unknown in (Case #1, Scenario #2A). - Option 3: RAN4 needs to further consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement. - Other option is not precluded

In this contribution, we share our views and analysis on the on-demand SSB related requirements.

- Discussion

SCell activation scenarios

For the on-demand SSB SCell indication, both RRC based signaling and MAC CE based signaling are supported, and more details about RRC based signaling are clarified as below in last RAN1#118 meeting as “this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration”. From the description of RRC based signaling, it seems the RRC based SCell activation should be supported, hence related RAN4 requirements for on-demand SSB should be discussed and defined. However, in last RAN4#112 meeting, in order to simplify the work at the beginning of this WI, companies agreed to first focus on the MAC CE based single SCell activation case as starting point. Considering current agreement on RRC based signaling to indicate on-demand SSB in RAN1, RAN4 may have to discuss the direct SCell activation scenario.

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Observation 1: For the on-demand SSB SCell indication, both RRC based signaling and MAC CE based signaling are supported, and more details about RRC based signaling are clarified in RAN1 as “this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration”. Therefore, RAN4 may have to discuss the direct SCell activation scenario in addition to MAC CE based SCell activation.

Deactivated SCell L3 measurement part

Case 1- OD-SSB based deactivated SCell measurement

For the deactivated SCell measurement, currently, the time periods are defined based on measCycleSCell which is ENUMERATED from {sf160, sf256, sf320, sf512, sf640, sf1024, sf1280}. On the one hand, the duration of on-demand SSB should be long enough to guarantee the know cell condition, otherwise current definition is not suitable anymore. On the other hand, long on-demand SSB duration may lose the gain of network energy saving. Therefore, it’s better to define the deactivated SCell measurement requirements based on the periodicity of on-demand SSB instead of measCycleSCell and/or DRX cycle. Latest RAN1 agreements mentioned the periodicity of on-demand SSB can be configured and indicated.

Agreement For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters

Proposal 1: For deactivated SCell measurement requirements of Case 1, follow the periodicity of on-demand SSB instead of measCycleSCell and/or DRX cycle.

Case 2- OD-SSB based deactivated SCell measurement

For Case 2, since both on-demand SSB and legacy always-on SSB may existing, the optimal solution should be using available signals as much as possible. And once UE receiving on-demand SSB indication, on-demand SSB should be at high priority if on-demand SSB and always-on SSB have conflict with each other. Therefore, for deactivated SCell measurement requirements of Case 2, follow the periodicity of on-demand SSB instead of measCycleSCell and/or DRX cycle once UE receiving on-demand SSB indication, otherwise follow measCycleSCell and/or DRX.

Proposal 2: For deactivated SCell measurement requirements of Case 2, follow the periodicity of on-demand SSB instead of measCycleSCell and/or DRX cycle once UE receiving on-demand SSB indication, otherwise follow legacy time period definition using measCycleSCell and/or DRX.

Known/Unknown SCell

There are different SCell activation time requirements for known SCell and unknown SCell due to different synchronization time. For the SCell is known or unknown, we have below definition based on valid measurement reports and detectable conditions during a time period. Considering the potential new time periods defined for on-demand SSB and the short duration character of on-demand SSB, the definition of known/unknown SCell for on-demand SSB should be updated.

SCell in FR1 is known if it has been meeting the following conditions: - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - the UE has sent a valid measurement report for the SCell being activated and - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - the UE has sent a valid L3-RSRP measurement report with SSB index, and - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown.

Meanwhile, as the on-demand SSB indication could be before the SCell activation command (Scenario#1, #2), together with the SCell activation command (Scenario#2A), after the SCell activation command (Scenario#3, #3A), for the SCell activation time, from our point of view, there is no need to discuss/define requirements per scenario but just define requirements based on known/unknown SCell activation.

Observation 2: For the on-demand SSB, current known/unknown definition is inappropriate anymore.

Proposal 3: The definition of known/unknown SCell should be updated for on-demand SSB, and should be discussed and decided before the SCell activation time/delay requirements.

Proposal 4: RAN4 to define SCell activation time/delay requirements based on the known/unknown cases, without specific requirements for different scenarios (Scenario#2, #2A, etc.).

Measurement results

If only on-demand SSB or only legacy always-on SSB exists, there is no doubt the L1/L3 measurements should be based on existing SSB (on-demand SSB or legacy always-on SSB). For the case when both on-demand SSB and legacy always-on SSB are existing, how to handle the measurement results of on-demand SSB and legacy always-on SSB is an issue need further discussion. Per our understanding, during the same time period, the more the SSB measurement samples, the higher accuracy could be achieved. Hence, if the same frequency offset and the same time offset could be promised between on-demand SSB and legacy always-on SSB, all measurement results (both on-demand SSB based and legacy always-on SSB based) during the measurement period should be considered. However, if there is UE receiving conflict between on-demand SSB and always-on SSB, on-demand SSB should have priority over legacy always-on SSB.

Proposal 5: For the L1/L3 measurement, if the same frequency offset and the same time offset could be promised between on-demand SSB and legacy always-on SSB, all measurement results (both on-demand SSB based and legacy always-on SSB based) during the measurement period should be considered.

Proposal 6: If there is UE receiving conflict between on-demand SSB and always-on SSB, on-demand SSB should have priority over legacy always-on SSB.

LS from RAN1

Latest RAN1 agreements about the timeline for on-demand SSB operation on SCell are as below [6]. In last meeting, they agreed the working assumption that the value of T_min for MAC-CE based on-demand SSB indication is the same value as UE’s MAC-CE processing time for SCell activation command, and need our RAN4’s confirmation about this working assumption.

Agreement The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ m+3Nslotsubframe,μ+1m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ m+3Nslotsubframe,μ+1 - FFS: Whether the value of T is predefined or indicated/configured by gNB - (Working assumption) T=T_min - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

From this description, we know if UE receives the MAC CE signalling which indicates the on-demand SSB transmission in slot n, then slot n+m is for PUCCH transmission with HARQ-ACK information, and at least until slot n+m+3Nslotsubframe,μ+1 is the slot for on-demand SSB burst(s) transmission as Figure 2-1, which is as same as the timeline required for UE’s MAC CE processing for SCell activation.

Figure 2-1. Channel Models in 3GPP

Proposal 7: RAN4 reply LS to RAN1 to confirm the working assumption of T_min= m+3Nslotsubframe,μ+1 can be as an agreement.

- Conclusion

In this contribution, we provide analysis and views on on-demand SSB related topic.

The observations and proposals could be summarized as:

Observation 1: For the on-demand SSB SCell indication, both RRC based signaling and MAC CE based signaling are supported, and more details about RRC based signaling are clarified in RAN1 as “this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration”. Therefore, RAN4 may have to discuss the direct SCell activation scenario in addition to MAC CE based SCell activation.

Proposal 1: For deactivated SCell measurement requirements of Case 1, follow the periodicity of on-demand SSB instead of measCycleSCell and/or DRX cycle.

Proposal 2: For deactivated SCell measurement requirements of Case 2, follow the periodicity of on-demand SSB instead of measCycleSCell and/or DRX cycle once UE receiving on-demand SSB indication, otherwise follow legacy time period definition using measCycleSCell and/or DRX.

Observation 2: For the on-demand SSB, current known/unknown definition is inappropriate anymore.

Proposal 3: The definition of known/unknown SCell should be updated for on-demand SSB, and should be discussed and decided before the SCell activation time/delay requirements.

Proposal 4: RAN4 to define SCell activation time/delay requirements based on the known/unknown cases, without specific requirements for different scenarios (Scenario#2, #2A, etc.).

Proposal 5: For the L1/L3 measurement, if the same frequency offset and the same time offset could be promised between on-demand SSB and legacy always-on SSB, all measurement results (both on-demand SSB based and legacy always-on SSB based) during the measurement period should be considered.

Proposal 6: If there is UE receiving conflict between on-demand SSB and always-on SSB, on-demand SSB should have priority over legacy always-on SSB.

Proposal 7: RAN4 reply LS to RAN1 to confirm the working assumption of T_min= m+3Nslotsubframe,μ+1 can be as an agreement.

- Reference

[1] RP-234061, “New WID: Enhancements of network energy savings for NR”, Ericsson

[2] RP-241650, “Revised WID: Enhancements of network energy savings for NR”, RAN#104 meeting, Ericsson, Apple

[3] RP-242354, “Revised WID: Enhancements of network energy savings for NR”, RAN#105 meeting, Ericsson

[4] RP-241182, “Status Report for WI – Enhancements of network energy saving for NR”, RAN#104 meeting, Ericsson, Apple

[5] RP-241866, “Status Report for WI – Enhancements of network energy saving for NR”, RAN#105 meeting, Ericsson, Apple

[6] R1-2407565, “LS on timeline for On-demand SSB operation on SCell”, RAN1#118 meeting

[7] R4-2413885, “WF on Netw_Energy_NR_enh”, RAN4#112 meeting, Ericsson