3GPP TSG-RAN WG4 Meeting #112bis	R4-2415687

Hefei, China, October 14th – 18th, 2024

Agenda item:	6.17.1

Source:	Intel Corporation

Title:	Views on general aspects of AI/ML testability and interoperability

Document for:	Discussion

# Introduction

In Rel-18 RAN WG4 conducted detailed studies on the testability and interoperability aspects of AI/ML enabled techniques for NR air interface as a part of FS_NR_AIML_Air SI [1] and the conclusions are summarized in TR 38.843 [2]. A new WI on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN #102 meeting with the latest version available in [3] with the following RAN4 objectives (the study stage objectives related to  RAN4 are marked in yellow). Furthermore, in accordance with the agreements in RAN #105 [4] the RAN4 work on the testability and interoperability was extended till the end of Rel-19.

### Objective of SI or Core part WI or Testing part WI Provide specification support for the following aspects: … - AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]: … - Beam management - DL Tx beam prediction for both UE-sided model and NW-sided model, encompassing [RAN1/RAN2]: … - Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - CSI feedback enhancement, encompassing [RAN1/RAN2]: - CSI prediction (UE-sided model): … - Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. … Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - … - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis NOTE: offline training is assumed for the purpose of this project. NOTE: the outcome of the study objectives should be captured in TR 38.843 for future reference. ### Objective of Performance part WI - For Beam Management and Positioning Accuracy enhancement use cases, specify performance requirements and test cases for AI/ML LCM procedures (including performance monitoring) and UE features enabled by UE-sided models - Specify necessary performance requirements and tests (including metrics) for the above-mentioned use cases - Specify necessary test cases and performance requirements for LCM procedure, including performance monitoring.

The work item stage discussions took place in RAN4 #110, #110bis, #111 and #112 with agreements summarized in [4-7]. In this contribution we provide views and proposals on the remaining issues of general aspects of AI/ML testability.

# Discussion

## Post-deployment validation due to model change/drift

The topic of post-deployment functionality/drift validation was discussed during both SI and WI stages with the following conclusions.

RAN4 #108bis (moderator summary) Issue 1-9: Post deployment testing - Proposals - Proposal 1: RAN4 should study a framework to enable post deployment tests for model updates and/or drift validation (and possible other use cases) - Following options can be taken for reference in further discussion: - Option 1- a: The changes/updates to the ML-enabled Functionality/Feature are tested and declared by the device vendor against RAN4 requirements before any deployment to the UE is performed. - Option 1- b: After deployment to the UE and before changed/updated ML-enabled Functionality/Feature is activated in the UE, a post-deployment validation is performed, e.g., a sanity check test loop is run, e.g., using the functionality performance monitoring and LCM activation/deactivation/switching procedures, - Option 1- c: At least one fallback/default Functionality/Feature that passed conformance testing must always be present in the device. - Other options can also be discussed - Proposal 2: RAN4 does not need to study such framework - Proposal 3: others, please provide some proposals RAN4 #109 (moderator summary) Issue 1-7: Tests post-deployment - Proposals - Proposal 1: The post deployment testing should be based on the model monitoring framework Postpone the discussion to a future release, possible as a study part of Rel-19 WI - Proposal 2: RAN4 should study the ways to validate performance after model updates and/or detected drift and discuss at least the following non-mutually exclusive options: - The changes/updates to the ML-enabled Functionality/Feature are tested and declared by the device vendor against RAN4 requirements before any deployment to the UE is performed. - After deployment to the UE and before changed/updated ML-enabled Functionality/Feature is activated in the UE, a post-deployment validation is performed, e.g., a sanity check test loop is run, e.g., using the functionality performance monitoring and LCM activation/deactivation/switching procedures, - At least one fallback/default Functionality/Feature that passed conformance testing must always be present in the device. - Proposal 3: There is no need for post-deployment testing - Proposal 4: other RAN4 #110 (WF) Issue 1-2: Post deployment handling Agreement: - To ensure the AI performance after device deployment, discuss the following options further - Option 1: Conduct the conformance testing for AI model/functionality before deployment - FFS on the feasibility - Option 2: Design the test to verify the performance monitoring - Depend on the other WG progress - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable - Other options are not precluded RAN4 #110bis (moderator summary) Issue 1-1: Post deployment testing options: - Proposals - Proposal 1: Add Option 3 (R4-240495 – vivo) Option 3: Define the test to verify the performance validation together with model Proposal /update - validation scheme can be designed by RAN1 - Proposal 2: Add Option 4 (R4-2405737 – Nokia) Proposal 4: RAN4 to test the procedure when updated/new model/functionality stays inactive in the device before is has passed assessment/verification and can substitute currently active model/functionality. - Proposal 3: include some mandatory fallback (e.g. to a “baseline” model or older model already tested/validate) as an option - Proposal 4: As a further option relating to post deployment testing, consider the possibility of capturing model input during testing for later testing of new models: Capture model input during conformance testing for later testing of new models. (FFS the captured data needs to be held completely by the UE vendor) (Ericsson – R4-2405610) - Proposal 5: other RAN4 #111 (moderator summary and WF) Issue 1-1: Post deployment testing - Proposals - Proposal 1: Option 1 is feasible only if model updates are infrequent - Proposal 2: Option 2 is feasible only if the validation can be performed by UE vendors individually (no need to test in a certified lab) - FFS whether some validation information needs to be shared with gNB - Proposal 3: Add an option in which the conformance test inputs are stored and used for post-deployment testing/validation - Proposal 4: Drop Option 1, only discuss post deployment handling as part of LCM - Proposal 5: RAN4 should discuss a framework to ensure that at least a validated model exists at the UE - Proposal 6: RAN4 should discuss a framework to ensure that non-validated/non-tested models are not used by the UE - Proposal 7: Others - Agreement - Interested companies are invited to bring analysis on the feasibility of the options proposed so far. RAN4 #112 (moderator summary and WF) Issue 1-1: Post deployment testing - Proposals - Option 1: Amend the agreements above as below (R4-2413391): - Option 1: Conduct the conformance testing for new/updated AI model/functionality before its deployment in already deployed UEs (considering the UE hardware that it will be used in) - FFS on the feasibility - Option 2: Design the test to verify the performance monitoring and proactive recovery from potential performance degradation - Depend on the other WG progress - Monitoring can be used for managing fallback, model update/model switching/model transfer, if applicable - Other options are not precluded - Option 2: Add Option 3 as below (R4-2413391): - Option 3: Capture model input during conformance testing for later testing of new models. - Option 3: UE shall support at least some default/baseline AI/ML models/features that passed conformance requirements. The respective models can be used as a fallback mechanism. (R4-2411980, R4-2412021 & R4-2412331) - Option 4: Drop Option 1 as framework is not clear and burden on UE vendors is very high - Option 5: RAN4 to consider handling post-deployment verification of updated/new functionality in inactive/standby state/mode before it is allowed to be used along with the inference of the currently used functionality (R4-2412021) - Option 6: RAN4 to send an LS to RAN2 and RAN1 WGs on a need to consider conformance testability of AI/ML enabled functionalities when designing such LCM operations as monitoring/activation/update/fallback for the devices already deployed in the field. (R4-2412021) - Option 7: RAN4 to study and compare the feasibility of Options 1, 2, and 3. RAN4 can inform other groups about the conclusions of this study. (R4-2413391) - Option 8: Others

Managing the performance of AI/ML model (ML-enabled Functionality/Feature) updates or changes over time, particularly for UE-side models is one of the key challenges of AI/ML framework. Per study item agreements, AI/ML models may originate from a UE, UE-side OTT server and there are two general alternatives considered: 1) model updates or changes that are transparent to the network, and 2) model updates are non-transparent to the network. One of the specific characteristics of AI/ML air interface is that AI/ML models (features/functionality) can change over the lifetime of UE via (re)configurations, models updates (e.g. due to additional training) and other reasons. In practice, the changes in the AI/ML models used at the UE side may lead to alterations in the performance of the UE, which in turn can impact the overall network performance. Therefore, it is crucial to establish a robust framework for post-deployment model verification.

While regular conformance tests are expected to be performed for certain default AI/ML models, the existing 3GPP conformance framework may not be directly suitable for post-deployment verification. In particular, the fact that UE has passed the conformance tests, does not guarantee that the performance will be maintained in case of AI/ML model updates or changes over UE lifetime.

Considering the discussion in previous meetings we observe the following generic options on how to handle post-deployment AI/ML model performance verification, which are eventually not mutually exclusive.

Option 1: Conformance testing for AI model/functionality before deployment to the UE.

The first option is to test AI/ML model/feature updates against RAN4 requirements and declare the changes or updates to the AI/ML model by the device vendor before any deployment to the UE is performed. This comprehensive approach guarantees that models meet the minimum performance requirements. One of benefits of such approach is that the minimum performance is guaranteed, which may be helpful for the network as it may not need to extensively rely on the performance monitoring framework (e.g., not use frequent monitoring or use a subset of metrics depending on RAN1 design).

Under an assumption that the AI/ML model update comes from the UE-side OTT server it is reasonable to assume that device vendors have ability to perform conformance testing of respective models, before the model can be updated (i.e., the testing is considered as feasible). The conformance testing does not necessarily imply lab testing and a simplified procedure can be considered including UE vendor testing & declaration (no certification lab). The respective validation information can be shared with gNB.

Same time we acknowledge that this option may limit the flexibility of UE implementation and is not compatible with more adaptive approaches: 1) training/updates performed directly at the UE side, 2) use of localized AI/ML models optimized for specific conditions (i.e., models are not generalized for a wide set of conditions and may eventually have degraded performance under conformance test conditions).

Furthermore, we expect that RAN4 requirements may not cover all possible scenarios in terms of different parameters affecting model implementation including, bandwidth, sub-band size, number of MIMO layers, Tx antenna configurations. Therefore, conformance tests and requirements may not be always available. The terms of “test setup” and “minimum performance” requirement term may not be precisely characterized and may need to be further clarified. For instance, it is possible to leave the “minimum performance” up to vendor discretion under condition that the respective information is conveyed to the NW.

Observation #1: Conformance testing for AI model/functionality before deployment at the UE

- Can guarantee the minimum performance of models under various conditions.

- Applicable at least for the case when AI/ML model update comes from the UE-side OTT server, and for generalized AI-ML models with infrequent updates

- Not applicable for 1) training/updates performed directly at the UE side, 2) localized AI/ML models optimized for specific scenarios.

- Can be applicable to a sub-set of scenarios covered by RAN4 specifications, but cannot cover all possible deployment scenario. So, the “test conditions” and “minimum performance” requirements for scenarios not covered by RAN4 specifications need to be clarified.

Option 2: Performance monitoring framework/requirements (incl. mandatory fallback and performance validation)

The second option is to not explicitly test the changes or updates to the models during UE lifetime (post-deployment), and instead, the network and/or UE would rely on performance monitoring mechanisms. We agree that the AI/ML performance monitoring framework is an integral part of the whole AI/ML feature, and it is important to define respective requirements to guarantee the control of performance. However, it is currently unclear whether the performance monitoring mechanism can provide reliable performance all the time and impose limited overhead at the same time. Also, it is yet unclear whether such test cases can fully replace conformance testing.

Observation #2: The details of performance monitoring framework are not yet defined, and it is unclear whether the mechanism can guarantee minimum performance in the field.

In previous meetings it was also proposed to include specific procedure for performance validation during model transfer/update, which we assume can be considered as part general performance monitoring framework. If such approach is defined, then the performance validation can be performed in a dynamic manner and the procedure of testing UE performance can be effectively built-in into the AI/ML framework. The details of such approach candidate approach are unclear and need to be further discussed.

Observation #3: The specific procedure for performance validation during model transfer/update can be further discussed with the goal to ensure minimum performance.

Similarly, there were proposals to require UE to have a mandatory support of model(s) fulfilling conformance requirements, which can be used as a fallback subject to decisions made via performance monitoring framework (“procedure when updated/new model/functionality stays inactive in the device before is has passed assessment/verification and can substitute currently active model/functionality”). The respective approach can be considered as a complimentary method to ensure that UE can guarantee proper performance. Same time, the respective fallback model may not be applicable to all deployment scenarios.

Observation #4: The support of mandatory support of model(s) fulfilling conformance requirements which can be used as a fallback as a part of performance monitoring framework may be considered as a complimentary method to ensure that UE can guarantee proper performance. However, the respective model may cover only a limited set of deployment conditions and not applicable under certain scenarios.

Conclusions

In our view neither approach is flexible enough, and the eventual design should take the pros/cons of both into account as follows:

- Support both approaches including conformance testing and performance monitoring. Further discuss conditions/scenario when each approach is applied.

- Conformance testing (i.e., vendor testing)

  - Changes or updates to the AI/ML models/features can be tested by the device vendors against requirements before the deployment to the UE is performed.

  - The conformance testing is performed by UE vendors and applicable in case of UE-side OTT server side model updates.

  - For scenarios covered by RAN4 specifications, the minimum requirements and testing conditions are based on RAN4 specifications. For scenarios not covered by RAN4 specifications, the minimum requirements and testing conditions need to be clarified.

  - The information on whether AI/ML model/feature update has passed conformance test (and potentially associated data) shall be conveyed to the network, and based on this, the network may adjust the model monitoring framework accordingly.

- Performance monitoring framework

  - The performance monitoring framework should be capable to ensure the AI-ML performance under some typical conditions. For instance, the model is expected to at least outperform non-AI-ML algorithms under same conditions (if applicable)

  - Given that details of performance monitoring are not yet defined, it is recommended to communicate with RAN1/2 to check whether the monitoring framework can guarantee specific minimum performance validation, and that performance is not degraded comparing to non AI-ML case (at least for the cases where it is possible to clearly identify legacy UE behavior).

- Fallback models

  - UE shall support at least some default/baseline AI/ML models/features that pass conformance requirements defined in RAN4 specifications.

  - The respective models can be used as a fallback and can substitute currently active models/features at least for certain deployment conditions (the conditions depend on the level of generalization of defined models).

  - Recommend to check with RAN1/2 whether it is feasible that UE supports at least some default/baseline AI/ML models/features that passed conformance requirements, and the respective models can be used as a fallback mechanism for certain scenarios

Proposal #1:	Coordinate with RAN1/2 on the following questions related to post-deployment model performance monitoring and LCM procedures:

  - Recommend RAN1/2 to ensure that the performance monitoring framework for AI/ML models deployed at the UE can be used to compare the performance of AI-ML model and non-AI-ML performance for use cases where legacy behavior is defined

  - Check whether it is feasible that UE supports at least some default/baseline AI/ML models/features that passed conformance requirements, and the respective models can be used as a fallback mechanism for certain scenarios

Proposal #2:	RAN4 to confirm that changes or updates to the AI/ML models/features can be tested by the device vendors against requirements before the deployment to the UE is performed

  - The conditions when testing can be performed are FFS

  - The information on whether AI/ML model/feature update has passed conformance test (and potentially associated data) shall be conveyed to the network.

## Reference AI/ML models for performance requirements

Due to a diverse nature of AI/ML models implementations, there may be a significant variation in their complexity and performance, which is illustrated by RAN1 studies [2]. From RAN4 requirements perspective it is important to establish a common baseline for performance and complexity (for both 1-sided and 2-side models) and a big divergence in models’ complexity/performance may become a problem. To overcome the challenge, it is recommended to define reference AI/ML models for minimum performance requirements definition. Many existing demodulation requirements are defined under an assumption of a certain reference receiver (e.g., MMSE, R-ML, etc.) and a similar concept can be taken for 1-sided and 2-side AI/ML models. The models, whether partially or fully defined, would be utilized during the performance requirements definition stage to achieve a certain minimum level of performance under an assumption of a specific complexity level. The reference model concept is not necessarily meant to be standardized in detail, but rather should serve as a general guideline. Given the complexity of discussions on reference models and assumptions, it is recommended that these discussions be initiated early in the WI process.

In previous meetings RAN4 discussed feasibility of testing 2-sided AI/ML models for CSI compression and several candidate options on how to test encoder/decoder implementation at the test equipment side were discussed. During the RAN4 #110 meeting it was decided to narrow the testing options for two-sided AI/ML models with the focus shifted towards defining a common reference decoder for 2-sided AI/ML models - Option 3 with fully standardized reference decoder and Option 4 with partially standardized reference decoder (dataset + key parameters). If either of respective approaches is identified to be feasible, a similar method can be applied to define reference models for requirements definition.

Proposal #3:	Define reference AI/ML models for one-sided and two-sided models (both encoder/decoder) for performance requirements definition taking into account performance/complexity tradeoffs for all use cases.

# Conclusion

In this paper we provide our views on general aspects of AI/ML testability and interoperability aspects, and, in summary, make the following proposals:

Proposal #1:	Coordinate with RAN1/2 on the following questions related to post-deployment model performance monitoring and LCM procedures:

  - Recommend RAN1/2 to ensure that the performance monitoring framework for AI/ML models deployed at the UE can be used to compare the performance of AI-ML model and non-AI-ML performance for use cases where legacy behavior is defined

  - Check whether it is feasible that UE supports at least some default/baseline AI/ML models/features that passed conformance requirements, and the respective models can be used as a fallback mechanism for certain scenarios

Proposal #2:	RAN4 to confirm that changes or updates to the AI/ML models/features can be tested by the device vendors against requirements before the deployment to the UE is performed

  - The conditions when testing can be performed are FFS

  - The information on whether AI/ML model/feature update has passed conformance test (and potentially associated data) shall be conveyed to the network.

Proposal #3:	Define reference AI/ML models for one-sided and two-sided models (both encoder/decoder) for performance requirements definition taking into account performance/complexity tradeoffs for all use cases.

# References

- RP-221348 SID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #96, June 2022

- 3GPP TR 38.843 Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR air interface

- RP-234039 WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, December 2023

- R4-2403712 WF on AI/ML, Qualcomm, RAN4 #110, February 2024

- R4-2406617 WF on AI/ML, Qualcomm, RAN4 #110bis, April 2024

- R4-2401570 WF on AI/ML, Qualcomm, RAN4 #111, May 2024

- R4-2414323 WF on AI/ML, Qualcomm, RAN4 #112, August 2024