# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415692

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.23.4.2

Source: 	Samsung

Title: 	RRM impact for Rel-19 LP-WUS/WUR

Document for:	Discussion

- Introduction

During RAN#105 meeting, a revised WID[1] was approved on low-power wake-up signal and receiver for NR (LP-WUS/WUR). In RAN4#112 meeting, RAN4 achieved some agreements in WF[3].

In this contribution, our views on LP-WUS/WUR and related analysis of RRM requirements are provided.

- Discussion

  - General aspect

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Agreement: - Support case #3 - For serving cell measurement, further discuss: - Option 1: Further discuss whether to combine the measurements across the two radios, i.e., MR and WUR - Option 2: From RAN4 requirement perspective, not consider combining the measurements across the two radios, i.e., MR and WUR - Option 3: It is up to UE implementation whether to combine the measurement across the two radios. - Other options are not precluded. RRM measurement case index; MR serving cell measurement; MR neighboring cell measurement; LR measurement. #3 Relaxed case b; On with relaxation measurement; On with relaxation measurement; ON. - Note: In RAN4 understanding, the discussion of the related measurement criteria (i.e., whether to use the legacy or new criteria) is out of RAN4 responsibility.

In last RAN4 meeting, RAN4 agreed at least case 1 (full offloading case) and case 3 (relaxed case b) need to considered for specify MR RRM offloading/relaxation requirements.

Several aspects need to be considered together including LR measurement performance, MR serving cell measurement relaxation, MR neighbouring cell measurement relaxation and corresponding side condition. RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package. Several states may be considered for MR RRM relaxation and LP-WUR RRM measurement which as summarized in Table 1 as example for further discussion (RRM measurement with high priority can be considered separately):

Table 1 States of MR RRM relaxation and LP-WUR RRM measurement

| RRM measurement state index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement | Applicable scenario and side condition |
| --- | --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off | ON | - MR enters into ultra-deep sleep state w/o paging monitoring and RRM measurement - LP-WUR enable for LP-WUS monitoring and serving cell RRM measurement. - LP-WUR can maintain good measurement performance and UE placed in good condition i.e., cell centre |
| #2 Relaxed case a | On with relaxation factor >8 | Off | ON | - MR still needs to perform RRM measurement with longer period - LP-WUS monitoring and partial RRM measurement rely on LP-WUR. LP-WUR can maintain good measurement performance with assistant measurement from MR. - UE placed in good condition w/o triggering neighbouring cell measurement |
| #3 Relaxed case b | On with relaxation factor >8 | On with relaxation factor >3 | On | - MR still needs to perform RRM measurement on serving cell and neighboring cell - LP-WUR can monitor LP-WUS, and maintain good measurement performance with assistant measurement from MR - UE with good condition e.g., fulfil existing Relaxed measurement criterion including low-mobility and not-at-cell edge |
| # Fall back to normal mode | On | On | Off | - MR follows normal RRM measurement - Exit from LP-WUS RRM measurement - Relationship with LP-WUS monitoring exit/entrance condition |

Case #3 already covered case 2 implicitly e.g.  under case 3 UE only enable neighbouring cell measurement when serving cell quality lower than certain level as existing thresholds e.g., S criteria.

Observation 1: Case 2(relaxed case b) was covered by case 3 implicitly with existing neighbouring cell measurement criteria.

Another issue was for serving cell measurement under case 3, both measurement from MR and LR existed and then how to handle above two measurements from different radios. In our view, it’s subject to UE implementation to decide and from RAN4 requirement perspective, we can specify requirements without combination from two radios.

Proposal 1: Under case 3 for measurement on serving cell, combine option 2 and option 3 i.e. From RAN4 requirement perspective, the baseline assumption not consider combining the measurements across the two radios between MR and WUR meanwhile requirements shall not make restriction of UE implementation.

Issue 1-1-4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

For MR in RRC_IDLE/INACTIVE state, MR is required to wake-up to perform serving cell measurement at least per one or two DRX cycles by current requirements. There is observation [2] that power saving gain can be achieved, if sufficient MR RRM measurement relaxation in time domain is applied e.g., relaxation factor larger than 8. According to the observation, OOK-based LP-WUS is more robust to frequency error than OFDM-based signal.

For MR RRM measurement relaxation and offloading to LR, at least following requirements need to be specified:

- LR measurement period and accuracy

- MR measurement relaxation

- Entry/exit criteria evaluation period for MR RRM relaxation (case 1, case 3)

Proposal 2: RAN4 specify following requirements for MR RRM relaxation and offloading:

- LR measurement period and accuracy

- MR measurement relaxation and the evaluation period for entry/exit criteria threshold evaluation under different states (case 1 fully offloading, and case 3 MR relaxation)

  - Existing requirements for S criteria or relaxed measurement criterion can be considered as starting point

Issue 1-1-5/1-1-6-/1-1-: Criteria (entry/exit conditions) for LP-WUR serving cell measurement/ MR measurement relaxation and LP-WUS paging monitoring

Regarding the criteria for LP-WUR measurement and MR RRM measurement relaxation, we think RAN4 needs to discuss the criteria related to measurement. Although RAN2 is the main group for design of criteria, traditionally RAN4 was involved in the designing criteria related to measurement. Otherwise, RAN2 may not find technical guidance for determining the criterion. Therefore, we think that RAN4 shall lead the discussion regarding criterion or scenario (entry/exit condition) for measurement.

Proposal 3: RAN4 shall be involved on criterion and scenario (entry/exit condition) design for LP-WUR measurement and MR RRM measurement relaxation together with  RAN1 and RAN2.

- RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighboring cell measurement and LP-WUR measurement as a package

Following agreements were reached in RAN2 on the exit/entrance criteria

##### RAN2#127: Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ. ##### RAN2#127: RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

With above agreements, we can summarize the status as following:

| Case | Entrance | Exit | Metric |
| --- | --- | --- | --- |
| LP-WUS monitoring | MR threshold and LR threshold | LR threshold | (LP-)RSRP and optional (LP-)RSRQ. |
| RRM measurement offloading (no serving cell measurement) | MR threshold LR threshold (can be considered) | LR threshold | (LP-RSRP) |

We can observe that:

Observation 2: For entrance condition of LP-WUS monitoring, both MR based and LR based threshold can be configured, and exit condition is based on LR based threshold.

Observation 3: For MR measurement offloading (case 1), entrance condition relies on MR measurement only or both MR measurement threshold and LR measurement threshold can be configured; and exit condition is LR based threshold.

For RRM relaxation case (Case 3), RAN2 has no conclusion yet, our understanding, both LR based threshold and MR based threshold can be configured.

RAN2 also agreed that separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Another issue needs to be further discussed on entry/exit condition is that depending on UE architecture, MR and LP-WUR measurement performance can be different i.e., antenna gain, IL and NF. Calibration between LP-WUR measurement and MR measurement required to have unified exit and entrance condition. In order to define such unified condition, it is worth noting that the RSRP measurement results difference between MR and LP-WUR should be stable and comparable. Based on UE RF discussion, both the antenna sharing architecture and sperate antenna between LP-WUR and MR are not precluded as candidate RF architectures.

Proposal 4: Calibration between MR measurement and LR measurement maybe required which also pending on the progress of UE RF session on LP-WUR RF architecture, NF assumption.

  - Sub-topic 1-2 Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Issue 1-2-2: Possible interruption related requirements for LP-WUR

Once LP-WUR detected LP-WUS indicating paging indication, then UE needs to wake up MR receiver and monitor paging on paging occasion.

The maximum interruption/processing time between UE being ready to receive Paging/PDCCH by MR and LP-WUS received “on” indication by LP-WUR consist of following factor (time offset between LP-WUS/paging indication to MR ready for Paging monitoring):

“LP-WUS detection processing time + MR wake-up time + MR sync time”

- MR wake-up time depending UE implementation can be 400 ms ~1s

- MR sync time pending on whether UE can utilize LR sync time/frequency information as starting point for MR i.e., if MR and LR sharing same of RF components e.g.  LO, and/or antenna then UE can utilize LR sync information for MR receiver:

  - For UE with shared architecture between LR and MR, LR sync info can be used

    - ~1~2 SSB required for MR sync time

  - Other wise

    - ~10# SSB required for MR sync time

RAN4 may need to specify the maximum interruption time between LP-WUS/LO to PO, which the UE expects to be ready for MR in Paging monitoring. And NW can configure PO offset to LO no less than above processing time/interruption time specified by RAN4.

Proposal 5: New requirements need to be specified for the processing/interruption time between LP-WUS received paging indication to MR ready for Paging monitoring.

Issue 1-2-4: LP-WUR operating carrier frequency

A LS from RAN1 (R4-2414909) was sent to RAN2/RAN4 on operating frequency assumption for LR:

In our understanding, the operation frequency for LR shall be same carrier frequency as serving cell of MR otherwise it’s not feasible to support MR measurement offloading to LR if operating frequency from MR and LR is different. On supporting bands on LR receiver, LP-WUS/WUR is an independent feature introduced under Rel-19. The supporting bands of LR receiver shall be subsets of the bands which supported by UE. The detailed capability of LP-WUS reception and granularity can be discussed further in later stage. With above understanding, MR measurement offloading shall be applicable only on the bands which LR supported.

Proposal 6: From Rel-19 RAN4 RRM requirement perspective, LR shall be operated as the same carrier frequency of serving cell for LP-WUS reception/paging monitoring and LR measurement.

Another issue related to frequency is FR2 handling, in previous RAN4 meeting following agreement reached:

RAN4 agreements - RAN4#110bis Issue 2-1-1: Operation bands for LP-WUR Agreement: - Focus on FR1 licensed bands - FR2 is not precluded - FFS on which licensed bands will be chosen as example bands for band specific requirements. - RAN4#112 Issue 1-1-6: Specify FR2 LP-WUS RF requirements Agreements: - Wait to RAN1 progress on FR2 LP-WUS work..

In last RAN-P, following agreement endorsed:

RAN#105 WF: It is proposed to endorse the following in RAN#105 for Rel-19 LP-WUS/WUR WI regarding FR2 support - Confirm in RAN#105 that Rel-19 LP-WUS work item scope includes both FR1 and FR2. - Request RAN1 to make a progress on the FFS “Whether the above is applicable to FR2” (under RAN1#117 agreement on LP-WUS and LP-SS bandwidth) in RAN1#118bis. - Reuse the FR1 design when possible - No additional guidance in RAN#105 for RAN4 work and TU allocation

With above agreements, FR1 and FR2 is the scope meanwhile FR2 will try to reuse FR1 design as much as possible from RAN1 perspective.

From RAN4 RF and RRM requirements perspective, supporting FR2 will double the effort in RAN4. To control RAN4 work load, we suggest to prioritize the discussion in FR1.

Proposal 7: From RAN4 requirement perspective, prioritize the work in FR1.

Issue 1-2-6: Number of Rx for LP-WUR requirements

According to UE RF session conclusion, 1Rx is baseline for LR. For LR based on RRM measurement, it’s reasonable to align with UE RF assumption.

Proposal 8: Align with UE RF conclusion, 1Rx as baseline for LR based on RRM measurement.

  - Sub-topic 1-3 MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

RAN2 is still on discussion for the neighbour cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

In R16, relaxation factor 8 was introduced in RAN4 for UE inter-frequency/intra-frequency measurement when UE meet ‘not at cell edge’ criteria and or ‘low mobility’.

According to LP-WUR SI discussion, LP-WUS can achieve power saving gain when relaxation factor larger than 8.

Proposal 9: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation.

- If not possible to down-selection as single value, multiple values can be considered with delta on threshold.

  - Sub-topic 1-4 LP-WUR CONNECTED mode

As indicated in the WI objective:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

During CONNECTED mode, UE RRM/RLM/BFD are performed by MR, these requirements are not applicable for LP_WUR during RRC CONNECTED mode. And timing requirements are also not applicable given LP-WUR don’t support UL transmission or CA/DC.

Observation 4: No impact is observed   for RRC_CONNECTED state mobility, timing, signalling characteristic, and measurement procedure requirements.

The details of LP-WUS procedure to trigger PDCCH monitoring are still on discussion in RAN1 with several alternatives. Also, the activation/deactivation of LP-WUS monitoring is still on discussion in RAN1 and RAN2.

Proposal 10: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress.

Paging reception

When LP-WUR is enabled, LP-WUR can monitor LP-WUS to wake up MR for triggering PDCCH/paging monitoring. RAN1/2 discussion is still on going. It’s FFS whether exiting RAN4 requirements for “Maximum interruption in paging reception” can be reused or not.

Proposal 11: Paging reception requirement impact can be discussed after further input from RAN2 and RAN1.

- Conclusion

In this contribution, our view on RRM requirement is provided.

General aspect

Observation 1: Case 2(relaxed case b) can be covered by case 3 implicitly with existing neighbouring cell measurement criteria.

Proposal 1: Under case 3 for measurement on serving cell, combine option 2 and option 3 i.e. From RAN4 requirement perspective, the baseline assumption not consider combining the measurements across the two radios between MR and WUR meanwhile requirements shall not make restriction of UE implementation.

Proposal 2: RAN4 specify following requirements for MR RRM relaxation and offloading:

- LR measurement period and accuracy

- MR measurement relaxation and the evaluation period for entry/exit criteria threshold evaluation under different states (case 1 fully offloading, and case 3 MR relaxation)

  - Existing requirements for S criteria or relaxed measurement criterion can be considered as starting point

Proposal 3: RAN4 shall be involved on criterion and scenario (entry/exit condition) design for LP-WUR measurement and MR RRM measurement relaxation together with  RAN1 and RAN2.

- RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighboring cell measurement and LP-WUR measurement as a package

Observation 2: For entrance condition of LP-WUS monitoring, both MR based and LR based threshold can be configured, and exit condition is based on LR based threshold.

Observation 3: For MR measurement offloading (case 1), entrance condition relies on MR measurement only or both MR threshold and LR threshold can be configured; and exit condition is LR based threshold.

Proposal 4: Calibration between MR measurement and LR measurement maybe required which also pending on the progress of UE RF session on LP-WUR RF architecture, NF assumption.

Sub-topic 1-2 LP-WUR requirements at RRC_IDLE/INACTIVE state

Proposal 5: New requirements need to be specified for the processing/interruption time between LP-WUS received paging indication to MR ready for Paging monitoring.

Proposal 6: From Rel-19 RAN4 RRM requirement perspective, LR shall be operated as the same carrier frequency of serving cell for LP-WUS reception/paging monitoring and LR measurement.

Proposal 7: From RAN4 requirement perspective, prioritize the work on FR1.

Proposal 8: Align with UE RF conclusion, 1Rx as baseline for LR based on RRM measurement.

Sub-topic 1-3 MR RRM relaxation

Proposal 9: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation.

- If not possible to down-selection as single value, multiple values can be considered with delta on threshold.

Sub-topic 1-4 LP-WUR CONNECTED mode

Observation 4: No impact is observed  for RRC_CONNECTED state mobility, timing, signalling characteristic, and measurement procedure requirements.

Proposal 10: FFS whether need to introduce LP-WUS monitoring activation and deactivation delay requirements pending on RAN2/RAN1 progress.

Proposal 11: Paging reception requirement impact can be discussed after further input from RAN2 and RAN1.

# References

RP-240801 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony

3GPP TR38.869: “Study on low-power wake up signal and receiver for NR”

R4-2413899	WF for RRM core requirements for LP-WUSWUR, vivo