3GPP TSG-RAN WG4 Meeting #112bis	R4-2415706

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	System parameters for LP-WUR

Agenda item:	6.23.2.1

Document for:	Discussion

Introduction

In this document we present our views on open issues regarding system parameters. Following agreements were made in the last RAN4 meeting [1].

Issue 1-1-1: New RAN4 TR to capture simulation and analysis of LP-WUS receiver and requirements Agreement: - RAN4 suggest having a new RAN4 TR to capture the simulations and analysis for LP-WUS RF. Issue 1-1-2: Performance metric for Rx RF requirements Agreement: - 1% MDR for evaluation [and core requirements] - FFS on the testability issue. Issue 1-1-3: Performance metric for Demodulation requirements Agreement: - [1% or 0.1%] FAR as assumption for MDR result calibration. Issue 1-1-6: Specify FR2 LP-WUS RF requirements Agreements: - Wait to RAN1 progress on FR2 LP-WUS work. ### 1.1.2 Sub-topic 1-2 System parameters Issue 1-2-1: Channel raster for LP-WUR Agreements: - RAN4 can further discuss and align understandings on whether new channel raster is needed for LP-WUS. - RAN1 and RAN2 status on how to indicate the centre operation frequency of LP-WUS will be helpful. Issue 1-2-3: number of RBs for LP-WUS with 15kHz SCS Agreements: - Leave RAN1 to make decision. - RAN4 can discuss assumption of RBs under 15kHz SCS for simulation Issue 1-2-5: Whether need to define CBW for LP-WUR? Agreements: - No need to define BW for LP-WUR

Discussion

Performance metric for RX requirements

In the last meeting, 1% MDR was agreed for simulation evaluation but not for deriving Rx requirements. Arguments for not using the 1% MDR for setting requirements were raised in suggesting difficulty in testing. In our opinion, requirements and evaluations are interlinked and should be done for the same MDR value. Conformance testing time depends on many other factors and is not a good argument for not using 1% MDR for setting the requirements.

During the study phase, companies had studied the performance of LP_WUS using link-level simulations with a metric of 1% MDR with FAR ranging from 0% to 1%. With these metrics, it was shown that the LP_WUS has power saving gain as high as 98%, subjected to effective UE paging rate [2]. As this feature is meant for saving UE power, it makes sense to set the MDR and FAR metric for requirement purposes as close to what values were used in study phase.

Use 1% MDR and 1% FAR values for defining the requirements.

Differentiation between LP-WUR receivers

There are two kinds of receivers under discussion, OOK based and OFDM based. It was already agreed that they can have different noise figure. Further, the SNR required to decode might be different for both receiver types. However, both receivers should meet the coverage requirements. Thus, it is possible that some RF requirements such as REFSENS is same for both receivers. However, the side conditions might differ for both of these receivers. Our suggestion is we analyze each RF requirement from both receiver types and differentiate between them only if there is a large gap between them.

Do not make a general agreement on if two sets or one set of requirements are defined.

Channel raster

UE will be provided both time and frequency domain resources configuration for LP-WUS and LP-SS. Whether that information comes as a broadcast message or via some other mechanism, that is up to RAN2 to decide. There can be multiple mechanisms via which frequency domain resource configuration may be configured such as, relative to first common RB in corresponding scsCarrier, based on PointA, based on relation to active BWP, to name a few. Irrespective of the option chosen by RAN1, our understanding is that it will be anyway configured on the common resource grid based on sub-carrier spacing applied for the LP-WUS.

UE will know the exact frequency location of LP-WUS and LP-SS in advance.

LP-WUS and LP-SS will be configured on common resource grid based on the SCS applied. Further, existing OFDM framework will be used to generate LP-WUS and LP-SS and they will be multiplexed with the existing NR transmission. There is no reason or requirement to not transmit these signals on the common resource grid. Based on this discussion, we can conclude that channel raster does not need to be defined for LP-WUS and LP-SS and the current NR channel raster can continue to be used.

No specification impact regarding channel raster.

Conclusion

The following Observations and Proposals were made:

Proposal 1: Use 1% MDR and 1% FAR values for defining the requirements.

Proposal 2: Do not make a general agreement on if two sets or one set of requirements are defined.

Observation 1: UE will know the exact frequency location of LP-WUS and LP-SS in advance.

Proposal 3: No specification impact regarding channel raster.

- References

- R4-2414309, WF on LP-WUS UE RF requirements, Vivo, RAN4#112, Maastricht, Netherlands, August 19th – August 23rd, 2024.

- 3GPP TR 38.869: “Study on low-power wake-up signal and receiver for NR”.