3GPP TSG-RAN WG4 Meeting #112bis	R4-2415707

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Fundamental RX requirements for LP-WUR

Agenda item:	6.23.2.2

Document for:	Discussion

Introduction

In this document we present our views on open issues regarding REFSENS, ACS, and ASCS requirements. Following agreements were made in the last RAN4 meeting [1].

Sub-topic 2-1 SNR simulation and values Issue 2-1-1: Channel model to specify LP-WUS RF requirements Agreement: - Follow typical RF requirements approach, AWGN should be selected. Issue 2-1-2: Target SNR simulation condition Agreement: - Follow similar approach of MR NR, no repetition should be used when simulate target SNR. Issue 2-1-3: Whether RAN4 should conclude a target SNR first Agreement: - Target SNR is the basis for many Rx requirements discussion. Group should conclude SNR definition and SNR values first. Issue 2-1-4: Target SNR value Agreements: - RAN4 further discuss target SNR value for LP-WUS - FFS single SNR for OOK-based and OFDM-based receiver Sub-topic 2-2 NF and REFSENS requirements Issue 2-2-2: Baseline architecture for OOK-based LP-WUS Agreement: - use zero-IF receiver as a baseline architecture for envelop based LP_WUR Issue 2-2-3: Baseline architecture for OFDM-based LP-WUS Agreements: - Wait for RAN1’s conclusion on the OFDM-based LP-WUR. Issue 2-2-4: whether different NF for OOK-based and OFDM-based Agreement: - Differentiate NF for OOK-based and OFDM-based receiver. Issue 2-2-9: MDR value for REFSENS test case Agreements: - FFS the MDR for conformance testing should be aligned with the value for RF requirement. Sub-topic 2-3 ASCS simulation and requirements Issue 2-3-1: ASCS requirements value Agreements: - RAN4 further discuss how to specify ASCS requirement based on simulation outcome. Issue 2-3-2: Required number of guard RB for ASCS Agreements: - RAN4 further discuss the required number of guard RBs for ASCS requirement based on simulation outcome. Issue 2-3-3: Test parameters for ASCS Agreements: - RAN4 further discuss the test parameters for ASCS requirement. Sub-topic 2-4 ACS simulation and requirements Issue 2-4-2: ACS requirements value Agreements: - RAN4 further discuss how to specify ACS requirements based on simulation outcome. - FFS same as MR, or further relaxed Issue 2-4-3: Required guard RB for ACS requirements Agreements: - RAN4 further discuss the number of guard RBs for ACS requirement based on simulation outcome. Issue 2-4-4: Test parameters for LP-WUR ACS case Agreements: - RAN4 further discuss the test parameters for ACS requirement.

Discussion

SNR definition

It has been raised during the past discussions that SNR definition has to be specified in the context of LP-WUS in order to match the simulated results. SNR can be defined in either time domain or in frequency domain. In our simulations we have used the frequency domain definition and is defined as the ratio of average energy of the LP-WUS signal over the average energy of the noise. The averaging is performed over the allocated LP-WUS sub carriers.

SNR is defined as the ratio of average energy of the LP-WUS signal per sub carrier over the average energy of Noise over the same sub carriers.

NF value

A zero-IF receiver was agreed to be a baseline for the OOK based LR. This architecture can have a high degree re-use of the NR radio like frontend, antennas, LNA, mixer, ADC (if it is configurable in frequency and sample depth) can be reused.

Figure 1 Basic UE LP-WUR architecture for utilizing a Zero-IF receiver.

Assumed values of the performance parameters of different components used in this architecture are shown in Table 1. These values are taken from published literature ([2]--[8]) and correspond to frequencies less than equal to 2.4GHz. As the direct conversion receiver folds the image spectrum, an image reject mixer is used. However, based on the frequency domain content of the signal, an acceptable solution can be designed without an image reject mixer. This however will work only for an envelope detector based receiver. An OFDM based detector will need an image reject mixer. Thus, further optimization can be done at the mixer. However, the overall NF level will only increase by around 0.1 dB. The NF of the whole receiver is being governed by the front-end filters and the LNA, which will be common for both OFDM based, and envelope based receivers.

Table 1 Gain, and NF values for a reference Zero-IF based receiver for a sequence detector LP_WUR.

| Component | Gain (dB) | NF (dB) | Remarks |
| --- | --- | --- | --- |
| RF BPF/Duplex filter | -1.5 | 1.5 |  |
| LNA | 15 | 7 | Low power LNA with large NF. |
| Mixer | 3 | 10 |  |
| BB amplifier | 40 | 25 | BB amplifier providing the majority of required gain. |
| Total | 56.5 | 11.6 |  |

Zero-IF architecture supports a high degree of reuse of the NR main radio components.

To support more than one band, the receiver could use a wideband LNA or multiple LNAs supporting smaller frequency area.

As the bandwidth of the WUS signal is expected to be scaled according to the sub carrier spacing the LP filter will most likely be required to have different cut off frequencies, e.g., one configuration for each sub-band spacing configuration.

The NF of the receive chain is being governed by front-end components and LNA, which are common for both OFDM based, and envelope based receivers.

Agree to use zero-IF receiver as a baseline RF architecture for OFDM based LP-WUR.

Agree to use the estimated NF of 12dB as a baseline for both envelope and OFDM based LP_WUR for low and mid bands.

In the last meeting, arguments were raised regarding using different NF for different bands. It is true that NF performance degrades with increasing frequency, so a receiver operating at higher center frequency will have worse NF compared to LP-WUR operating on low and mid-bands. This phenomenon can be accommodated by adding a delta for bands with frequency greater than 2.4 GHz on top of the above agreed NF number.

Agree to increase the NF used for REFSENS calculation by a fixed delta value for bands with frequency greater than 2.4 GHz.

Implementation margin

For E-UTRA and NR in FR1, 2.5dB of IM had been used for the main receiver in case of 5 MHz channel bandwidth. However, for a LP-WUR there is no interference from its own transmitter as the transmitter should not be working because UE is in deep or ultra-deep sleep. Further, there is only a single receiver. Thus, IM can be significantly tightened to a value less than 2.5dB.

Agree to have implementation margin to be 1dB.

ASCS

Link level simulations were performed with both AWGN and TDL-C 300 channel. Simulated performance for OOK4 M=4 modulation in AWGN channel is shown below. A flat PSD is used i.e., LP-WUS and NR signals have the same PSD. Rest of the results for the AWGN channel can be seen from the Annex A of this document. Simulations were performed with zero guard RB and no ADC quantization. Different curves in a single figure corresponds to different band filter order (bfo). Major observations from the results are following:

Figure 1 ASCS performance with envelope detector with zero guard RB for OOK4 M=4.Different curves correspond to different band filter order (bfo).

The SNR required for achieving a MDR of 1% does not improve much after third order filter.

Frequency offset has no significant impact on the envelope detector performance.

A SNR value of -2.5dB is sufficient to achieve a MDR of 1% in case of an envelope detector based LP-WUR.

Agree to have zero guard RB for the ASCS case.

To understand the impact of NR signal on LP-WUR performance, LLS were performed with the REFSENS test setup i.e., with no additional NR interference signal. The results from the LLS are shown below in Figure 2. Comparing results from Figure 1 and Figure 2, there is no noticeable performance degradation in the LP-WUS decoding performance because of the NR signal, provided the PSD of NR signal and LP-WUS is same. This implies that the REFSENS test is sufficient and ASCS test will not provide any additional information.

Agree to have no separate ASCS requirement.

Agree to have no separate ASCS test case.

Figure 2 LLS performance in REFSENS test environment i.e., no NR interferer.

ACS requirement value

ACS requirement value is linked with ACS guard RBs and there is a trade-off between the requirement and guard RBs. Further, the limited dynamic range of the LP-WUR will have an impact on the detection of LP-WUS in presence of an interferer. Given the LP-WUR is going to face the similar interferer as the main receiver, the requirement and side conditions for the ACS test case can be the same as of the normal UE.

Test parameters defined in Table 7.5-3, 7.5-4, 7.5-5, and 7.5-6 of TS 38.101-1 apply for LP-WUR ACS test case.

In test case where Pinterferer depends on REFSENS, LP-WUR REFSENS should be used.

ACS guard RBs including ADC quantization

In Appendix C is ACS simulation results presented with ADC resolution of 4 bit, 8 bit and 16 bit resolution. The simulations are based on best “fit” resolution where the scaling is done before quantization. This gives a better performance than in real implementation as in real implementation will the ADC resolution be used for both AGC scaling and quantization of the signal. As the AGC settings typical will be based on averaged values and not sample by sample accuracy the resulting performance is expected to be worse than the simulation.

Simulation of ADC quantization is often sample based and not based on average signal level and thereby the result for low resolution ADC’s are better than real implementation.

Methods for AGC handling for both LP-SS and LP-WUS should be discussed.

From the simulation results it can be seen that 4-bit resolution gives app. 1 dB worse performance than 8 bit and 16 bit resolution.

For further discussion assume more than 4 bit ADC quantization is used to be able to have AGC estimation, acceptable LP-RSRP and LP-RSRQ level reporting and acceptable receiver performance.

Conclusion

The following Observations and Proposals were made:

Proposal 1: SNR is defined as the ratio of average energy of the LP-WUS signal per sub carrier over the average energy of Noise over the same sub carriers.

Observation 1: Zero-IF architecture supports a high degree of reuse of the NR main radio components.

Observation 2: To support more than one band, the receiver could use a wideband LNA or multiple LNAs supporting smaller frequency area.

Observation 3: As the bandwidth of the WUS signal is expected to be scaled according to the sub carrier spacing the LP filter will most likely be required to have different cut off frequencies, e.g., one configuration for each sub-band spacing configuration.

Observation 4: The NF of the receive chain is being governed by front-end components and LNA, which are common for both OFDM based, and envelope based receivers.

Proposal 2: Agree to use zero-IF receiver as a baseline RF architecture for OFDM based LP-WUR.

Proposal 3: Agree to use the estimated NF of 12dB as a baseline for both envelope and OFDM based LP_WUR for low and mid bands.

Proposal 4: Agree to increase the NF used for REFSENS calculation by a fixed delta value for bands with frequency greater than 2.4 GHz.

Proposal 5: Agree to have implementation margin to be 1dB.

Observation 5: The SNR required for achieving a MDR of 1% does not improve much after third order filter.

Observation 6: Frequency offset has no significant impact on the envelope detector performance.

Observation 7: A SNR value of -2.5dB is sufficient to achieve a MDR of 1% in case of an envelope detector based LP-WUR.

Proposal 6: Agree to have zero guard RB for the ASCS case.

Proposal 7: Agree to have no separate ASCS requirement.

Proposal 8: Agree to have no separate ASCS test case.

Proposal 9: Test parameters defined in Table 7.5-3, 7.5-4, 7.5-5, and 7.5-6 of TS 38.101-1 apply for LP-WUR ACS test case.

Proposal 10: In test case where Pinterferer depends on REFSENS, LP-WUR REFSENS should be used.

Observation 8: Simulation of ADC quantization is often sample based and not based on average signal level and thereby the result for low resolution ADC’s are better than real implementation.

Proposal 11: Methods for AGC handling for both LP-SS and LP-WUS should be discussed.

Proposal 12: For further discussion assume more than 4 bit ADC quantization is used to be able to have AGC estimation, acceptable LP-RSRP and LP-RSRQ level reporting and acceptable receiver performance.

- References

- R4-2414309, WF on LP-WUS UE RF requirements, Vivo, RAN4#112, Maastricht, Netherlands, August 19th – August 23rd, 2024.

- Pletcher, N.M., Gambini, S. and Rabaey, J., 2008. A 52uW Wake-Up Receiver with 72 dBm Sensitivity Using an Uncertain-IF Architecture. IEEE Journal of solid-state circuits, 44(1), pp.269-280

- Cook, B.W., Berny, A.D., Molnar, A., Lanzisera, S. and Pister, K.S., 2006, February. An ultra-low power 2.4 GHz RF transceiver for wireless sensor networks in 0.13um CMOS with 400mV supply and an integrated passive Rx front-end. In 2006 IEEE International Solid State Circuits Conference-Digest of Technical Papers (pp. 1460-1469).

- E. -H. V. Yeh, A. -H. Lo, W. -S. Chen, T. -J. Yeh and M. Chen, "A 16 nm FinFET 0.4 V inductor-less cellular receiver front-end with 10 mW ultra-low power and 0.31 mm2 ultra-small area for 5G system in sub-6 GHz band," 2017 Symposium on VLSI Circuits, Kyoto, Japan, 2017, pp. C72-C73.

- Du, Y., Hu, W., Li, G.F. and Liu, S., 2018. Low power hybrid PG_Filter-AGC analog baseband for wireless receivers. IEICE Electronics Express, 15(3), pp.20171197-20171197.

- P. -H. P. Wang et al., "A 400 MHz 4.5 nW −63.8 dBm sensitivity wake-up receiver employing an active pseudo-balun envelope detector," ESSCIRC 2017 - 43rd IEEE European Solid State Circuits Conference, Leuven, Belgium, 2017, pp. 35-38..

- N. M. Pletcher, S. Gambini and J. Rabaey, "A 52 μ W Wake-Up Receiver With − 72 dBm Sensitivity Using an Uncertain-IF Architecture," in IEEE Journal of Solid-State Circuits, vol. 44, no. 1, pp. 269-280, Jan. 2009.

- H. Bialek et al., "A Passive Wideband Noise-Canceling Mixer-First Architecture With Shared Antenna Interface for Interferer-Tolerant Wake-Up Receivers and Low-Noise Primary Receivers," in IEEE Journal of Solid-State Circuits, vol. 57, no. 9, pp. 2611-2625, Sept. 2022.

- Annex A: ASCS Results

Simulated results with zero guard RB for selected few cases are collated below. The detector used is envelope based. TDL-C results followed the same trend as for AWGN channel and are shown after AWGN channel results.

- Annex B: ACS Results

Simulated results with different guard RBs for selected few cases are collated below. The detector used is envelope based. The are the results with no quantization i.e., impact of limited number of ADC bits hasn’t been included here. See section Anex C for ADC impact of the ACS performance.

- Annex C: ACS Results with different ADC resolutions

Simulated results with different guard RBs for selected few cases are collated below. The detector used is envelope based. The results are with quantization of the signal i.e., impact of limited number of ADC is included here.