3GPP TSG-RAN WG4 Meeting #112bis	R4-2415709

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	Testability aspects of LP-WUR

Agenda item:	6.23.2.4

Document for:	Discussion

Introduction

In this document we present our views on the testability aspects of LP-WUR. Following agreements were made in the last RAN4 meeting [1].

Issue 4-1-1: General framework on LP-WUS testing Agreements: - RAN4 further discuss the high-level approach on how to achieve LP-WUS performance verification.

Discussion

Test procedure

The test procedure must make sure that it is testing only the LP-WUR performance and not the combined performance of MR and LP-WUR. This might require using high SINR for the MR whenever there is a communication between TE and MR. However, designing detailed test cases comes under RAN5 responsibility.

RAN5 defines the detailed test procedure for the conformance tests.

A higher SNR for the main radio signals can be used, so that just the performance of the LP-WUR is tested.

Test mode

As the LP-WUR is going to be a receiver which has no direct external interface to the test equipment (TE) as it can only talk with TE via the main radio (MR) unless there is a test mode to enable LP-WUR to enable such functionality. A test mode will help in simplifying the test procedure and will facilitate in faster testing. However, how to design the test mode and how that will work in IDLE/INACTIVE and CONNECTED mode can be left for RAN5 to define with RAN4 input.

RAN5 can design the details of Test mode required to test the LP_WUR with RAN4 input.

Testing time

To achieve a ≤1% miss detection rate (MDR) of LP-WUS in a statistically confident way will take long time. The total time taken to have a statistically significant value depends on multiple factors such as test case execution time, TE setup and response time, UE feedback time, repetition cycle, wait times between repetition, etc. As the motivation to have a higher MDR (≥1%) is to save on conformance test time, it is best for RAN5 to decide the confidence interval for the requirements in order to test them in a reasonable amount of time.

Only motivation to have a higher (≥1%) MDR is to save conformance test time.

Lot of factors impacting the time taken for test case execution are under the scope of RAN5 work.

RAN5 can decide the confidence level to be used for testing the LP-WUR requirements in a reasonable amount of time.

Conclusion

In this paper following observations and proposals were made:

Proposal 1: RAN5 defines the detailed test procedure for the conformance tests.

Proposal 2: A higher SNR for the main radio signals can be used, so that just the performance of the LP-WUR is tested.

Proposal 3: RAN5 can design the details of Test mode required to test the LP_WUR with RAN4 input.

Observation 1: Only motivation to have a higher (≥1%) MDR is to save conformance test time.

Observation 2: Lot of factors impacting the time taken for test case execution are under the scope of RAN5 work.

Proposal 4: RAN5 can decide the confidence level to be used for testing the LP-WUR requirements in a reasonable amount of time.

- References

- R4-2414309, WF on LP-WUS UE RF requirements, Vivo, RAN4#112, Maastricht, Netherlands, August 19th – August 23rd, 2024.