3GPP TSG-RAN WG4 Meeting #112bis	R4-2415710

Hefei, China, Oct 14 – Oct 18, 2024

Source:	Nokia

Title:	RedCap NTN UEs

Agenda item:	6.26.2.1

Document for:	Discussion

Introduction

In this paper, we provide our views on the support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands. RedCap specific objectives mentioned in the WID [1] are copied below for ease of reading.

Objectives:

- Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] - Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

The following agreements were made in the last RAN4#111 meeting [2]

## Issue 1-2: HD-FDD refsens for 2 Rx Agreement: As baseline - n256: 0.5 dB tightening ## Issue 1-4: Simultaneous operation with GNSS Agreement: Further study NR NTN UE simultaneous operation with GNSS considering e.g. - Operating bands with UL close to GNSS frequencies (n254, n255) - Emissions on GNSS frequencies - Whether transmission gaps enabling successful GNSS reception are present - Potential impact on field performance - Potential need for specification updates, e.g. frequency error ## Issue 1-5: Specification updates Agreement: - Include information in specification suffix clause on suffix applicability on FR1-NTN, FR2-NTN or both, with RedCap being applicable to FR1-NTN - Use suffix I for RedCap, if accepted by MCC - Add a clause on max output power for RedCap indicating RedCap supports only PC3, similar to TN spec 38.101-1 clause 6.2I.1 ## Issue 2-1: UL capacity enhancements Agreement: - Continue evaluation on potential UE RF specification impact - As starting point, no UE RF impact at least from inter-symbol OCC scheme

Discussion

HD-FDD REFSENS for two RX

In the last meeting, 0.5 dB tightening was agreed for n256, however, the tightening amount for n254 and n255 are still under discussion. Companies have proposed to use 0.5dB for n254 and 0dB for n255 based on the tightening rules that were used for TN RedCap HD-FDD REFSENS [3]. Table 1 illustrates the rules for reference.

Table 1 Proposed REFSENS tightening rules in TN.

| FD-FDD 5MHz REFSENS | HD-FDD REFSENS Tightening | Bands |
| --- | --- | --- |
| ≤ -100 dBm | 0 dB | n1, n18, n24, n70 |
| > -100 dBm and ≤ -99 dBm | 0.5 dB | n30, n65, n66, n74 |
| > -99 dBm | 0.8 dB | n2, n3, n5, n7, n8, n12, n13, n14, n20, n25, n26, n28, n71, n85 |

The reasoning for 0.5dB goes back to almost 10 years old limited survey on insertion loss difference between a SAW RF filter vs SAW duplex filter. As SAW RF filters have less insertion loss compared to SAW duplex filters and since HD-FDD implementations do not need duplex filters, REFSENS can be tightened by 0.5 dB. Further, the reasoning for 0dB was based on REFSENS of TDD bands.

Reasoning for 0.5dB improvement goes back to almost 10 years old survey on insertion loss differences.

Reasoning for 0dB tightening was related to REFSESN of TDD bands.

We did a limited survey of duplexers, RF filters, and TRX switches for n255 and n254, and noticed that the insertion loss difference ranges from 1.2dB to 2dB. This observation supports the 0.8dB number that was proposed under more recent discussions done under Release 17 RedCap for HD-FDD case. Given that lot of improvement in device technology has happened over last 10 years, it is possible to leverage that improvement for improving the REFSENS.

Recent survey on RF components showed that insertion loss difference ranges from 1.2dB to 2dB.

Further, there are no TDD NTN bands, so utilizing TDD bands as an excuse for inaction hinders overall system efficiency and improvements.

There are no TDD bands in NTN, so sticking to 0dB tightening is unjustified.

Utilize 0.8dB of tightening for both n255 and n254.

Simultaneous operation with GNSS

During last meeting, concerns were raised regarding simultaneous operation of NTN RedCap devices with GNSS given the overlap of n254 UL and n255 DL with several of GNSS frequencies.  While RAN4 worked on introducing support for NTN band n255, potential issues with co-existence with GNSS were raised and agreement was reached by RAN4 that protection of GNSS shall be ensured given it is a managed spectrum by the license holder.

- Further, regarding issues with n254 UL, it was a problem with normal NTN UEs too and mechanisms were discussed under the WI In-Device Co-existence (IDC) enhancements for NR and MR-DC [4].

RAN4 have agreement that protection of GNSS shall be ensured.

UL in n254 was discussed for NTN UEs and was tackled under IDC co-existence WI.

Further, the WID clearly specifies simultaneous GNSS and NR-NTN operation by NTN RedCap and eRedCap UEs, so our understanding is that no special handling has to be done in the specifications and UE implementation can deal with it.

No impact to RAN4 regarding simultaneous operation of RedCap/eRedCap NTN UEs with GNSS.

PC3 power level

In RAN4#112, it was proposed to increase the tolerance level of PC3 for NTN RedCap UE which only supports HD-FDD operation from +2 to +2.8 dB. The reasoning for this was related to hardware implementation. In our opinion, although it is beneficial to have higher output power, doing so in the specification by changing the nominal value for PC3 is not a good way. A better way would be via introduction of a new power class, for which work should be defined in the WID.

Changing PC3 level for HD-FDD in NTN context is not a good approach.

PC3 power level should be the same in NTN and TN context.

Conclusion

The following Observations and Proposals were made in this paper:

Observation 1: Reasoning for 0.5dB improvement goes back to almost 10 years old survey on insertion loss differences.

Observation 2: Reasoning for 0dB tightening was related to REFSESN of TDD bands.

Observation 3: Recent survey on RF components showed that insertion loss difference ranges from 1.2dB to 2dB.

Observation 4: There are no TDD bands in NTN, so sticking to 0dB tightening is unjustified.

Proposal 1: Utilize 0.8dB of tightening for both n255 and n254.

Further, regarding issues with n254 UL, it was a problem with normal NTN UEs too and mechanisms were discussed under the WI In-Device Co-existence (IDC) enhancements for NR and MR-DC [4].

Observation 5: RAN4 have agreement that protection of GNSS shall be ensured.

Observation 6: UL in n254 was discussed for NTN UEs and was tackled under IDC co-existence WI.

Proposal 2: No impact to RAN4 regarding simultaneous operation of RedCap/eRedCap NTN UEs with GNSS.

Observation 7: Changing PC3 level for HD-FDD in NTN context is not a good approach.

Proposal 3: PC3 power level should be the same in NTN and TN context.

- References

- RP-240775, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#103, Maastricht, Netherlands, 18 – 22 March 2023.

- R4-2413527, Way Forward for [112][311] NR_NTN_Ph3_UE_RF, Qualcomm Inc, RAN4#112, Maastricht, Netherlands, 19 – 23 August 2024.

- R4-2200442, RedCap UE HD-FDD REFSENS requirements, Apple, RAN4#101bis-e, Online, January 17 – 25 2022.

- RP-221281: New WI: In-Device Co-existence (IDC) enhancements for NR and MR-DC