3GPP TSG-RAN WG4 Meeting #112-Bis	R4-2415724

Hefei, China, Oct. 2024

Source	:	Qualcomm Incorporated, Sony

Title	:	FR1 3Tx power scaling and FR2 asymmetric connection scenario

Agenda item	:	6.19.2

Document for	:	Approval

# 1.	Introduction

- Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2

  - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP.

  - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources

The revised Rel-19 WID on NR MIMO Phase [2] includes the following objective:

Previously, we identified a scenario that is consistent with the WID [3] and had the potential to improve UL performance significantly. In this contribution, we provide more detail on its feasibility, including a discussion on possible concerns.

# 2. 	FR2 enhancement discussion

## 2.1	Asymmetric scenario for FR2 in Rel-19

### 2.1.1	Background

Previously, we identified various asymmetric FR2 scenarios that warranted further investigation and then proposed a down selection to ‘scenario 2’ [3]. For the possible scenarios, two different TRP types are assumed:

- Two different TRP-types: an UL-specific TRP (type 2) and an UL+DL TRP (type 1, or legacy style).

- Two different UE types, based on whether the UE supports STxMP or not. STxMP refers to simultaneous transmission to two different TRPs (multi-TCI-state).

All the possible scenarios are presented in the table below, including the proposed down selection in green highlight.

| (FR2 asymmetric scenarios for Rel-19) |  | Tx capability of UL-specific TRP |  |
| --- | --- | --- | --- |
|  |  | Limited DL capability | No transmitter |
| UE UL TCI-state | STxMP Simultaneous transmission with multiple TCI states | ‘Scenario 1’ | ‘Scenario 3’ |
|  | Single UL TRP can be same or different from DL TRP | ‘Scenario 2’ | ‘Scenario 4’ |

UL + DL TRP, type 1

UL-specific TRP, type 2 (*)

DL

SSB + PDSCH

UL

(type2, *) – limited DL capability

SSB only DL

2.1.2	‘Scenario 2’ [3]

In scenario 2, the TRP assigned to handle UL is type 2, and is different from the TRP for DL (type 1) with the intent of improving the UL budget. In more precise terms, the UL and DL are associated with different TCI states.

There are multiple motivations for this scenario:

- Legacy configured Tx power requirement applies, due to single TCI state transmission

- Can be used for MPE related direction re-evaluation by UE and network

- Compatibility with HetNet deployments

- Further evolution in beam management framework (i.e. lifting of constraints in beam management) can be absorbed in the future.

## 2.2	RACH procedure detail for ‘scenario 2’

One important detail is how a UE would connect to a network populated with both types of TRPs (‘type 1’ and ‘type 2’) and operate according to scenario 2 above. The RACH procedure primarily depends on what DL capabilities the type 2 TRP has in the network:

- the UL-specific (type2) TRP can support IA (initial access)

- the UL-specific (type2) TRP is limited to transmitting SSBs only.

The discussion below looks at both possibilities in more detail to identify the path to be taken by RAN4 to leverage the WI to improve FR2 UL. The overall goal for the UE is to successfully RACH onto both kinds of TRP and obtain separate TAs for each.

### 2.2.1	type 2 TRP can support IA (in addition to SSB transmissions)

In figure 2.2.1-1, we sketch out a RACH scheme assuming all TRP types in the network can support RACH. (Green boxes represent legacy operation). In this scheme, there is no need for a UE to be able to discriminate between the TRPs, so the first step for the UE is simply connecting to the network using legacy procedure. At this time the UE does not know the TRP type used for initial access (IA). Per legacy procedure, a valid TA is obtained through RAR. The UE continues legacy operation, specifically it continues to measure other SSBs and reports back their relative strengths. The next steps depend on the TRP type that the UE has attached to:

- If the IA TRP is type 2, the network would now have to ensure that a type 1 is connected to secure downlink. If an appropriate type 1 candidate is found among the reported SSBs (the network would know by id), the network can initiate a contention-free random access (CFRA) attachment, and the UE can obtain a second TA for the second connection. If not, the network would drop the call to remain consistent with legacy operation which requires connection to a type 1 TRP.

- If the IA TRP is type 1, there is no novelty yet, and the situation is still consistent with Rel-15 behavior. Now the network opportunistically tries to locate a type 2 TRP among the UE’s SSB reports (the network would know by id) and determines whether a candidate is worthwhile from a link budget perspective. Note that even though the type 2 TRP could be physically closer to the UE, the RSRP can be lower, owing to lower DL EIRP from a type 2 TRP. If successful the network would have to initiate a CFRA to the type 2 TRP, and the UE can obtain a second TA for the second connection.

Observation 1: One possible RACH scheme is for the UL-specific TRP to support initial access in addition to SSBs, for the Rel-19 enhancement

Note the presumption that even the UL-specific TRP must support complex DL duties like RAR, PDCCH, data and some minimum configuration capability, even if only limited to IA. These amount to a full-feature DL capability.

Observation 2: For an UL-specific TRP to support IA, it needs a full featured DL

Unconnected UE measures SSBs

UE sends RACH MSG1in direction of strongest SSB

New network algo: can UE see type 2 TRPs?

DL + UL TRP (type 1)

UL-only TRP (type 2)

New network algo: What kind of TRP used for initial access?

NO

YES

Network requests CFRA to identified type 2 TRP

UE sends RACH, and second valid TA is obtained from RAR

IA TRP sends back RAR+ minimum config and UE is connected, with valid TA

UE measures neighbor cell SSBs and reports back

New network algo: can UE see type 1 TRPs?

Network requests CFRA to identified type 1 TRP

New network procedure for error case needed

NO

YES

UE disconnected

Continue legacy operation

Figure 2.2.1-1: Example RACH procedure to operate UE with different TCI states for UL and DL, assuming all TRPs support IA

UE sends RACH, and second valid TA is obtained from RAR

### 2.2.2	type 2 TRP cannot support IA

In figure 2.2.2-1, we sketch out a possible RACH scheme assuming type 2 TRPs in the network cannot support RACH and IA. (Green boxes represent legacy operation). These type 2 TRPs would send NCD SSBs (non-cell-defining SSBs, a Rel-17 feature). In this scheme, the key detail is that the UE can discriminate between the TRP types (based on SSB type) and chooses a legacy TRP type (type 1) for initial access. Per legacy procedure, a valid TA is obtained through RAR.

The UE continues legacy operation, specifically it continues to measure other SSBs and reports back their relative strengths. Now the network opportunistically tries to locate a type 2 TRP among the UE’s NCD SSB reports and determines whether a candidate is worthwhile from a link budget perspective. If successful the network would have to initiate a CFRA to the identified type 2 TRP, and the RAR and subsequent communication can be provided to the UE through the type 1 TRP that the UE originally connects to. This arrangement is possible because the UE monitors RAR based on the same search space (DL TRP) irrespective of whether CFRA is toward DL TRP (for first TA) or UL TRP (for second TA).

Observation 3: Another possible RACH scheme is for the UL-specific TRP (type 2) to only transmit NCD SSBs, for the Rel-19 enhancement

UE ignores NCD SSBs for IA

UE sends RACH to type 1 TRP with strongest SSB

New network algo: Did UE report any NCD SSBs?

NO

type 1 TRP sends back RAR and UE is connected, with valid TA

UE measures neighbor cell SSBs and reports back

UE initiates RACH towards type 2 TRP per CFRA

UE receives subsequent RACH process communication from original type 1 TRP

UE is connected, with second valid TA

YES

Figure 2.2.2-1: RACH procedure to operate UE with different TCI states for UL and DL, assuming type 2 TRPs cannot support IA

Unconnected UE measures SSBs

Network requests CFRA to type 2 TRP with best NCD SSB

Continue legacy operation

## 2.3	Concerns with operations with different TCI states for UL and DL

### 2.3.1	‘Why are 2 TAs required for single UL?’

The feature is built around the scenario of ‘UL in a different direction than DL’. This would imply just one TA is necessary for the UE. In practical networks, it is helpful to retain limited UL capability at the UE in the direction of the DL, even if not simultaneous with the ‘regular’ UL. This secondary capability is useful ACK/NACKs, as well as DL channel sounding by the UE.

Observation 4: The proposed scenario (different TCI states for UL and DL) benefits UEs with limited UL capability towards the DL TRP. The different TCI states are expected to have different physical locations, consequently, 2 TA capability is required.

### 2.3.2	gNB timing error

gNB timing error is a well understood phenomenon. Timing mismatch between the UL TRP and DL TRP can cause elevated probability of UL/DL collision at the UE. The annex has some example timing diagrams to aid in visualizing the problem. There are other challenging possibilities like Tx to Tx collisions, for example when DL channel sounding SRS (associated with the DL TCI state) is followed by UL SRS or PUSCH (associated with the UL TCI state). All these cases represent legacy 2TA challenges however, and no new challenge is introduced by the Rel-19 proposal. By precedent then, it can be left to the network implementation to provision enough time at the UE for switching for this proposed feature also.

Observation 5: Inter-TRP timing error is a legacy 2TA timing challenge for the network, and no novel RAN1 design is required for the network to provision sufficient switching time at the UE.

## 2.4	The FR2 proposal for this WI

In this contribution we identified the potential benefits of the proposed scenario, we have elaborated on possible RACH procedures for the proposed scenario, as well as discussed and addressed concerns ranging from timing to release applicability. It is therefore possible to confirm that the previous proposal [3] is indeed feasible and potentially beneficial to networks owing to better UL conditions.

Proposal 1: RAN4 to adopt the scenario of different TCI states for UL and DL as the basis for an FR2 UE RF requirement for the Rel-19 asymmetric connection objective.

From observations 1-3, it is evident that both RACH schemes discussed in the previous section are feasible, but it is self-defeating for an UL-specific TRP to need a full-fledged DL capability comparable to a type 1 TRP.  From a network point of view, this is simply a densification argument. This makes the scheme of 2.2.1 (UL TRP supports IA) not viable as an enhancement, see observation 3.

Proposal 2: For the FR2 UE RF requirement for the Rel-19 asymmetric connection objective, UL-specific TRPs are assumed to have limited DL capability, and that capability is limited to SSB transmissions.

An example requirement could be UE spherical coverage with a different TCI state for UL compared to DL. Further RAN4 discussion will help determine if necessary to specify other requirements unique to this configuration.

# 3. 	FR1- Establishing UL specifications for 3 Tx

The 3Tx WI was started in RAN1#116 and to date the following 1-layer, 2-layer and 3-layer TPMIs have been defined.

In the WF [4] generated in RAN4#112 companies were asked to comment on the establishing power class specifications and PA configurations for 3 Tx UL MIMO:

It has been stated in the WI [1] that UL full power transmission mode1 and mode2 will not be supported. Therefore, only UL full power mode 0 will be supported in 3Tx. Furthermore, it should be noted that the above TPMIs will support both Rel-15 power scaling as well. For power scaling the factor 13        will be used while for full power mode the scaling factor will be set to 1.0. Also, it has been stated in the WI [1] that these 3Tx TPMIs will support only non-coherent operation.

Observation 6: UL full power transmission mode1 and mode 2 are not supported in 3 Tx

Observation 7: UL full power transmission mode 0 and Rel-15 power scaling are supported in 3 Tx

To enable the 3Tx feature to be used in a broad range of applications it should be capable of supporting PC3, PC2 and PC1.5. An inspection of the above TPMIs reveal that it is not possible to support PC1.5 in mode 0 operation because such a device would have to support the same power class for all ranks and currently the highest power achievable from a single PA based on current technology is presumed to be PC2. However, a UE supporting Rel-15 power scaling will be able to support all power classes as it can scale its output power with rank. The below table illustrates the output power as a function of the rank and declared UL power modes.

| UE UL power mode | mode 0 | mode 0 | Rel-15 pwr scaling | Rel-15 pwr scaling | Rel-15 pwr scaling |
| --- | --- | --- | --- | --- | --- |
| UE power class | 3 | 2 | 3 | 2 | 1.5 |
| Rank 1 Pout (dBm) | 23 | 26 | 18.3 | 21.3 | 24.3 |
| Rank 2 Pout (dBm) | 23 | 26 | 21.3 | 24.3 | 27.3 |
| Rank 3 Pout (dBm) | 23 | 26 | 23 | 26 | 29 |

Table 1: UE output power as a function of rank for different UL power modes

Currently, the UE declares power class on a per band basis. If 3 Tx is to exploit both UL full power mode 0 and Rel-15 power scaling, then a new power class per band per declared UL power mode would have to be defined.

Proposal 3: Define new UE power class signalling capability based on power class per band per declared UL power mode.

Also, MPR tables should be defined for 3Tx operation for handheld UEs as well as CPE/FWA devices using either measurements or simulations.

Proposal 4: Create MPR tables for 3Tx for both UE handheld and CPE/FWA devices. Use 10 dB antenna-antenna isolation for UE handhelds and 20 dB antenna-antenna isolation for CPE/FWA devices.

We are of the opinion that enabling 2-layer operation would give greater flexibility to the 3 Tx feature which should be exploited to enable this feature to reach its full potential.

Proposal 5: Enable the use of 2-layer TPMIs for the 3 Tx feature

# 4.	Conclusions

FR2:

Observation 1: One possible RACH scheme is for the UL-specific TRP to support initial access in addition to SSBs, for the Rel-19 enhancement

Observation 2: For an UL-specific TRP to support IA, it needs a full featured DL

Observation 3: Another possible RACH scheme is for the UL-specific TRP (type 2) to only transmit NCD SSBs, for the Rel-19 enhancement

Observation 4: The proposed scenario (different TCI states for UL and DL) benefits UEs with limited UL capability towards the DL TRP. The different TCI states are expected to have different physical locations, consequently, 2 TA capability is required.

Observation 5: Inter-TRP timing error is a legacy 2TA timing challenge for the network, and no novel RAN1 design is required for the network to provision sufficient switching time at the UE.

Proposal 1: RAN4 to adopt the scenario of different TCI states for UL and DL as the basis for an FR2 UE RF requirement for the Rel-19 asymmetric connection objective.

Proposal 2: For the FR2 UE RF requirement for the Rel-19 asymmetric connection objective, UL-specific TRPs are assumed to have limited DL capability, and that capability is limited to SSB transmissions.

FR1:

Observation 6: UL full power transmission mode1 and mode 2 are not supported in 3 Tx

Observation 7: UL full power transmission mode 0 and Rel-15 power scaling are supported in 3 Tx

Proposal 3: Define new UE power class signalling capability based on power class per band per declared UL power mode.

Proposal 4: Create MPR tables for 3Tx for both UE handheld and CPE/FWA devices. Use 10 dB antenna-antenna isolation for UE handhelds and 20 dB antenna-antenna isolation for CPE/FWA devices.

Proposal 5: Enable the use of 2-layer TPMIs for the 3 Tx feature

# 5.	References

- RP-234007, New WID: NR MIMO Phase 5, Samsung.

- RP-242394, ‘WID revision: NR MIMO Phase 5’, Samsung, RAN#105, Melbourne, AUS

- R4-2313224, ‘On asymmetric connection scenarios for NR MIMO Phase 5’, Qualcomm, RAN4#112, Maastricht, NL

- R4-2414310, ‘WF on UE RF requirements for Rel-19 MIMO enhancement, RAN4#112’, Samsung,, RAN4#112, Maastricht, NL

# 6.	Annex (gNB timing error)

## 6.1	Baseline case, connection to single TRP

The baseline case is the legacy single TCI state, single TA case.

At the outset, it is reasonable to assume that the TA associated with the UL TRP is lower than that for the DL TRP. The reasoning is that an UL TRP is chosen by the network for link budget reasons, which typically means proximity, and therefore lower TA. The figures below explore some typical scenarios. Figure 6.1-1 shows the legacy (baseline) timing, symbol types are (D, X, U). The special slot content is randomly chosen as1D-1X-12U, but the parameters are not specific to this illustration.

The quantity TRxTx_UE is the time available for the UE to make the Rx to Tx transition and it is up to the network to provision enough Nsw switching symbols to satisfy the UE’s requirement of 7 us. Similarly, the quantity TTxRx_UE is the time available for the UE to make the Tx to Rx transition and is typically not a challenge.

0

1

2

3

4

11

12

13

TRP1 Timeline

TAoffset

Slot

0

1

2

3

4

11

12

13

UE Timeline with legacy connection to TRP1

Tprop

Tprop

TA+TAoffset

TA = Tprop

TAoffset

Figure 6.1-1: Baseline timing diagram for connection with single gNB

TRxTx_UE = Nsw*Tsym - (2* TA + TAoffset2)

TTxRx_UE = TA + TAoffset

## 6.2	UL TRP clock running ahead of the DL TRP clock

Figure 6.2-1 shows a scenario where the UL TRP clock is running ahead of the DL TRP clock when the baseline case is extended to the 2TA case. When the UL TRP runs ahead, the critical parameter is the quantity TRxTx_UE, and it can become smaller than the 7 us allowance for UE switching depending on relative timing error ‘e’. The burden on the network is to now account for the worst-case timing error in its implementation in determining number of switching symbols. Note that the UE typically gains some time for the opposite direction, i.e. the Tx to Rx transition

0

1

UL TRP2 Timeline

TRP1 slot

0

1

2

3

4

11

12

13

UE Timeline with 2TA

TA1 = Tprop1

TA2 = Tprop2

Figure 6.2-1: Timing diagram with UL TRP2 running ahead of DL TRP1

DL TRP1 Timeline

TAoffset2

2

3

4

11

12

13

TRP2 slot

TRP Timing error = e

TRP Timing error = e

e + TA1

TA2 + TAoffset2

TRxTx_UE = Nsw*Tsym - (e +TA1 + TA2 + TAoffset2)

## 6.3	UL TRP clock running behind the DL TRP clock

Figure 5.3-1 shows a scenario where the UL TRP clock is running behind the DL TRP clock when the baseline case is extended to the 2TA case. When the UL TRP runs behind, the critical parameter is the quantity TTxRx_UE. It can become smaller than the 7 us allowance for UE switching depending on relative timing error ‘e’. The burden on the network is to now account for the worst-case timing error in its implementation in not scheduling DL in the first symbol after the switch. Note that the UE typically gains some time for the opposite direction, i.e. the Rx to Tx transition

=

0

1

UL TRP2 Timeline

TRP1 slot

0

1

2

3

4

11

12

13

UE Timeline with 2TA

TA1 = Tprop1

TA2 = Tprop2

Figure 6.3-1: Timing diagram with UL TRP2 running behind DL TRP1

DL TRP1 Timeline

TAoffset2

2

3

4

11

12

13

TRP2 slot

TRP Timing error = e

TRP Timing error = e

e + TA1

TTxRx_UE = TA2 + TAoffset2- e

0