3GPP TSG-RAN WG4 Meeting #112-Bis	R4-2415726

Hefei, China, Oct. 2024

Source	:	Qualcomm Incorporated

Title	:	24 GHz EESS band phase 2 requirements and UE compliance

Agenda item	:	5.21.2

Document for	:	Approval

# 1.	Introduction

In WRC-19, resolution 750, progressively stronger emissions limits were recommended for UEs and gNBs to protect EESS services in the 23.6 – 24.0 GHz band. The next significant event pertaining to this protected band is the adoption of ‘phase 2’ limits, currently recommended for September 2027 but brought ahead by several regional regulators. In RAN#105, a new WID [1] was approved for this purpose. In this contribution, we break the problem down into its two branches: signaling and AMPR determination. For the latter, we build on prior work on phase 1 limits to derive expected back-off requirements for UEs for phase 2 limits.

# 2. 	Discussion

## 2.1	Need for new signaling

WRC-19, in resolution 750, identified the passive EESS band highlighted below for protection from spurious emissions from various technologies, along with the associated requirements (excerpted from Table 1-1 of resolution 750):

Figure 2.1-1: EESS emissions limits from WRC19

A footnote further recommends that these limits be tightened for devices ‘brought into use’ after Sept. 1, 2027, tentatively referred to as ‘phase 2’ here:

Figure 2.1-2: ‘Phase 2’ EESS recommendation from WRC19, post 2027

NS_203 was instituted for UE adherence to the pre-2027 limit (+1 dBm/200 MHz), and UEs in the field before the 2027 date are expected to be ‘grandfathered in’. Changing the definition of NS_203 is therefore not feasible, and the phase 2 limits are best handled with some new signaling tentatively [NS_205].

Observation 1: The EESS phase 2 emissions requirements require a bespoke solution, independent of NS_203

Proposal 1:  RAN4 to create a new NS to support UE compliance of EESS phase 2 requirements.

The emission limits for UEs over the protected EESS band (23.6  f  24.0, GHz) can summarized as follows:

| Spectrum emission limit (dBm) | Measurement BW (MHz) | Handle | Notes |
| --- | --- | --- | --- |
| +1 | 200 | NS_203 | Phase 1 |
| -5 | 200 | [NS_205] | Phase 2 |
| -8 | 200 | NS_201 | Reference for back off derivation |

Table 2.1-3: 3GPP ‘additional emissions requirements’ for the EESS band corresponding to WRC19 limits

As is evident, NS_203 involved a 9 dB relaxation from NS_201 limits, while [NS_205] represents a 3 dB relaxation.

Further discussion in this contribution assumes that a signaling solution exists and is represented by [NS_205]. It should also be noted that any time there is a new combination of region and applicability date of the phase 2 regulations, a new NS will need to be created.

## 2.2	Legacy procedure to derive NS_203 A-MPR

NS_203 backoff values for single CC UL and contiguous allocations in contiguous CA were derived from those of the now obsolete NS_201 by extrapolating for a 9 dB relaxation relative to the NS_201 emissions limit [2]. This procedure is reproduced below, and the reader is directed to [2] for further details. Note the factor of 1 for the rate of signal power reduction to reduction of first order products, and a factor 1/3 for rate of signal power reduction to reduction of third order products:

| PC1 | Back off requirements (dB) by spur type: |  | A-MPR assuming co-location of all types (dB) |
| --- | --- | --- | --- |
|  | First Order | Higher Order |  |
| NS_203 | 9-1x9=0 | < 5-1/3*9, i.e. < 2 | 3.0 (reducing to 0.0 with enough frequency offset) |
| CA_NS_203, contiguous allocations | 9-1x9=0 | < 9-1/3*9, i.e. < 6 | 6.5 (reducing to 0.0 with enough frequency offset) |

Table 2.2-1: PC1 NS_203 A-MPRs derivation

| PC2/3/4 | Back off requirements (dB) by spur type: |  | A-MPR assuming co-location of all types (dB) |
| --- | --- | --- | --- |
|  | First Order | Higher Order |  |
| NS_203 | 0 (NS_201 did not have any) | <1.5-1/3*9, i.e. = 0 (with margin) | 0.0 |
| CA_NS_203, contiguous allocations | 0 | <5-1/3*9, i.e. < 2 | 2.5 (reducing to 0.0 with enough frequency offset) |

Table 2.2-2: PC2/3/4/5/6/7 NS_203 A-MPRs derivation

## 2.3	Deriving backoff requirements for Phase 2 (‘[NS_205]’)

### 2.3.1	Contiguous RB allocations

The procedure used in [2] is now repeated for [NS_205] in tables 2.4.1-1 -2, with a 3 dB relaxation from NS_201, rather than 9 dB for NS_203.

| PC1, contiguous allocations | Back off requirements (dB) by spur type: |  | Summary back off requirement (dB) |
| --- | --- | --- | --- |
|  | First Order | Higher Order |  |
| [NS_205] | 9-1x3=6 | < 5-1/3*3, i.e. < 4 | 8.0 (reducing to 6.0 with enough frequency offset) |
| CA_[NS_205] | 9-1x3=6 | < 9-1/3*3, i.e. < 8 | 9.0 (reducing to 6.0 with enough frequency offset) |

Table 2.3.1-1: PC1 [NS_205], derivation of necessary backoff

| PC2/3/4, contiguous allocations | Back off requirements (dB) by spur type: |  | Summary back off requirement (dB) |
| --- | --- | --- | --- |
|  | First Order | Higher Order |  |
| [NS_205] | 0 (assume marginal) | <1.5 – 1/3*3, i.e. < 0.5 | 2.0 (reducing to 0.0 with enough frequency offset) |
| CA_[NS_205] | 0 (assume marginal) | <5-1/3*3, i.e. < 4 | 5.0 (reducing to 0.0 with enough frequency offset) |

Table 2.3.1-2: PC2/3/4/5/6/7 [NS_205], derivation of necessary backoff

Note that for PC1 with [NS_205] (-1 table above), no amount of frequency offset will restore the required back-off to 0, it stays floored at 6 dB. This is because first order spurs do not fall off with distance from the UL signal (a first order approximation). This observation is captured below:

Observation 2: PC1 UEs in n257/n261 will also need backoff due to first order spurs (despite the large frequency distance from the EESS protected band).

### 2.3.2	Non-contiguous RB allocations

NC RB allocation refers to the condition where the UL comprises multiple contiguous chunks of RBs separated in frequency. NC RB allocations primarily manifest with non-contiguous CA, but because the standard does not preclude NC RB allocations in contiguous CA, UEs need to check for this latter case as well and take appropriate back off for emissions compliance. Consequently, the standard must provide A-MPR for both cases, and they are treated together here.

In general, owing to some frequency distance between band edge and the protected band, some non-contiguous RB allocations can ‘concentrate’ more emissions in the protected band than contiguous CA. Consequently, larger back-offs are foreseen for emissions compliance.

At this time, it is useful to note that NS_203 was defined only for n258, which does not have NC CA configurations, so A-MPR was not defined for NC RB allocations. Unfortunately, as argued above, NC RB allocations are not precluded for contiguous CA, so NS_203 A_MPR must be revised also, albeit separately.

Observation 3: NS_203 A-MPR does not account separately for non-contiguous RBs in contiguous CA and will need to be revisited separately for completeness.

Due to wide frequency spread of PA output for the NC allocation case, we turned to measurements from commercially available hardware. Our method is outlined below:

- Employ ‘RAN4 calibration’ procedure of normalizing the UE’s TRP limit to the 0 dB MPR reference waveform, as outlined in 6.2.2 in TS38.101-2

- 2 NC RB allocation test condition: Since the measurement BW is large for [NS_205], power in signal is more important than PSD. In this context, only the relative powers in the two clusters are important, the RB lengths in each allocation less so. To capture the worst-case relative power condition (to maximize low side IMD product), our tests were conducted with a 2RB + 1RB CP-OFDM UL with equal PSD.

- Correction for higher number of RB allocations: As the number of contiguous RB allocations that comprise the NC RB allocation arrangement increases, the amount of power in IMD tones inside the protected band can increase from the 2-cluster baseline. Mathematical analysis shows an additional 2 dB of back off is required to cover these conditions.

For Steps 1 and 2 (i.e. limited to 2-allocation consideration), our observations from measurements are presented below:

| UE power class | TRP limit | Back-off needed for 2-allocation UL for [NS205] | Back-off needed for UL with more than 2 allocations for [NS205] |
| --- | --- | --- | --- |
| PC1 | 35 dBm | 10 dB | 12 dB |
| PC2/3/4/5/6/7 | 23 dBm | 6 dB | 8 dB |

Table 2.3.2-1: Backoffs required for phase 2 EESS compliance

A final consideration is first-order spurs. These are not caused by non-linearities in the PA, and so, remain unchanged whether generated by 1 RB allocation or multiple. The back-off requirements for first-order spurs are thus identical to those derived for contiguous RB allocations.

The data and arguments can be summarized as follows for non-contiguous RB allocations:

| PC1, NC allocations | Back off requirements (dB) by spur type: |  | Summary back off requirement (dB) |
| --- | --- | --- | --- |
|  | First Order (unchanged from contiguous case) | Higher Order |  |
| [NS_205] as well as CA_[NS_205] | 6 | 12 | 13.0 (reducing to 6.0 with enough frequency offset) |

Table 2.3.2-2: PC1 [NS_205], derivation of necessary backoff

| PC2/3/4, NC allocations | Back off requirements (dB) by spur type: |  | Summary back off requirement (dB) |
| --- | --- | --- | --- |
|  | First Order (unchanged from contiguous case) | Higher Order |  |
| [NS_205] as well as CA_[NS_205] | 0 | 8 | 8.0 (reducing to 0.0 with enough frequency offset) |

Table 2.3.2-3: PC2/3/4/5/6/7 [NS_205], derivation of necessary backoff

### 2.3.3	A-MPR proposal for Phase 2 (‘[NS_205]’)

#### 2.3.3.1	On frequency offset for A-MPR applicability

Note that some of the projected back-off requirements in the section above reference ‘enough frequency offset’. The legacy definition (i.e. as used in NS_201, then reused in NS_203) of ‘enough’ is the frequency distance between channel edge and the edge of the FR2 band. From the physical process however, a more accurate definition would depend on frequency distance from the edge of the protected band rather than the band edge. This distinction is now more important because n257 and n261 are projected to need A-MPR, and in these bands, there is a significant difference between frequency offset from protected band (relevant) and frequency offset from FR2 band (not relevant). Moreover, referencing the protected band edge rather than the channel edge allows A-MPR specification to be channel agnostic (i.e n257 does not need separate repetitive treatment from n258)

Proposal 2: For [NS_205], frequency distance to protected band edge (rather than frequency distance to band edge) is adopted as frequency offset criterion. This is because n257 and n261 are projected to need A-MPR allowance.

Note that the upper edge of the protected band is 24.0 GHz. In the standard, this number can be referenced from the section where the emissions limits would be specified, for example upper edge of protected band as specified in table 6.5.3.2.5-1.

#### 2.3.3.2	Spec. wording format

The proposals and observations are present in specification form below, and apply equally to n258, n257 and n261. The wording can be refined, if necessary, with the intention of turning it into a proposal when the details are stable. In the meantime, it is presented for scrutiny. Note that A-MPR allowances are bunched by the max. TRP allowed for the UE, so PC2/4/5/6/7 are expected to co-adopt the A-MPR agreed for PC3. In the treatment below, these power classes are left out for compactness.

Single CC:

#### 6.2.3.6 A_MPR for [NS_205] ##### 6.2.3.6.1 PC1 For a single contiguous allocation, the A-MPR is 8.0 dB if offset frequency < BWChannel, 6.0 dB otherwise. For non-contiguous allocations, the A-MPR is 13.0 dB if offset frequency < BWChannel, 6.0 dB otherwise; where frequency offset is the frequency distance between the lower channel edge and the upper edge of the protected band in table [6.5.3.2.6-1] ##### 6.2.3.6.3 PC3 For a single contiguous allocation, the A-MPR is 2.0 dB if offset frequency < BWChannel, 0 dB otherwise. For non-contiguous allocations, the A-MPR is 8.0 dB if offset frequency < BWChannel, 0 dB otherwise; where frequency offset is the frequency distance between the lower channel edge and the upper edge of the protected band in table [6.5.3.2.6-1]

CA:

#### 6.2A.3.6 A_MPR for [CA_NS_205] ##### 6.2A.3.6.1 PC1 For contiguous ULCA with a single contiguous allocation, the A-MPR is 9.0 dB if offset frequency < BWintraCA, 6.0 dB otherwise. For contiguous ULCA with non-contiguous allocations or NC UL CA, the A-MPR is 13.0 dB if offset frequency < BWintraCA, 6.0 dB otherwise; where: - frequency offset is the frequency distance between the lowest channel edge and the upper edge of the protected band in table [6.5A.3.2.6-1] - BWintraCA is BWChannel_CA in case of contiguous CA and frequency separation in case of UL NC CA ##### 6.2A.3.6.3 PC3 For contiguous ULCA with a single contiguous allocation, the A-MPR is 5.0 dB if offset frequency < BWintraCA, 0 dB otherwise. For contiguous ULCA with non-contiguous allocations or NC UL CA, the A-MPR is 8.0 dB if offset frequency < BWintraCA, 0 dB otherwise; where: - frequency offset is the frequency distance between the lowest channel edge and the upper edge of the protected band in table [6.5A.3.2.6-1] - BWintraCA is BWChannel_CA in case of contiguous CA and frequency separation in case of UL NC CA

# 3.	Conclusion

Observation 1: The EESS phase 2 emissions requirements require a bespoke solution, independent of NS_203

Proposal 1:  RAN4 to create a new NS to support UE compliance of EESS phase 2 requirements.

Observation 2: PC1 UEs in n257/n261 will also need backoff due to first order spurs (despite the large frequency distance from the EESS protected band).

Observation 3: NS_203 A-MPR does not account separately for non-contiguous RBs in contiguous CA and will need to be revisited separately for completeness

Proposal 2: For [NS_205], frequency distance to protected band edge (rather than frequency distance to band edge) is adopted as frequency offset criterion. This is because n257 and n261 are projected to need A-MPR allowance.

# 4.	References

[1] 	RP-242371, ‘New WID on mmWave UE spurious emission’, China Unicom et al, RAN#105, Melbourne, AUS, Sept. 2024

[2] 	R4-2000216, ‘Impact of WRC19 resolutions on FR2’, Qualcomm, RAN4#94-e, Feb. 2020