3GPP TSG-RAN WG4 Meeting #112bis	R4-2415729

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.8.2.2.2

Source:	MediaTek Inc.

Title:	Discussion on NR-NTN HPUE RX RSD requirements

Document for:	Discussion

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable PC2 high power UE (HPUE) with defining RF requirements targeting at support of NR and IoT NTN UEs for FR1 single carrier scenario.

In this contribution, our evaluation about PC2 HPUE RX reference sensitivity degradation (RSD) requirements for NR NTN is provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

Based on the WID’s objective, it indicates that it is necessary to evaluate whether RX RSD requirements would be needed when TX HPUE is enabled.  In RAN4#112 meeting, the agreed WF [3] provides the following works about RX RSD for NR-NTN HPUE operation. The contents are shown as follows.

Issue 4-2: RSD for NR-NTN - Agreement - Reference sensitivity degradation (RSD) requirements is needed for NTN HPUE. - Further discuss the following aspects: - Use similar way in TN spec by introducing a new table of RSD compared to the PC3 - Whether default Tx-Rx frequency separation is used for test point

Regarding NR-NTN RSD framework, it is necessary to check NR TN RSD framework first before deciding whether it would be possible to reuse NR TN RSD framework for NR NTN.

According to survey of PC2 UE RX RSD from FR1 TN bands, the RSD values in TS 38.101-1 [2] are provided in Tables below for reference.

Table 7.3.2-1c Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 | 0.8 | 0.9 | 1.1 | 1.2 | 1.3 | 2.7 | 2.8 | 3.5 |  |  |
| n3 | 0.5 | 0.5 | 0.5 | 0.5 | 0.6 | 0.8 | 1.1 | 1.5 | 2.3 | 2.8 |
| n8 | 0.5 | 0.7 | 0.8 | 2.3 | 2.8 | 3.2 | 3.1 |  |  |  |
| n14 | 0.6 | 0.8 |  |  |  |  |  |  |  |  |
| n25 | 0.8 | 0.8 | 0.9 | 1.1 | 1.3 | 2.7 | 2.8 | 3.5 | 3.7 |  |
| n66 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 | 0.5 | 0.9 | 0.9 | 2.2 | 2.42 2.53 | 2.52 2.43 | 2.92 3.13 |  |  |  |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.2-1d Reference Sensitivity Degradation from PC3 to PC2 for FDD bands for UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 | 0 | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n2 | 1.2 | 1.2 | 1.3 | 1.2 | 1.2 | 5.8 | 6.0 | 6.5 |  |  |
| n3 | 1.4 | 1.5 | 1.5 | 1.5 | 1.6 | 1.7 | 2.8 | 5 | 5.5 | 6.0 |
| n8 | 1.3 | 1.4 | 2.1 | 5.8 | 6.1 | 6.5 | 7.0 |  |  |  |
| n14 | 1.1 | 1.3 |  |  |  |  |  |  |  |  |
| n25 | 1.5 | 1.5 | 1.6 | 1.6 | 1.7 | 6.0 | 6.2 | 6.7 | 7.1 |  |
| n66 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |
| n71 | 1.1 | 1.1 | 1.7 | 5.5 | 5.92 6.93 | 6.22 7.23 | 6.52 7.33 |  |  |  |
| n70 | 0 | 0 | 0 | 0 | 0 |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in clause 6.2G.4 NOTE 2: Applies to UEs that support a maximum uplink BW of 20 MHz in this band. NOTE 3: Applies to UEs that support optional symmetric UL/DL for this BW. |  |  |  |  |  |  |  |  |  |  |

It is observed that RX RSD values are needed for some NR TN mid-bands when UE operates in PC2 TX. Therefore, it is expected that RX RSD may be also needed for NR NTN band.

Observation 1: RX RSD values are needed for some NR TN mid-bands when UE operates in PC2 TX. Therefore, it is expected that RX RSD is also needed for NR-NTN band.

Proposal 1: Regarding NR-NTN HPUE UE, to reuse TN RSD framework for NR-NTN RSD is feasible.

According to our evaluation, because of high TX-RX duplex spacing frequency, no RX RSD is needed for NR NTN bands n256 and n254 when UE enables PC2 TX. Regarding NR NTN band n255, the RX RSD is needed when UE enables PC2 TX.

Proposal 2: Regarding NR NTN UE operated in PC2, define the RX RSD values as shown in tables below for bands n256, n255 and n254.

Table 1 RSD from PC3 to PC2 for FDD bands for NR NTN UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |
| n255 | [0.4] | [0.4] | [0.5] | [0.5] |
| n254 | 0 | 0 | 0 | 0 |

Table 2 RSD from PC3 to PC2 for FDD bands for NR NTN UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |
| n255 | [0.8] | [0.8] | [0.9] | [0.9] |
| n254 | 0 | 0 | 0 | 0 |

Regarding whether default Tx-Rx frequency separation is used for RSD test point, we think this approach is straightforward and can be considered as starting point.

Proposal 3: At least consider using default Tx-Rx frequency separation for RSD test point as starting point.

3 Conclusion

In this paper, our views on RX RSD requirements for NR-NTN PC2 UE for bands n256, n255, and n254 are provided as follows:

Observation 1: RX RSD values are needed for some NR TN mid-bands when UE operates in PC2 TX. Therefore, it is expected that RX RSD is also needed for NR-NTN band.

Proposal 1: Regarding NR-NTN HPUE UE, to reuse TN RSD framework for NR-NTN RSD is feasible.

Proposal 2: Regarding NR NTN UE operated in PC2, define the RX RSD values as shown in tables below for bands n256, n255 and n254.

Table 1 RSD from PC3 to PC2 for FDD bands for NR NTN UE not supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |
| n255 | [0.4] | [0.4] | [0.5] | [0.5] |
| n254 | 0 | 0 | 0 | 0 |

Table 2 RSD from PC3 to PC2 for FDD bands for NR NTN UE supporting Tx Diversity

| Operating Band | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) |
| --- | --- | --- | --- | --- |
| n256 | 0 | 0 | 0 | 0 |
| n255 | [0.8] | [0.8] | [0.9] | [0.9] |
| n254 | 0 | 0 | 0 | 0 |

Proposal 3: At least consider using default Tx-Rx frequency separation for RSD test point as starting point.

Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- R4-2414452, “WF on UE RF requirements for NTN HPUE”, Samsung