3GPP TSG-RAN WG4 Meeting #112bis	R4-2415731

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.8.2.2.1

Source:	MediaTek Inc.

Title:	Discussion on NR-NTN HPUE TX RF requirements

Document for:	Discussion

- Introduction

In RAN#103 plenary meeting, the WID on enhanced requirements and test methodology for NR and IoT NTN was approved in [1]. One of the main objectives is to enable PC2 high power UE (HPUE) with defining RF requirements targeting at support of NR and IoT NTN UEs for FR1 single carrier scenario.

In this contribution, our views about PC2 HPUE TX RF requirements for NR-NTN UE are provided for discussion.

- Discussion

2.1		High power UE (HPUE) for NTN

Some of the main Rel-19 WID [1] objectives for enhanced requirements and test methodology for NR and IoT NTN are shown below for reference.

- Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-handheld UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary

In RAN4#112 meeting, the agreed WF [3] provides the following works for specifying TX requirements (e.g., ACLR, MPR, A-MPR, and SEM) for NR-NTN HPUE operation. The contents are shown as follows.

NTN HPUE UE RF – Tx requirements Issue 3-1-3: Consideration of power classes - Agreement - All power classes in WID, including Power class 1/1.5/2 for NR-NTN and Power class 1/2 for IoT-NTN, will be further discussed Issue 3-3-6: ACLR - Agreement: - The ACLR requirements will depend on co-existence study outcomes. Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified. Issue 3-3-5: SEM - Agreement: - For NR-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs - For IoT-NTN: further discuss whether to reuse existing SEM requirements from PC3 to other PCs

According to specification of HPUE UE ACLR for FR1 NR TN bands, the ACLR values in TS 38.101-1 [2] are provided in Table below for reference.

TS 38.101-1 Table 6.5.2.4.1-2: NR ACLR requirement

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR ACLR | 37 dB | 31 dB | 31 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

However, based on R4-2412463[4] and R4-2413352[5] NTN-TN coexistence evaluations, these scenario-4 evaluations show that NR-NTN ACLR values can be lower than 26dB for PC2 to PC1 HPUE due to sufficient ISD protection.

R4-2412463[4] contents below for reference: Table 2: ACIR requirements BS Antenna type; ACIR Requirement @ 5%-tile Throughput Loss Ratio. Satellite; GEO. Aggressor; NR NTN UL. Victim; TN UL (FDD). PC level; PC1 (31 dBm); PC1.5 (29 dBm); PC2 (26 dBm). Urban; Non-AAS; 20.822 dB; 18.822 dB; 15.704 dB. AAS; 25.349 dB; 23.349 dB; 19.978 dB. Observation 1: Based on the simulation results in Table 2: - No tighten ACIR requirement since NTN UL interference to TN UL with a required ACIR < 26 dB.

R4-2413352[5] contents below for reference: Scenario 4 – 23dBm vs 31dBm UE Tx power GEO Observation 1: For Scenario 4, throughput loss increases from 0.035% to 0.20% with 12 dB ACIR from 23 dBm to 31 dBm UE. Observation 2: For Scenario 4, throughput loss is 0 at 30 dB ACIR for 31 dBm UE. Proposal 3: Rel-19 NTN HPUE to reuse Rel-17 NTN UE ACLR for all power classes up to 31dBm (PC1).

Observation 1: Based on R4-2412463[4] and R4-2413352[5] NTN-TN coexistence evaluations, these scenario-4 evaluations show that NR-NTN ACLR values can be lower than 26dB for PC2 to PC1 HPUE due to sufficient ISD protection.

Proposal 1: Discuss whether to reuse NR-NTN PC3 ACLR value for PC2, PC1.5, and PC1 ACLR because NTN-TN coexistence evaluations [4-5] show NR-NTN ACLR values can be lower than 26dB for PC2 to PC1 HPUE.

Table: NR-NTN ACLR requirements

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR-NTN ACLR | 30 dB | 30 dB | 30 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

Regarding SEM for NR-NTN HPUE, we think that some SEM requirements could be reused for PC2 for narrow CBW. But if the existing SEM requirements are more stringent than the PC2, PC1.5 and PC1 ACLR requirements (e.g., ACLR of 30dB for PC2 to PC1), we suggest discussing whether SEM requirements would be relaxed.

Proposal 2: Regarding SEM for NTN HPUE, if no specific concern, consider relaxing SEM requirements when the existing SEM requirements are more stringent than the PC2, PC1.5 and PC1 ACLR requirements (e.g., ACLR of 30dB for PC2 to PC1).

The TX MPR is relevant to ACLR, SEM and modulation EVM requirements. If NR-NTN PC3 ALCR could be reused for HPUE power class and relaxing would be possbile, we think that reusing NR-NTN PC3 MPR values for PC2 MPR would be one way for consideration.

TS 38.101-5 6.2.2 UE maximum output power reduction UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 3, the allowed maximum power reduction (MPR) is defined as Table 6.2.2-1 in 3GPP TS 38.101-1 clause 6.2.2.

TS 38.101-1 Table 6.2.2-1 Maximum power reduction (MPR) for power class 3 Modulation; MPR (dB). Edge RB allocations; Outer RB allocations; Inner RB allocations. DFT-s-OFDM; Pi/2 BPSK; ≤ 3.51; ≤ 1.21; ≤ 0.21. ≤ 0.52,3; ≤ 0.52; 02,4. Pi/2 BPSK w Pi/2 BPSK DMRS; ≤ 0.52,3; 02; 02,4. QPSK; ≤ 1; 05. 16 QAM; ≤ 2; ≤ 1. 64 QAM; ≤ 2.5. 256 QAM; ≤ 4.5. CP-OFDM; QPSK; ≤ 3; ≤ 1.5. 16 QAM; ≤ 3; ≤ 2. 64 QAM; ≤ 3.5. 256 QAM; ≤ 6.5. TS 38.101-1 6.2G.2 UE maximum output power reduction for Tx Diversity For UE supporting Tx diversity, the allowed MPR for the maximum output power is specified in Table 6.2.2-1 for UE power class 3, in Table 6.2D.2-1 for UE power class 2, in Table 6.2D.2-2 and Table 6.2D.2-3 for UE power class 1.5 with dual TX, in Table 6.2D.2-4 and 6.2D.2-5 for UE power class 1.5 with 4 Tx. For UE power class 1.5 with dual Tx, the allowed maximum power reduction (MPR) defined in Table 6.2D.2-3 is in accordance with the indicated modifiedMPR-Behavior specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The maximum output power is defined as the sum of the maximum output power at each UE antenna connector. If a UE that supports PC1.5 has to apply the requirements of PC2 according to the rules in clause 6.2.1, the MPR requirements in Table 6.2.2-2 apply

Proposal 3: If NR-NTN PC3 ACLR value could be reused for NR-NTN HPUE ACLR and relaxing SEM would be possible under HPUE operation, discuss whether reusing NR-NTN PC3 MPR values (both single Tx and Tx diversity) for PC2 MPR would be one possible way.

3 Conclusion

In this paper, our views about HPUE TX RF requirements for NR-NTN UE are provided as follows:

Observation 1: Based on R4-2412463[4] and R4-2413352[5] NTN-TN coexistence evaluations, these scenario-4 evaluations show that NR-NTN ACLR values can be lower than 26dB for PC2 to PC1 HPUE due to sufficient ISD protection.

Proposal 1: Discuss whether to reuse NR-NTN PC3 ACLR value for PC2, PC1.5, and PC1 ACLR because NTN-TN coexistence evaluations [4-5] show NR-NTN ACLR values can be lower than 26dB for PC2 to PC1 HPUE.

Table: NR-NTN ACLR requirements

|  | Power class 1 | Power class 1.5 | Power class 2 | Power class 3 |
| --- | --- | --- | --- | --- |
| NR-NTN ACLR | 30 dB | 30 dB | 30 dB | 30 dB |
| NOTE 1: Void |  |  |  |  |

Proposal 2: Regarding SEM for NTN HPUE, if no specific concern, consider relaxing SEM requirements when the existing SEM requirements are more stringent than the PC2, PC1.5 and PC1 ACLR requirements (e.g., ACLR of 30dB for PC2 to PC1).

Proposal 3: If NR-NTN PC3 ACLR value could be reused for NR-NTN HPUE ACLR and relaxing SEM would be possible under HPUE operation, discuss whether reusing NR-NTN PC3 MPR values (both single Tx and Tx diversity) for PC2 MPR would be one possible way.

Reference

- RP-240857, “New WID: Enhanced requirements and test methodology for NR and IoT NTN”, Huawei

- TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- R4-2414452, “WF on UE RF requirements for NTN HPUE”, Samsung

- R4-2412463, “Simulation for NTN co-existence study”, MediaTek Inc.

- R4-2413352, “Discussion on FR1-NTN High Power UE (HPUE)”, THALES

- TS 38.101-5, “NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”