# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415733

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.15.2.2

Source:	China Telecom

Title:	Discussion on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

- Introduction

In RAN4 #112 meeting, FR2-1 SSB based L3 measurement delay reduction for connected mode was discussed and the WF was approved [1].

In this paper, we present our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC issues captured in the WF [1].

- Discussion

Status in the WF [1]:

- Issue 2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF [Way forward]: FFS: - Option 1 (CATT): Not to preclude the solutions based on 3 searchers assumption in current stage. - Option 1a (CMCC, HW): it is proposed to consider both the CSSF optimization based on 2 searchers and the CSSF optimization based on 3 searchers. - Option 1b (QC): RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS). - Option 2 (Apple, CTC, Nokia, Ericsson): RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF. - Option 2a (Apple): If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. - Option 2b (Nokia) RAN4 to confirm if one of the searchers is assumed for PCC/PSCC measurement and the other is assumed for the measurements on all SCCs.

For searcher assumption to apply L3 measurement delay reduction by optimizing CSSF, since the baseline assumption on number of searchers is 2 in WID, it’s proposed to consider the enhancement based on 2 searchers firstly. The enhancement on the number of searchers can be considered later or in further release.

Proposal 1: For searcher assumption to apply L3 measurement delay reduction by optimizing CSSF, it’s proposed to consider the enhancement based on 2 searchers firstly.

Status in the WF [1]:

- Issue 2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC [Way forward]: FFS: - Option 1 (Apple, CTC. Ericsson, Intel, Samsung, ZTE): UE only needs to measure one serving CC per band if multiple serving CCs are in the same band - Option 2 (CATT, Apple, CTC, ZTE): UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions - Option 4 (CATT, HW): - For UE supporting per FR gap, when all MOs are to be measured outside gap, the searcher used for within gap can be leveraged for outside gap. CSSFoutsidegap can be optimized, as these MOs can share totally three searchers. (HW, CATT) - We are open to further discuss the applicable condition of the three searcher solution and whether a new capability is needed. (HW)

For solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, CSSF value is related to the number of serving cells, with large number of serving cells, L3 measurement delay would be long, thus the number of serving cells related enhancements in CSSF equation can be considered as a starting point. We support to allow UE to measure one serving CC per band if multiple serving CCs are in the same band, and the measurement result of this serving CC can be reused for other serving CCs. Besides, the solution of changing CSSF allocation (statically or dynamically) among PCC/PSCC/SCC can be considered, UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

Proposal 2: UE only needs to measure one serving CC per band if multiple serving CCs are in the same band.

Proposal 3: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

Status in the WF [1]:

- Issue 2-6: feature capability of L3 measurement delay reduction by optimizing CSSF [Way forward]: FFS: - Option 1 (Apple, Nokia): RAN4 to introduce a new individual capability for CSSF reduction in R19. - Option 1a(Nokia):The reduced CSSF shall be applied to the UE supporting the capability and starting from R19 - Option 2 (CATT, Apple): delay the capability discussion to the end of the core part.

It’s proposed to introduce a new individual capability for CSSF reduction in R19, and the capability discussion can be delayed to the end of the core part.

Proposal 4: It’s proposed to introduce a new individual capability for CSSF reduction in R19, and the capability discussion can be delayed to the end of the core part.

Status in the WF [1]:

- Issue 2-7: Other WID scope discussion [Way forward]: FFS: - Proposal 1(CMCC, CTC): after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded, the technical solutions can be extended to FR1 if applicable.

The bands in FR1 are widely used in network deployment, there also exists the demand of L3 measurement delay reduction in FR1, from operator's perspective, the solution of enhanced CSSF can be applied in FR1 if applicable, after the work for FR2-1 L3 measurement delay reduction is done.

Besides, in RAN #103 meeting, it was regarded as common understanding that RAN4 will consider to apply CSSF enhancements in FR1 once the work for FR2-1 is done, in our views, the CSSF enhancements in FR1 is also in the WID scope.

Proposal 5: The solution of enhanced CSSF can be applied in FR1 if applicable, after the work for FR2-1 L3 measurement delay reduction is done.

- Conclusion

The contribution presents our views on FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC, with the following proposals:

Proposal 1: For searcher assumption to apply L3 measurement delay reduction by optimizing CSSF, it’s proposed to consider the enhancement based on 2 searchers firstly.

Proposal 2: UE only needs to measure one serving CC per band if multiple serving CCs are in the same band.

Proposal 3: UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions.

Proposal 4: It’s proposed to introduce a new individual capability for CSSF reduction in R19, and the capability discussion can be delayed to the end of the core part.

Proposal 5: The solution of enhanced CSSF can be applied in FR1 if applicable, after the work for FR2-1 L3 measurement delay reduction is done.

- References

- R4-2414026, WF for NR_RRM_Ph5_Part1, Apple, RAN WG4 Meeting #112, 19 – 23 August, 2024.