# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415735

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.20.3

Source:	China Telecom

Title:	Discussion on RRM core requirements for evolution of NR duplex operation

Document for:	Discussion

- Introduction

In RAN4 #112 meeting, RRM core requirements for evolution of NR duplex operation was discussed and the WF was approved [1].

In this paper, we present our views on RRM core requirements for evolution of NR duplex operation issues captured in the WF [1] and summary [2].

- Discussion

RRM impacts of UE-to-UE CLI handling

Status in the summary [2]:

- Issue 2-1-2: Measurement quantities - Proposals - Proposal 1 (Apple, HW, E//): - RAN4 to first focus on L1-SRS-RSRP and L1-CLI-RSSI, and FFS if other measurement quantities to be considered pending on further RAN1 conclusion - Proposal 2 (MTK): - RAN4 RRM should wait for RAN1 WG to define the new measurement quantities and framework before defining the new RRM requirements. - Recommended WF - RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. This can be revisited based on further RAN1 agreement - FFS if other measurement quantities to be considered in the core requirements based on further RAN1 agreement

In RAN1 #118 meeting, it was agreed to support two additional report quantities {‘cli-RSSI’, ‘cli-SRS-RSRP’} to the higher layer parameter reportQuantity for L1 UE-to-UE CLI measurement and reporting. Therefore, RAN4 shall define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements.

Proposal 1: RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements.

Status in the summary [2]:

- Issue 2-1-3: Baseline for defining requirements - Proposals - Proposal 1 (CATT, LGE, CTC, E///, Samsung): - R16 CLI requirements can be used as a baseline for defining CLI requirements for SBFD. - Proposal 2 (Nokia): - The CLI measurement requirement can be specified following L1-RSRP measurement and same assumption can be applied - Recommended WF - Both R16 CLI measurement requirements and L1-RSRP measurement requirements are to be considered for defining requirements for L1 based UE-to-UE CLI measurement. - RAN4 can directly discuss the requirements for L1 based UE-to-UE CLI measurement in next meeting.

Rel-19 enhancements for CLI handling are focused on L1 based UE-to-UE CLI measurement and reporting, in our views, Rel-16 L3 CLI measurement requirements and L1-RSRP measurement requirements can be used as baseline for defining requirements for L1 based UE-to-UE CLI measurement.

Proposal 2: Rel-16 L3 CLI measurement requirements and L1-RSRP measurement requirements can be used as baseline for defining requirements for L1 based UE-to-UE CLI measurement.

Status in the summary [2]:

- Issue 2-1-4: Measurement methods - Background Agreement For SBFD aware UEs, CLI measurements is performed within the active DL BWP and the following can be considered - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Method#3: UE measures RSSI within UL subband - Method#4: UE measures RSSI within guard band, if guard band exists Note: If DL subband, UL subband or guard band is outside the active DL BWP, the above methods does not apply. Note: Method#4 does not imply that guard band is explicitly configured.. - Proposals - Proposal 1 (Nokia): - RAN4 to prioritize specifying the measurement requirements for the CLI measurements in SBFD on UL subbands of victim UEs i.e. Method#2 and #3 - Proposal 2 (LGE): - RAN4 need to discuss how to measure the UL CLI leakage by UL sub-band aggressor UE at DL sub-band at the UL sub-band UE as aggressor and DL sub-band UE as victim in same DL BWP scenario. - Proposal 3 (E///, ZTE): - The measurement period requirements for the four candidate UE-to-UE CLI measurements should be discussed one by one. - Proposal 4 (QC): - RAN4 to discuss the measurement bandwidth for L1 CLI measurements, e.g., DL subband. - When more than one DL sub-bands are configured, RAN4 to discuss whether the measurement quantities can be filtered/combined across DL subbands. - Recommended WF - RAN4 to wait for RAN1 conclusion on the supported methods of L1 based UE-to-UE CLI measurement. - Meanwhile, RAN4 to discuss the impacts of different measurement methods on the requirements.

In RAN1 #118 meeting, it was agreed that for L1 UE-to-UE CLI measurement and reporting, CLI measurement is performed within the active DL BWP, Method#1 and Method#2 are supported, and FFS for Method#3. In our views, RAN4 can discuss the requirements for UE-to-UE CLI measurements based on Method#1 and Method#2.

Proposal 3: RAN4 to discuss the requirements for UE-to-UE CLI measurements based on Method#1 and Method#2.

Status in the summary [2]:

- Issue 2-1-9: Measurement reporting - Proposals - Proposal 1 (CATT, Nokia, CTC, Samsung): - RAN4 to discuss measurement reporting requirements for L1 based UE-to-UE CLI measurement - Proposal 2 (CATT): - More RAN1 progress on reporting design is needed for defining CLI reporting requirements - Proposal 3 (Nokia): - At least the L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting - Proposal 4 (Samsung): - Focus on Aperiodic CSI reporting mode for Rel-19 CLI measurement, for other reporting modes hold on the discussion pending on further progress in RAN1. - Recommended WF - RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting - FFS on reporting requirements for periodic and semi-persistent reporting pending on RAN1 agreement - FFS whether L1-RSRP measurement reporting requirement can be re-used.

L1 based UE-to-UE CLI measurement and reporting are based on existing CSI framework, measurement resource can be periodic, semi-persistent, or aperiodic measurement resource, and measurement reporting is at least aperiodic reporting. Rel-19 L1 CLI measurement reporting is different from Rel-16 CLI measurement reporting including periodic, event-triggered periodic, event triggered reporting, therefore, Rel-19 L1 CLI measurement reporting requirements need to be specified. RAN4 shall define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting.

Proposal 4: RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting.

Status in the summary [2]:

- Issue 2-1-10: Measurement accuracy - Proposals - Proposal 1 (CATT, CTC, E///, vivo, Samsung, HW, ZTE, QC): - RAN4 to discuss measurement accuracy requirements for L1 based UE-to-UE CLI measurement - Proposal 2 (CATT): - No performance requirements need to be defined for RS index reporting - More RAN1 progress on resource configurations is needed to decide if legacy CLI measurement accuracy requirements can be reused. - Proposal 3 (CTC): - Define better measurement requirements for Rel-19 L1 CLI measurement compared to Rel-16 L3 CLI measurement - Proposal 4 (ZTE): - The legacy UE-to-UE CLI accuracy requirements can not be reused directly. Some adaptation to facilitate L1 measurement should be considered. - Recommended WF - RAN4 to define measurement accuracy requirements for L1 based UE-to-UE CLI measurement based on the agreed side condition and measurement period.

For L1 UE-to-UE CLI measurement accuracy requirements, it’s proposed to define measurement accuracy requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements. Since Rel-16 CLI measurement requirements are based on L3 framework while Rel-19 CLI measurement requirements are based on L1 framework, it’s proposed not to reuse legacy UE-to-UE CLI accuracy requirements directly.

Proposal 5: RAN4 to define L1 UE-to-UE CLI measurement accuracy requirements, and it is proposed not to reuse legacy UE-to-UE CLI accuracy requirements directly.

Status in the summary [2]:

- Issue 2-1-11: Scheduling and measurement restriction - Proposals - Proposal 1 (CATT, Nokia, CTC, vivo, Samsung, HW, QC): - RAN4 to discuss scheduling and measurement restriction for L1 based UE-to-UE CLI measurement - Proposal 2a (Nokia): - RAN4 to start discussing the CLI measurements in SBFD without considering dynamic TDD in R19 - RAN4 shall wait for RAN1 conclusion on the CLI measurement methods before discussing the measurement and scheduling restriction. - Proposal 2b (Samsung): - UE measurement behaviour and corresponding measurement restriction and/or scheduling availability pending on further progress from RAN1, e.g. measurement methods and Rx beam assumption - Recommended WF - RAN4 to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement. - FFS the impact of measurement methods and Rx beam assumption

For scheduling and measurement restriction of L1 UE-to-UE CLI measurement, scheduling restriction are defined for SRS-RSRP and CLI-RSSI in Rel-16, it is proposed to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement and to discuss whether to define same scheduling restriction as Rel-16 CLI measurement for Rel-19 L1 based UE-to-UE CLI measurement.

Proposal 6: It’s proposed to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement and to discuss whether to define same scheduling restriction as Rel-16 CLI measurement for Rel-19 L1 based UE-to-UE CLI measurement.

- Conclusion

The contribution presents our views on RRM core requirements for evolution of NR duplex operation, with the following proposals:

Proposal 1: RAN4 to define core requirements for L1-SRS-RSRP and L1-CLI-RSSI measurements.

Proposal 2: Rel-16 L3 CLI measurement requirements and L1-RSRP measurement requirements can be used as baseline for defining requirements for L1 based UE-to-UE CLI measurement.

Proposal 3: RAN4 to discuss the requirements for UE-to-UE CLI measurements based on Method#1 and Method#2.

Proposal 4: RAN4 to define measurement reporting requirements for L1 based UE-to-UE CLI measurement at least for aperiodic reporting.

Proposal 5: RAN4 to define L1 UE-to-UE CLI measurement accuracy requirements, and it is proposed not to reuse legacy UE-to-UE CLI accuracy requirements directly.

Proposal 6: It’s proposed to define scheduling and measurement restriction for L1 based UE-to-UE CLI measurement and to discuss whether to define same scheduling restriction as Rel-16 CLI measurement for Rel-19 L1 based UE-to-UE CLI measurement.

- References

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon, RAN WG4 Meeting #112, 19 – 23 August, 2024.

- R4-2411816, Topic summary for [112][221] NR_duplex_evo, Huawei, HiSilicon, RAN WG4 Meeting #112, 19 – 23 August, 2024.