# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415736

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.22.2.1

Source:	China Telecom

Title:	Discussion on On-demand SSB SCell operation for enhancements of network energy savings for NR

Document for:	Discussion

- Introduction

In RAN4 #112 meeting, enhancements of network energy savings for NR was discussed and the WF was approved [1].

In this paper, we present our views on On-demand SSB SCell operation for enhancements of network energy savings for NR issues captured in the WF [1] and summary [2].

- Discussion

General On-demand SSB(OD-SSB) requirements

Status in the summary [2]:

Issue 2-1-4: OD-SSB based L3 neighbour cell measurement - Proposals - Option 1: Huawei - RAN4 to deprioritize OD-SSB based L3 neighbour cell measurement, as feasibility and reliability are questionable and gain is not clear. - Option 2: Samsung - RAN4 needs to clarify if L3 measurements based on on-demand SSB is needed or not. - Recommended WF - Moderator suggests the group to check whether the following proposal can be agreed. - RAN4 to focus on OD-SSB based L3 serving cell measurement and deprioritize OD-SSB based L3 neighbour cell measurement.

In our views, OD-SSB based L3 serving cell measurement is beneficial for network energy savings, but the benefits of OD-SSB based L3 neighbour cell measurement is not very clear, therefore, it’s proposed to focus on OD-SSB based L3 serving cell measurement and deprioritize OD-SSB based L3 neighbour cell measurement.

Proposal 1: RAN4 to focus on OD-SSB based L3 serving cell measurement and deprioritize OD-SSB based L3 neighbour cell measurement.

Status in the summary [2]:

Issue 2-1-5: OD-SSB type - Background RAN1 #116bis Agreement - For a cell supporting on-demand SSB SCell operation, - Note: It is up to gNB implementation whether always-on SSB (if transmitted) on the cell is cell-defining SSB or not. - For on-demand SSB on the cell, downselect between the following alternatives - Alt-1: It is up to gNB implementation whether on-demand SSB is cell-defining SSB or not. - Alt-2: On-demand SSB is limited to non-cell-defining SSB. - FFS: Further limitations to on-demand SSB. - Proposals - Option 1: Qualcomm - RAN4 should start working on OD-SSB requirements for NCD-SSB. - Defining OD-SSB requirements for CD-SSB should be postponed till RAN1 has decided. - Recommended WF - Moderator suggests the group to check whether the following proposal can be agreed. - RAN4 to define OD-SSB based requirements for NCD-SSB first

In RAN1 #118 meeting, it was agreed that On-demand SSB on the cell is not located on synchronization raster and is non-cell-defining SSB, FFS for additional support of OD-SSB for CD-SSB located on sync-raster. Therefore, RAN4 shall start to discuss OD-SSB requirements for NCD-SSB type, and the discussion on OD-SSB requirements for CD-SSB type should wait until RAN1 has conclusion.

When the type of OD-SSB is NCD-SSB, always-on SSB is also NCD-SSB. The reason is that, the periodicity of OD-SSB is smaller than the periodicity of always-on SSB in order to achieve NES gains, if always-on SSB is CD-SSB, it was defined in the specification that network configures only periodicities of NCD-SSB are larger than the periodicity of serving cell's CD-SSB, which is opposite to NES condition. Network can configure frequency, periodicity, time offset for NCD-SSB, the frequencies, periodicities, time offsets of OD-SSB and always-on SSB can be same or different. In our views, we can start the discussion on same frequency and time offset, different periodicities firstly.

Proposal 2: RAN4 shall start to discuss OD-SSB requirements for NCD-SSB type, and the discussion on OD-SSB requirements for CD-SSB type should wait until RAN1 has conclusion.

Proposal 3: The frequencies, periodicities, time offsets of OD-SSB and always-on SSB can be same or different, it’s proposed to start the discussion on same frequency and time offset, different periodicities firstly.

Status in the summary [2]:

Issue 2-1-6: OD-SSB based deactivated SCell measurement components - Proposals - Option 1: vivo - RAN4 to clarify the deactivated SCell measurement components includes both cell identification and measurement.

In our views, RAN4 to define OD-SSB based deactivated SCell measurement includes cell identification requirements and measurement period requirements just like legacy requirements of intra-frequency measurements without measurement gaps.

Proposal 4: RAN4 to define OD-SSB based deactivated SCell measurement includes cell identification requirements and measurement period requirements.

Status in the summary [2]:

Issue 2-1-6: OD-SSB SCell activation requirement - L1 measurements - Proposals - Option 1: Intel - RAN4 specifies SCell activation delay requirements considering L1 measurements based on OD-SSB configurations during the SCell activation process. - Option 2: Samsung - RAN4 discusses the impacts on L1 measurements. - Option 3: Huawei - For L1 measurement based on OD SSB, which is already agreed in RAN1, RAN4 to wait for further RAN1 progress. - Recommended WF - Further discussion

It was agreed in RAN1 to support L1 measurement based on On-demand SSB, and for L1 measurement based on On-demand SSB, periodic, semi-persistent, and aperiodic L1 measurement reports based on existing CSI framework are supported. In our understanding, for Case #1, there is no always-on SSB on the cell, UE activates the SCell based on OD-SSB, SCell activation procedures including L1 measurement need to be performed based on OD-SSB. Therefore, it’s proposed to specify SCell activation delay requirements including L1 measurements based on OD-SSB during the SCell activation procedure.

Proposal 5: It’s proposed to specify SCell activation delay requirements including L1 measurements based on OD-SSB during the SCell activation procedure.

OD-SSB requirements (Case 1)

Status in the WF [1] and summary [2]:

Deactivated SCell L3 measurement part Issue 2-2-1: Case 1- OD-SSB based deactivated SCell measurement - Proposals - Option 1: For deactivated SCell measurement requirements, follow the periodicity of configured OD-SSB (not the measurement cycle). - Option 2: For deactivated SCell measurement requirements, follow the measCycleSCell. - Other option is not precluded

For OD-SSB based deactivated SCell measurement requirements, it’s proposed to follow the periodicity of configured OD-SSB, which is beneficial for network energy savings.

Proposal 6: For OD-SSB based deactivated SCell measurement requirements, considering gains of network energy savings, it’s proposed to follow the periodicity of configured OD-SSB.

Status in the WF [1] and summary [2]:

SCell activation part Issue 2-2-6: Case 1 - SCell activation known condition - Proposals - Option 1: CATT - RAN4 to discuss known condition of SCell activation based on on-demand SSB measurement considering the following possible aspects. - Option 1a: The configuration of the on-demand SSB and the measurement report.(Qualcomm) - Option 1b: The new L3 deactivated SCell measurement requirement.(CMCC for Scenario 2) - Recommended WF - Further discussion Issue 2-2-7: Case 1 - OD-SSB indication in known SCell activation - Proposals - Option 1: Ericsson - RAN4 to support the following scenarios for OD-SSB based known SCell activation. - Scenario 1: Single OD-SSB indication for OD-SSB transmission in both deactivated SCell measurement phase and SCell activation phase - Scenario 2: Two separate OD-SSB indications for OD-SSB transmission in deactivated SCell measurement and SCell activation phases separately - Recommended WF - Further discussion Issue 2-2-8: Case #1 and Scenario #2 - known SCell activation - Proposals - Option 1: Ericsson, CATT - When OD-SSB transmission is earlier than the SCell activation command, UE follows the legacy Rel-15 known SCell activation requirement if the known cell condition is met. - FFS: Clarify that the SSB used in activation procedure is on-demand SSB - FFS: the OD-SSB transmission time instance A - Recommended WF - Further discussion Issue 2-2-9: Case #1 and Scenario #2 - unknown SCell activation - Proposals - Option 1: Ericsson - When OD-SSB transmission is earlier than the SCell activation command, UE follows the legacy Rel-15 unknown SCell activation requirement once the unknown cell condition is met. - FFS: Clarify that the SSB used in activation procedure is on-demand SSB - FFS: the OD-SSB transmission time instance A - Recommended WF - Further discussion Issue 2-2-10: Case #1 and Scenario #2A - unknown SCell activation - Wayforward: - Option 1: RAN4 to only define unknown SCell activation provided that the OD-SSB is the first OD-SSB triggered by NW for (Case #1, Scenario #2A). - Option 2: RAN4 only specifies requirements assuming the target SCell is unknown in (Case #1, Scenario #2A). - Option 3: RAN4 needs to further consider the scenario (Case #1, Scenario #2A) together with (Case #1, Scenario #2) for deactivated SCell OD-SSB measurement. - Other option is not precluded

For Case #1 and Scenario #2, OD-SSB is triggered before UE receives SCell activation command, and OD-SSB may be transmitted before SCell activation command, thus L3 deactivated SCell measurement may be performed if OD-SSB is transmitted before SCell activation command. If the known SCell conditions are met, the SCell to be activated can be considered as known SCell, otherwise, the SCell to be activated can be considered as unknown SCell. Therefore, RAN4 to define OD-SSB based known and unknown SCell activation requirements for Case #1 and Scenario #2.

For Case #1 and Scenario #2A, there is no SSB transmitted on the SCell before SCell activation command, the SCell to be activated is considered as unknown. Therefore, RAN4 to define OD-SSB based unknown SCell activation requirements for Case #1 and Scenario #2A.

Proposal 7: For Case #1, it’s proposed to define OD-SSB based known and unknown SCell activation requirements, the SCell to be activated is considered as known/ unknown SCell when the known/ unknown SCell conditions are met.

OD-SSB requirements (Case 2)

Status in the summary [2]:

Deactivated SCell measurement part Issue 2-3-1: Case 2 Scenario 2 - Deactivated SCell measurement - Proposals - Option 1: Ericsson - The UE is assumed to perform measurement based on legacy SSB before UE receiving OD-SSB transmission indicated during deactivated Scell. - The UE is assumed to switch to OD-SSB to perform measurement/Scell activation when OD-SSB transmission is indicated. - Option 2: Nokia - RAN4 to discuss how to handle the measurement on the always-on SSBs and OD-SSBs. - The UE measurement based on always-on SSBs in the SCell shall be considered to reduce the SCell activation delay. - Option 3: CMCC - L3 measurement requirement after on-demand SSB triggering and during the SSB periodicity transition period should be re-designed . - Option 4: Xiaomi - For Rel19 NES capable UE, the requirements under case2 shall assume only OD-SSB to be used. - Recommended WF - Further discussion Issue 2-3-2: Case 2 Scenario 2A - Deactivated SCell measurement - Proposals - Option 1: CMCC - In case 2 scenario 2A, the legacy L3 measurement requirement can be reused. - Recommended WF - Further discussion

For Case #2, always-on SSB is periodically transmitted on the cell, before UE receives OD-SSB transmission indication, deactivated SCell measurement can be performed based on always-on SSB. After UE receives OD-SSB transmission indication and when OD-SSB is transmitted on the cell, UE can perform deactivated SCell measurement based on OD-SSB, or based on both OD-SSB and always-on SSB, if deactivated SCell measurement is based on both OD-SSB and always-on SSB, how to combine the measurement results based on both OD-SSB and always-on SSB can be further discussed.

Proposal 8: For Case #2, always-on SSB is periodically transmitted on the cell, before UE receives OD-SSB transmission indication, deactivated SCell measurement can be performed based on always-on SSB.

Proposal 9: After UE receives OD-SSB transmission indication and when OD-SSB is transmitted on the cell, UE can perform deactivated SCell measurement based on OD-SSB, or based on both OD-SSB and always-on SSB, if deactivated SCell measurement is based on both OD-SSB and always-on SSB, how to combine the measurement results based on both OD-SSB and always-on SSB can be further discussed.

Status in the summary [2]:

SCell activation part Issue 2-3-3: Case 2 Scenario 2: SCell activation - Proposals - Option 1: CATT - RAN4 to discuss how to handle the different configurations e.g., periodicity of two types of SSB. - Option 2: Nokia - RAN4 shall specify the SCell activation requirement based on OD-SSB in case 2 scenario 2A. - Recommended WF - Further discussion Issue 2-3-4: Case 2 Scenario 2: known condition for SCell activation - Proposals - Option 1: CMCC - The legacy FR1 known condition should be re-designed. - Recommended WF - Further discussion

For Case #2, always-on SSB is periodically transmitted on the cell, after UE receives OD-SSB transmission indication and when OD-SSB is transmitted on the cell, UE can perform deactivated SCell measurement based on OD-SSB, or based on both OD-SSB and always-on SSB. If the known SCell conditions are met, the SCell to be activated can be considered as known SCell, otherwise, the SCell to be activated can be considered as unknown SCell. Therefore, RAN4 to define OD-SSB based known and unknown SCell activation requirements for Case #2.

Proposal 10: For Case #2, it’s proposed to define OD-SSB based known and unknown SCell activation requirements, the SCell to be activated is considered as known/ unknown SCell when the known/ unknown SCell conditions are met.

- Conclusion

The contribution presents our views on RRM core requirements for On-demand SSB SCell operation for enhancements of network energy savings for NR, with the following proposals:

Proposal 1: RAN4 to focus on OD-SSB based L3 serving cell measurement and deprioritize OD-SSB based L3 neighbour cell measurement.

Proposal 2: RAN4 shall start to discuss OD-SSB requirements for NCD-SSB type, and the discussion on OD-SSB requirements for CD-SSB type should wait until RAN1 has conclusion.

Proposal 3: The frequencies, periodicities, time offsets of OD-SSB and always-on SSB can be same or different, it’s proposed to start the discussion on same frequency and time offset, different periodicities firstly.

Proposal 4: RAN4 to define OD-SSB based deactivated SCell measurement includes cell identification requirements and measurement period requirements.

Proposal 5: It’s proposed to specify SCell activation delay requirements including L1 measurements based on OD-SSB during the SCell activation procedure.

Proposal 6: For OD-SSB based deactivated SCell measurement requirements, considering gains of network energy savings, it’s proposed to follow the periodicity of configured OD-SSB.

Proposal 7: For Case #1, it’s proposed to define OD-SSB based known and unknown SCell activation requirements, the SCell to be activated is considered as known/ unknown SCell when the known/ unknown SCell conditions are met.

Proposal 8: For Case #2, always-on SSB is periodically transmitted on the cell, before UE receives OD-SSB transmission indication, deactivated SCell measurement can be performed based on always-on SSB.

Proposal 9: After UE receives OD-SSB transmission indication and when OD-SSB is transmitted on the cell, UE can perform deactivated SCell measurement based on OD-SSB, or based on both OD-SSB and always-on SSB, if deactivated SCell measurement is based on both OD-SSB and always-on SSB, how to combine the measurement results based on both OD-SSB and always-on SSB can be further discussed.

Proposal 10: For Case #2, it’s proposed to define OD-SSB based known and unknown SCell activation requirements, the SCell to be activated is considered as known/ unknown SCell when the known/ unknown SCell conditions are met.

- References

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson, RAN WG4 Meeting #112, 19 – 23 August, 2024.

- R4-2411817, Topic summary for [112][222] Netw_Energy_NR_enh, Ericsson, RAN WG4 Meeting #112, 19 – 23 August, 2024.