3GPP TSG-RAN WG4 Meeting #112bis	R4-2415740

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.27.2

Source:	MediaTek Inc.

Title:	Discussion on RF requirement impact for IoT NTN phase 3

Document for:	Discussion

- Introduction

In RAN#104 plenary meeting, the WID of “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3” was approved in [1]. One of the main objectives is to enable multiplexing of multiple UEs via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH and to update RF requirements accordingly if needed.

In this contribution, our views on the RF requirements impact are provided for discussion.

- Discussion

2.1		RF requirement impact for IoT-NTN phase 3

Some of the main Rel-19 WID [1] objectives for IoT NTN are shown below for reference.

- Support of Store&Forward (S&F) satellite operation with full eNB as regenerative payload, therefore: - Define the necessary enhancements into E-UTRAN (network & UE) to support S&F operation for delay-tolerant services [RAN3, RAN2] - At least specify necessary enhancements e.g. related to S1 protocol, especially to address the feeder link switch over as needed [RAN3] Note: Strive to minimise UE impact. Note: Coordination with SA2 (Rel-19 SA2 led Sat-Arch ph3 SI) is needed on the detail requirements (e.g. traffic type, or QoS parameters for S&F), network architecture (e.g. whether consider (partial) core network on satellite) etc.; further coordination with CT1 might be required - Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account

In RAN4#112 meeting, the agreed WF [2] provides the following works for evaluating RF requirement impact. The contents are shown as follows.

RF core requirements NOTE1: Enhancements to enable multiplexing of multiple UEs in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH. Issue 2-1: UE RF requirement impact for NPUSCH with OCC feature - Agreement: - No UE RF requirement impact from symbol-level for NPUSCH - FFS on UE RF requirement impact from slot-level for NPUSCH Issue 2-2: UE RF requirement impact for NPRACH with OCC feature - Agreement: - Wait RAN1 reach conclusions on OCC feature before RAN4 evaluates the UE RF impact. Issue 2-3: SAN RF requirement impact for NPUSCH/NPRACH with OCC feature - Agreement: - Wait RAN1 reach conclusions for OCC feature before RAN4 evaluates the SAN RF impact.

Regarding Tdocs in RAN1 meetings, the agreed proposals [3-5] and useful evaluations [6-7] are provided below for reference.

RAN1#116bis “Session notes for 9.11.4 Non-Terrestrial Networks (NTN) for NR Phase 3” [3] Agreement For OCC of NPUSCH format 1, RAN1 will not consider multiplexing more than 4 UEs.

RAN1#118 R1-2407297 “FL Summary #2 for IoT-NTN” [4] contents below for reference: Agreement The following combinations are considered for further simulation in RAN1 for 3.75kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 2: OCC2, Symbol-level, CDM DMRS with new pattern - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 6: OCC4, Symbol-level, CDM DMRS with new pattern The following combinations are considered for further simulation in RAN1 for 15kHz SCS OCC for NPUSCH format 1: - Option 1: OCC2, Symbol-level, TDM DMRS - Option 3: OCC2, Slot-level, TDM DMRS - Option 4: OCC2, Slot-level, CDM DMRS with legacy pattern - Option 5: OCC4, Symbol-level, TDM DMRS - Option 7: OCC4, Slot -level, TDM DMRS - Option 8: OCC4, Slot-level, CDM DMRS with legacy pattern Note 1: For TDM, the legacy DMRS pattern, with DMRS symbols appropriately muted/blanked is used. Companies to report their assumption on whether spreading is applied to the legacy DMRS pattern for 15 kHz SCS. Note 2: Companies to report DMRS sequence applied. Agreement For 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

RAN1#118 R1-2407563 “FL Summary #1 for IoT-NTN” [5] contents below for reference: Proposed Agreement - For 3.75kHz SCS single-tone transmission OCC for NPUSCH format 1, symbol-level OCC is supported. - For 15kHz SCS single-tone transmission OCC for NPUSCH format 1, slot-level OCC is supported.

From SAN’s perspective, to avoid too much phase difference between UEs at maximum frequency offset (e.g., +/- 0.1ppm) is preferred when combing the limited number of UEs (e.g., 4 UEs).

RAN1#118 R1-2405842 “Discussion on UL capacity enhancements for IoT NTN” [6] The eNB may estimate CFO of NPUSCH from two consecutive DMRS symbols and the phase rotation between these two consecutive DMRS symbols should be within the range of [-pi, pi]. As shown in Table 1, phase rotation for 3.75 kHz SCS is about 4 times that of 15kHz SCS due to the longer slot duration. Table 1 Phase rotation between DMRS symbols caused by 200Hz CFO Phase rotation. Distance between two DMRS symbols; 1 slot; 2 slots. 15 kHz SCS; 0.18pi; 0.36pi. 3.75 kHz SCS; 0.75pi; 1.5pi. - OCC2; (b) OCC4. Figure 5 - Performance of 15 kHz Single-tone NPUSCH format 1 w/ and w/o OCC

Based on references [4][5][6], there was reasonable understanding as follows:

    - Symbol-level OCC worked at 3.75kHz SCS

    - Slot-level OCC may not work well at high SNR/MCS at 3.75kHz SCS due to accumulated phase error from CFO

    - Slot-level OCC worked at 15kHz SCS

    - Although slot-level OCC may work at 15kHz SCS, the simulation results show that symbol-based had better performance.

RAN1#118 R1-2406780 “Discussion on IoT-NTN uplink capacity and throughput” [7] A ST 15 kHz is specified in TS 38.211 Table 10.1.2.3-1 to be NslotsUL=16 with a slot duration 0.5 ms and ST 3.75 kHz kHz is NslotsUL=16 with a slot duration 2 ms. The time duration of OCC applied to repetitions of NPUSCH Format 1 is 16 ms for OCC length 2 and 32 ms for OCC length 4 with ST 15 kHz, and 64 ms for OCC length 2 and 128 ms for OCC length 4 with ST 3.75 kHz Phase rotation over time duration X ms as summarize in Table 1can be calculated as - Δφ = 2π (rad) × timing drift (us/s) × X (ms) × channel bandwidth (Hz) /2 - Δφ = 2π (rad) × CFO × X (ms) × Fc/2 Where max time drift = 80 ppm and max CFO = ±0.1 ppm Observation 1: The maximum phase rotation due to timing drift or CFO may allow inter-slot OCC with NPUSCH Format 1 with OCC length 2 or 4 with ST 15 kHz, and OCC length 2 with ST 3.75 kHz. Max ΔΘ [deg] @ Timing drift 80 ppm; Max ΔΘ [deg] @ Max CFO ±0.1 ppm @ 2 GHz. ST 15 kHz; ST 3.75 kHz; ST 15 kHz; ST 3.75 kHz. 1.728 deg (OCC=2); 1.728 (OCC=2); 92 deg (OCC=2); 368 deg (OCC=2). 3.456 deg (OCC=4); 3.456 (OCC=4); 184 deg (OCC=4); 1472 deg (OCC=4). Table 1: Max phase rotation due to time drift and CFO with OCC schemes

Additionally, when we only do evaluation for CFO phase rotation for OCC2/OCC4 slots, it is observed that slot-level OCC2/OCC4 is much feasible for 15kHz SCS single-tone transmission OCC for NPUSCH format 1. However, the slot-level OCC2/OCC4 may be not feasible for 3.75kHz SCS single-tone transmission OCC for NPUSCH format 1 under high MCS operation.

| Max ΔΘ [deg] @ Max CFO ±0.1 ppm @ 2 GHz |  |
| --- | --- |
| ST 15 kHz | ST 3.75 kHz |
| 36 deg (OCC=2) | 144 deg (OCC=2) |
| 72 deg (OCC=4) | 288 deg (OCC=4) |

Table: Max phase rotation due to CFO with OCC schemes

Observation 1: Based on references [3-7], at least for 3.75kHz SCS single-tone transmission OCC for NPUSCH format 1, symbol-level OCC is feasible.

Observation 2: Based on references [3-7] and evaluation, slot-level OCC2/OCC4 may be not feasible for 3.75kHz SCS single-tone transmission OCC for NPUSCH format 1 under high MCS operation.

Observation 3: Based on references [3-7] and evaluation, at least for 15kHz SCS single-tone transmission OCC for NPUSCH format 1, slot-level OCC2 and OCC4 is feasible.

Observation 4: Based on reference [4], for 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

Proposal 1: For slot-level OCC2 of 15 kHz single-tone NPUSCH format 1, no UE RF requirement impact.

Proposal 2: For slot-level OCC4 of 15 kHz single-tone NPUSCH format 1, consider whether there are no UE RF requirement impacts at appropriate MCS (e.g., at least SNR in the range of -8dB to 0dB).

It is observed that RAN1 thinks symbol-level OCC would be feasible for 3.75kHz SCS single-tone NPUSCH and slot-level OCC would be feasible for 15kHz SCS single-tone NPUSCH.

- For 3.75kHz SCS single-tone transmission OCC for NPUSCH format 1, symbol-level OCC is supported.

- For 15kHz SCS single-tone transmission OCC for NPUSCH format 1, slot-level OCC is supported.

Proposal 3: Regarding slot-level OCC2/OCC4 of 3.75 kHz single-tone NPUSCH format 1, although high CFO phase offset has an impact on the used MCS performance, wait RAN1 conclusions on slot-level OCC feature for 3.75kHz single-tone NPUSCH.

3 Conclusion

In this paper, our views on RF requirements impact for NPUSCH with OCC feature are provided as follows:

Observation 1: Based on references [3-7], at least for 3.75kHz SCS single-tone transmission OCC for NPUSCH format 1, symbol-level OCC is feasible.

Observation 2: Based on references [3-7] and evaluation, slot-level OCC2/OCC4 may be not feasible for 3.75kHz SCS single-tone transmission OCC for NPUSCH format 1 under high MCS operation.

Observation 3: Based on references [3-7] and evaluation, at least for 15kHz SCS single-tone transmission OCC for NPUSCH format 1, slot-level OCC2 and OCC4 is feasible.

Observation 4: Based on reference [4], for 3.75kHz SCS, NPUSCH format 1 simulations are performed using an appropriate MCS with SNR at least in the range of -8dB to 0dB.

Proposal 1: For slot-level OCC2 of 15 kHz single-tone NPUSCH format 1, no UE RF requirement impact.

Proposal 2: For slot-level OCC4 of 15 kHz single-tone NPUSCH format 1, consider whether there are no UE RF requirement impacts at appropriate MCS (e.g., at least SNR in the range of -8dB to 0dB).

Proposal 3: Regarding slot-level OCC2/OCC4 of 3.75 kHz single-tone NPUSCH format 1, although high CFO phase offset has an impact on the used MCS performance, wait RAN1 conclusions on slot-level OCC feature for 3.75kHz single-tone NPUSCH.

4 Annex

TS 36.211 Table 10.1.2.3-1: Supported combinations of , , and  for frame structure type 1.

| NPUSCH format |  |  |  |  |
| --- | --- | --- | --- | --- |
| 1 | 3.75 kHz | 1 | 16 | 7 |
|  | 15 kHz | 1 | 16 |  |
|  |  | 3 | 8 |  |
|  |  | 6 | 4 |  |
|  |  | 12 | 2 |  |
| 2 | 3.75 kHz | 1 | 4 |  |
|  | 15 kHz | 1 | 4 |  |

Since the slot duration is 2ms, the RU length spans = 16 slots, which results in 32ms. Figure 1 depicts the slot format for NPUSCH Format 1 with 3.75 kHz SCS.

| CP+Data symbol (275 us) | CP+Data symbol (275 us) | CP+Data symbol (275 us) | CP+Data symbol (275 us) | CP+DMRS symbol (275 us) | CP+Data symbol (275 us) | CP+Data symbol (275 us) | Guard period (75 us) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Slot (2ms) |  |  |  |  |  |  |  |

Figure 1: slot format for NPUSCH Format 1 with 3.75 kHz SCS

Since the slot duration is 0.5ms, the RU length spans = 16 slots, which results in 8ms. Figure 2 depicts the slot format for NPUSCH Format 1 with 15 kHz SCS.

| CP+Data symbol | CP+Data symbol | CP+Data symbol | CP+DMRS symbol | CP+Data symbol | CP+Data symbol | CP+Data symbol |
| --- | --- | --- | --- | --- | --- | --- |
| Slot (0.5 ms) |  |  |  |  |  |  |

Figure 2: slot format for NPUSCH Format 1 with 15 kHz SCS.

TS 36.213 Table 16.5.1.1-3: Number of repetitions () for NPUSCH.

| 0 | 1 |
| --- | --- |
| 1 | 2 |
| 2 | 4 |
| 3 | 8 |
| 4 | 16 |
| 5 | 32 |
| 6 | 64 |
| 7 | 128 |

5 Reference

- RP-241624, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, MediaTek Inc.

- R4-2413530, “Way Forward for [112][316] IoT_NTN_Ph3”, MediaTek Inc.

- Session notes for 9.11.4 Non-Terrestrial Networks (NTN) for NR Phase 3, Agenda Item 9.11.4, Changsha, Hunan Province, China, April 15th – 19th, 2024.

- R1-2407297, “FL Summary #2 for IoT-NTN”, Sony

- R1-2407563, “FL Summary #1 for IoT-NTN”, Sony

- R1-2405842, “Discussion on UL capacity enhancements for IoT NTN”, Huawei, HiSilicon

- R1-2406780, “Discussion on IoT-NTN uplink capacity and throughput”, MediaTek Inc.