3GPP TSG-RAN WG4 Meeting #112bis	R4- 2415741

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda Item:	6.26.2.1

Source:	MediaTek Inc.

Title:	Discussion on Rel-19 NR-NTN RedCap UE RF requirements

Document for:	Discussion

- Introduction

In RAN#102 plenary meeting, the Rel-19 WID of Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in [1]. One of the main objectives is to support Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN band.

In RAN4#111 and 112 meetings, the WF [2-3] for guiding further discussion on residual issues were proposed.

In this contribution, our views about the residual issues are provided.

- Discussion

2.1		Guidance for Rel-19 phase 3 NR-NTN UE RF requirements

The Rel-19 WID [1] objectives indicate that NTN for NR Phase 3 supports Redcap and eRedcap with NR NTN operating in FR1-NTN band. The objectives are shown below.

4.1 Objective of SI or Core part WI or Testing part WI Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, check whether any essential changes are needed for their support (i.e. focusing on HD collision rules) by end of Q2/2024 [RAN1] - Depending on feasibility assessment above, define the RF and RRM requirements [RAN4] Notes for this objective: GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

The Rel-19 NR_NTN_Ph3_UERF WF [2] in RAN4#111 provided the guidance as follows.

Issue 1-2: HD-FDD refsens for 2 Rx Agreement: [0.5] dB tightening for n254 and n256, FFS for n255. n256 to be confirmed in RAN4#112. Note: RAN4#111 maintenance agreement on n256 refsens in R4-2408787 was not considered for this agreement.

Issue 1-5: Maximum output power options for further discussion: Option 1: Consider increase in PC3 power level for NTN RedCap UE which only supports HD-FDD operation for a band. Option 2: PC3 power level is the same for non-RedCap NR NTN UE and (e)RedCap NR NTN UE

The Rel-19 NR_NTN_Ph3_UERF WF [3] in RAN4#112 provided the guidance as follows.

Issue 1-2: HD-FDD refsens for 2 Rx Agreement: As baseline - n256: 0.5 dB tightening Issue 1-4: Simultaneous operation with GNSS Agreement: Further study NR NTN UE simultaneous operation with GNSS considering e.g. - Operating bands with UL close to GNSS frequencies (n254, n255) - Emissions on GNSS frequencies - Whether transmission gaps enabling successful GNSS reception are present - Potential impact on field performance - Potential need for specification updates, e.g. frequency error Issue 2-1: UL capacity enhancements Agreement: - Continue evaluation on potential UE RF specification impact - As starting point, no UE RF impact at least from inter-symbol OCC scheme

2.2		HD-FDD refsens for 2 Rx

Regarding HD-FDD REFSENS for 2 Rx, because NTN connection performance between SAN and UE is important, some comments indicate that it is desired to achieve more than 0.5 dB tightening on FR1 NTN bands. However, some companies prefer to keep the 0.5 dB tightening on band n256.

According to face-to face discussion in RAN4#111 and RAN4#112 meetings, regarding NTN FR1bands, it is observed that 2RX HD-FDD REFSENS could be further optimized compared to 2RX FD-FDD RFESENS.

Observation 1: Regarding NTN HD-FDD REFSENS for 2Rx, because NTN connection performance between SAN and UE is important and not as good as TN connection performance, some comments indicate that it is desired to achieve more than 0.5 dB tightening for FR1 NR-NTN bands.

Observation 2: In TS 38.101-5[4], 2Rx FD-FDD REFSENS are -100dBm/5MHz and -99.5dBm/5MHz for NTN n255 and n254, respectively.

Observation 3: Switching from FD-FDD mode to HD-FDD mode would improve REFSENS due to lower front-end insertion loss.

Proposal 1: Regarding 1.5GHz NTN band’s 2RX HD-FDD REFSENS, 0dB tightening seems not to optimize NTN REFSENS compared to FD-FDD operation, discuss n255 HD-FDD REFSENS within [-100dBm/5MHz ~ -100.5dBm/5MHz] for further decision. For example, n255 2RX HD-FDD REFSENS is -100.2dBm/5MHz.

Proposal 2: Regarding the 2RX HD-FDD REFSENS of NTN n254, discuss whether -100.1dBm/5MHz would be applicable if there is no TN bands’ hardware reused for NTN.

2.3		Maximum output power

It is observed that the HD-FDD RF front-end insertion loss (FE IL) is lower compared to FD-FDD FE IL and can improve the RX REFSENS and TX maximum output power (MOP) simultaneously.

Observation 4: HD-FDD FE IL is lower compared to FD-FDD IL, which can simultaneously improve HD-FDD RX REFSENS and TX MOP with 0.8dB.

Observation 5: When NTN-RedCap UE operates in HD-FDD mode for a band, it is observed that the HD-FDD TX MOP would be obviously better compared to FD-FDD TX MOP.

From EC Decision 2016/687[6], it is observed that a tolerance of up to + 2 dB is to take account of operation under extreme environmental conditions and production spread. Regarding UE TX output power, the power limits are specified as equivalent isotropically radiated power (EIRP) for terminal stations designed to be mobile or nomadic.

Observation 6: From EC Decision 2016/687[6], a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread.

From ETSI EN 301 908[7], it is observed that PC3 is specified as 23dBm +/-2.7dB or 23dBm +3/-4dB.

Observation 7: From ETSI EN 301 908[7], PC3 is specified with allowing higher power tolerance (i.e., 23dBm +/-2.7dB or 23dBm +3/-4dB).

In TS 38.521-1[8], it is observed that there is the additional FR1 UE TX output power test tolerance relative to the TX conductive output power upper bound. The partial contents are shown below for reference.

TS 38.521-1[8] Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR Band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±(2+TT) |
| n2 |  |  |  |  | 23 | ±(23+TT) |
| n3 |  |  |  |  | 23 | ±(23+TT) |
| n5 |  |  |  |  | 23 | ±(2+TT) |
| n7 |  |  |  |  | 23 | ±(23+TT) |

TS 38.521-1[8] Table 6.2.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Observation 8: Based on TS 38.521-1 [9] conformance-test specification, the actual UE TX PC3 conductive output power can be higher than PC3 upper bound (e.g., 25dBm).

From WF [9] on Rel-19 FR1 NTN bands, it is observed that NTN antenna gain for smartphone is -5.5dBi for evaluating NTN UE A-MPR.

Proposal 3: Because NTN UE antenna gain for smartphone of -5.5dBi is considered for evaluating NTN A-MPR, the NTN antenna gain of -5.5dBi is also used when evaluating NTN UE TX regulatory requirements.

Proposal 4: To increase HD-FDD PC3 nominal power (e.g., change 23dBm to 23.8dBm) or tolerance (e.g., change +2dB to +2.8dB) because HD-FDD TX nominal output power is expected to be higher (e.g., 23.8dBm) and therefore there is less margin to meet the upper bound (i.e., 23 dBm + 2 dB) over PVT.

In our understanding, the regulatory requirements regarding in-band TX output power are also associated with SAR testing.

Observation 9: The UE TX output power upper bound would also be considered when performing SAR testing. Compared with FD-FDD TX SAR, HD-FDD TX SAR issues have been further alleviated.

Proposal 5: If necessary, regarding NTN RedCap HD-FDD MOP or MOP upper bound, consider capability reporting, or add the note to indicate applicable region where feasible when there is regional restriction from regulatory requirements.

3 Conclusion

In this paper, our views on NTN RedCap RF requirements in HD-FDD mode are provided as follows:

Observation 1: Regarding NTN HD-FDD REFSENS for 2Rx, because NTN connection performance between SAN and UE is important and not as good as TN connection performance, some comments indicate that it is desired to achieve more than 0.5 dB tightening for FR1 NR-NTN bands.

Observation 2: In TS 38.101-5[4], 2Rx FD-FDD REFSENS are -100dBm/5MHz and -99.5dBm/5MHz for NTN n255 and n254, respectively.

Observation 3: Switching from FD-FDD mode to HD-FDD mode would improve REFSENS due to lower front-end insertion loss.

Proposal 1: Regarding 1.5GHz NTN band’s 2RX HD-FDD REFSENS, 0dB tightening seems not to optimize NTN REFSENS compared to FD-FDD operation, discuss n255 HD-FDD REFSENS within [-100dBm/5MHz ~ -100.5dBm/5MHz] for further decision. For example, n255 2RX HD-FDD REFSENS is -100.2dBm/5MHz.

Proposal 2: Regarding the 2RX HD-FDD REFSENS of NTN n254, discuss whether -100.1dBm/5MHz would be applicable if there is no TN bands’ hardware reused for NTN.

Observation 4: HD-FDD FE IL is lower compared to FD-FDD IL, which can simultaneously improve HD-FDD RX REFSENS and TX MOP with 0.8dB.

Observation 5: When NTN-RedCap UE operates in HD-FDD mode for a band, it is observed that the HD-FDD TX MOP would be obviously better compared to FD-FDD TX MOP.

Observation 6: From EC Decision 2016/687[6], a tolerance of up to +2 dB is to take account of operation under extreme environmental conditions and production spread.

Observation 7: From ETSI EN 301 908[7], PC3 is specified with allowing higher power tolerance (i.e., 23dBm +/-2.7dB or 23dBm +3/-4dB).

Observation 8: Based on TS 38.521-1 [9] conformance-test specification, the actual UE TX PC3 conductive output power can be higher than PC3 upper bound (e.g., 25dBm).

Proposal 3: Because NTN UE antenna gain for smartphone of -5.5dBi is considered for evaluating NTN A-MPR, the NTN antenna gain of -5.5dBi is also used when evaluating NTN UE TX regulatory requirements.

Proposal 4: To increase HD-FDD PC3 nominal power (e.g., change 23dBm to 23.8dBm) or tolerance (e.g., change +2dB to +2.8dB) because HD-FDD TX nominal output power is expected to be higher (e.g., 23.8dBm) and therefore there is less margin to meet the upper bound (i.e., 23 dBm + 2 dB) over PVT.

Observation 9: The UE TX output power upper bound would also be considered when performing SAR testing. Compared with FD-FDD TX SAR, HD-FDD TX SAR issues have been further alleviated.

Proposal 5: If necessary, regarding NTN RedCap HD-FDD MOP or MOP upper bound, consider capability reporting, or add the note to indicate applicable region where feasible when there is regional restriction from regulatory requirements.

4 Annex

TS 38.101-5[4] Table 5.2.2-1: NTN satellite bands in FR1-NTN

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

After checking TS 38.101-5 [4] and TS 38.101-1 [5], some tables are shown below for reference.

TS 38.101-5 [4] Table 7.3.2-1: Two antenna port reference sensitivity QPSK REFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz |
|  |  | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) | (dBm) |
| n256 | 15 | -99.5 | -96.3 | -94.5 | -93.8 |  |  |
|  | 30 |  | -96.6 | -94.6 | -94 |  |  |
|  | 60 |  | -97 | -94.9 | -94.2 |  |  |
| n255 | 15 | -100 | -96.8 | -95 | -93.8 |  |  |
|  | 30 |  | -97.1 | -95.1 | -94 |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |
| n254 | 15 | -99.5 | -96.3 | -94.5 |  |  |  |
|  | 30 |  | -96.6 | -94.6 |  |  |  |
|  | 60 |  | -97 | -94.9 |  |  |  |

Table 1: NR-TN and NR-NTN operating bands

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high |
| --- | --- | --- |
| n1 | 1920 MHz – 1980 MHz | 2110 MHz – 2170 MHz |
| n2 | 1850 MHz – 1910 MHz | 1930 MHz – 1990 MHz |
| n3 | 1710 MHz – 1785 MHz | 1805 MHz – 1880 MHz |
| n7 | 2500 MHz – 2570 MHz | 2620 MHz – 2690 MHz |
| n24 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz |

EC Decision 2016/687

The power limits are specified as equivalent isotropically radiated power (EIRP) for terminal stations designed to be fixed or installed and as total radiated power (TRP) (2) for terminal stations designed to be mobile or nomadic.

Total radiated power (TRP) is a measure of how much power the antenna actually radiates. The TRP is defined as the integral of the power transmitted in different directions over the entire radiation sphere.

ETSI EN 301 908

4.2.2 	Transmitter Maximum Output Power

4.2.2.1 	Transmitter maximum output power for Single Carrier

4.2.2.1.1 	Definition

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth. The period of measurement shall be at least one sub-frame (1 ms).

4.2.2.1.2 	Limits

The UE maximum output power shall be within the shown value in table 4.2.2.1.2-1.

Table 4.2.2.1.2-1: UE power classes

| E-UTRA Band | Power Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| 1 | 23 | ±2,7 |
| 3 | 23 | ±2,7 (see note) |
| 7 | 23 | ±2,7 (see note) |
| 8 | 23 | ±2,7 (see note) |
| 20 | 23 | ±2,7 (see note) |
| 22 | 23 | +3,0/-4,5 |
| 28 | 23 | +2,7/-3,2 |
| 31 | 23 | ±2,7 |
| 33 | 23 | ±2,7 |
| 34 | 23 | ±2,7 |
| 38 | 23 | ±2,7 |
| 40 | 23 | ±2,7 |
| 42 | 23 | +3,0/-4,0 |
| 43 | 23 | +3,0/-4,0 |
| 65 | 23 | ±2,7 |
| 68 | 23 | ±2,7 |
| NOTE: For transmission bandwidths (ETSI TS 136 521-1 [1], clause 5) confined within FUL_low and FUL_low + 4 MHz or FUL_high - 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1,5 dB (tolerance = +2,7/-4,2). |  |  |

TS 38.521-1

Table 6.2.1.5-1: Maximum Output Power test requirement for Power Class 3

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 23 | ±(2+TT) |
| n2 |  |  |  |  | 23 | ±(23+TT) |
| n3 |  |  |  |  | 23 | ±(23+TT) |
| n5 |  |  |  |  | 23 | ±(2+TT) |
| n7 |  |  |  |  | 23 | ±(23+TT) |
| n8 |  |  |  |  | 23 | ±(23+TT) |
| n12 |  |  |  |  | 23 | ±(23+TT) |
| n13 |  |  |  |  | 23 | ±(2+TT) |
| n14 |  |  |  |  | 23 | ±(2+TT) |
| n20 |  |  |  |  | 23 | ±(23+TT) |
| n24 |  |  |  |  | 23 | +2+TT/-3.03-TT |
| n25 |  |  |  |  | 23 | ±(23+TT) |
| n26 |  |  |  |  | 23 | ±(23+TT) |
| n28 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n30 |  |  |  |  | 23 | ±(2+TT) |
| n34 |  |  |  |  | 23 | ±(2+TT) |
| n38 |  |  |  |  | 23 | ±(2+TT) |
| n39 |  |  |  |  | 23 | ±(2+TT) |
| n40 |  |  |  |  | 23 | ±(2+TT) |
| n41 |  |  |  |  | 23 | ±(23+TT) |
| n48 |  |  |  |  | 23 | +2+TT/-3-TT |
| n50 |  |  |  |  | 23 | ±(2+TT) |
| n51 |  |  |  |  | 23 | ±(2+TT) |
| n53 |  |  |  |  | 23 | ±(2+TT) |
| n65 |  |  |  |  | 23 | ±(2+TT) |
| n66 |  |  |  |  | 23 | ±(2+TT) |
| n70 |  |  |  |  | 23 | ±(2+TT) |
| n71 |  |  |  |  | 23 | +2+TT/-2.5-TT |
| n74 |  |  |  |  | 23 | ±(2+TT) |
| n77 |  |  |  |  | 23 | +2+TT/-3-TT |
| n78 |  |  |  |  | 23 | +2+TT/-3-TT |
| n79 |  |  |  |  | 23 | +2+TT/-3-TT |
| n85 |  |  |  |  | 23 | ±(23+TT) |
| n91 |  |  |  |  | 23 | ±(23, 5+TT) |
| n92 |  |  |  |  | 23 | ±(23, 5+TT) |
| n93 |  |  |  |  | 23 | ±(23, 5+TT) |
| n94 |  |  |  |  | 23 | ±(23, 5+TT) |
| n100 |  |  |  |  | 23 | ±(2+TT) |
| n101 |  |  |  |  | 23 | ±(2+TT) |
| n106 |  |  |  |  | 23 | ±(2+TT) |

Table 6.2.1.5-2: Maximum Output Power test requirement for Power Class 2

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  | 26 | +2+TT/-3-TT |  |  |
| n3 |  |  | 26 | +2+TT/-33-TT |  |  |
| n8 |  |  | 26 | +2+TT/-33-TT |  |  |
| n34 |  |  | 26 | +2+TT/-3-TT |  |  |
| n39 |  |  | 26 | +2+TT/-3-TT |  |  |
| n41 |  |  | 26 | +2+TT/-33-TT |  |  |
| n77 |  |  | 26 | +2+TT/-3-TT |  |  |
| n78 |  |  | 26 | +2+TT/-3-TT |  |  |
| n79 |  |  | 26 | +2+TT/-3-TT |  |  |

Table 6.2.1.5-2a: Maximum Output Power test requirement for Power Class 1

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| n75 | 31 | +2+TT/-3-TT |  |  |  |  |
| n145 | 31 | +2+TT/-3-TT |  |  |  |  |
| n415 | 31 | +2+TT/-3-TT |  |  |  |  |
| n785 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1005 | 31 | +2+TT/-3-TT |  |  |  |  |
| n1015 | 31 | +2+TT/-3-TT |  |  |  |  |

Table 6.2.1.5-3: Test Tolerance (UE maximum output power)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

5 Reference

- RP-234078, “New WID: Non-Terrestrial Networks (NTN) for NR Phase 3”

- R4-2410574, “WF on Rel-19 phase 3 NR-NTN UE RF requirements”, Qualcomm

- R4-2413527, “Way Forward for [112][311] NR_NTN_Ph3_UE_RF”, Qualcomm

- TS 38.101-5, “NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements”

- TS 38.101-1, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- EC Decision 2016/687

- ETSI EN 301 908-13 V13.2.0 (2021-11)

- TS 38.512-1, “NR; User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone; (Release 18)”

- R4-2414287, “WF on Rel-19 FR1 NTN bands”, Inmarsat