# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415742

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Further discussion on HPUE for intra-band contiguous and non-contiguous CA

Agenda Item:	6.1.1.1.2

Document for:	Approval

Introduction

In the last meeting, RAN4 discussed MPR evaluation methodology and Pcmax,c/Pcmax issue on PC1.5 for intra-band contiguous and non-contiguous and the WF was captured in [1]. In this contribution, we will continue on the discussion on PC1.5 for intra-band contiguous and non-contiguous CA.

Discussion

MPR analysis

MPR evaluation is one of the remaining issues for PC1.5 intra-band contiguous and non-contiguous CA. The WF for MPR evaluation was captured as follows:

- 1.1 MPR evaluation methodology and assumption Online agreement: - MPR and CANS_04 A-MPR studies for PC1.5 contiguous intra-band ULCA focusses on TxD architecture and may account for PSD imbalance with up to 6dB. - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB - PC1.5 contiguous intra-band ULCA based on dualPA architecture with two LOs is not specified in R19 - MPR studies for PC1.5 non-contiguous intra-band ULCA focusses on dualPA architecture with two LOs and may account for PSD imbalance with up to 6dB. - FFS on whether to define the requirements based on equal PSD or PSD imbalance with 6dB - PC1.5 non-contiguous intra-band ULCA based on TxD architecture is not specified in R19

In the last meeting, there were MPR value proposals based on simulation and analysis. MPR simulation didn’t consider the reverse IMD introduce by multiple Tx, and some additional MPR margin was added to derive the final MPR results. MPR analysis was conducted based on existing PC 1.5 single CC MPR and PC2 single CC and contiguous UL CA MPR. The concept of derivation of PC1.5 contiguous UL CA MPR was well explained in [2] and we thought it was reasonable since we think the RF architecture extensions are similar for PC2 and PC1.5 single carrier and UL contiguous CA. PC1.5 2Tx CA MPR can be derived using PC2 2Tx CA MPR with addition of MPR offset between PC1.5 2Tx single carrier MPR and PC2 2Tx single carrier MPR, which can be shown in the following equation. This equation is quite aligned with the concept in [2].

PC1.5 2Tx CA MPR ~= PC2 2Tx CA MPR + [PC1.5 2Tx SC MPR - PC2 2Tx SC MPR]

The benefit of this analysis is that it saves a lot of simulation or measurement efforts and the results are of some degree reasonable. For now, there were no MPR measurements for PC 1.5 UL CA yet. For MPR measurements, it requires a lot of PA set up, RF parameters set up, and baseband set up before the measurements. In my understanding, only few companies can do the measurements and derive MPR results. Also, different companies may provide results with difference because of different measurement setups, which are hard to align. Therefore, we propose MPR analysis in [2] as a starting point using exiting PC1.5 single carrier and PC2 MPR data. Of course, simulation and measurement MPR results can further modify the MPR results calculated using exiting PC1 single carrier and PC2 MPR data.

Proposal 1：MPR analysis for PC 1.5 UL CA from [2] can be a starting point using exiting PC1.5 single carrier and PC2 single carrier and UL CA MPR data.

Pcmax,C and Pcmax

For Pcmax,c and Pcmax issue, we had following WF for PC 1.5 intra-band contiguous and non-contiguous CA:

## 1.2 PCMAX,C Adhoc agreement: - For intra-band UL NC CA with dualPA-architecture, the upper bound of Pcmax,c is 26dBm per CC - Whether or how to reflect the above agreement in spec is FFS - For intra-band UL contiguous CA with TxD, the upper bound of Pcmax,c is 29dBm per CC - Whether or how to reflect the above agreement in spec is FFS ## 1.3 PCMAX Online agreement with adhoc modification: - For intra-band contiguous ULCA with 2Tx-TxD, the upper bound of Pcmax is 29dBm - For intra-band UL NC CA with dual PA architecture, the upper bound of Pcmax is 29dBm

From our perspective, we think we need to capture these two agreements for Pcmax,c in TS 38.101-1. For PC1.5 intra-band UL NC CA, dualPA-architecture with two LOs are the baseline RF architecture. One CC using one PA, the upper bound of transmitted power is indeed 26dBm, which is determined by the current commercial PA capability. We think it is necessary to capture this in the spec. For PC 1.5 intra-band UL contiguous CA, single CC can achieve 29dBm using the TxD capability, which is also true. We can add this sentence in the spec.

Proposal 2: These two Pcmax,c restriction are suggested to be captured in configured output power for CA in TS 38.101-1:

- For intra-band UL NC CA with dualPA-architecture, the upper bound of Pcmax,c is 26dBm per CC

- For intra-band UL contiguous CA with TxD, the upper bound of Pcmax,c is 29dBm per CC

Conclusion

This contribution discusses MPR evaluation framework for PC1.5 intra-band contiguous and non-contiguous CA. The following proposals are made:

Proposal 1：MPR analysis for PC 1.5 UL CA can be a starting point using exiting PC1.5 single carrier and PC2 single carrier and UL CA MPR data.

Proposal 2: These two Pcmax,c restriction are suggested to be captured in configured output power for CA in TS 38.101-1:

- For intra-band UL NC CA with dualPA-architecture, the upper bound of Pcmax,c is 26dBm per CC

- For intra-band UL contiguous CA with TxD, the upper bound of Pcmax,c is 29dBm per CC

- References

[1] R4-2414277, WF on HPUE for CA in TN, Samsung, RAN4#112.

[2] R4-2411168, MPR for PC1.5 intra-band contiguous UL CA, Apple, RAN4#112.