# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415743

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Further discussion on 6Rx REFSENS requirements for FWA and Handheld UE

Agenda Item:	6.1.1.3.1

Document for:	Approval

Introduction

In the last meeting, we discussed REFSENS requirements and ΔRIB,6R values for 6Rx and the WF was captured in [1]. 6Rx REFSENS ΔRIB,6R were defined differently for handheld and FWA UE and the values are averaged based on companies’ input. In this contribution, we further discuss the remaining issues for 6Rx REFSENS requirements.

Discussion

The WF for 6Rx REFSENS requirements in RAN4#112 meeting were captured as follows:

Issue 1-1-1: Whether band n104 should be included in the high band (n77, n78 and n79) category for 6Rx case Agreement: Include n104 in the high band (n77, n78, n79) category. Issue 1-1-2: Whether to use same ΔRIB,6R value for handheld UE and FWA Agreement: Different value for handheld UE and FWA. Issue 1-1-3: Verification of 6Rx receiver requirements Way Forward: Companies encouraged to provide their views on the ZTE proposal in Proposal 5 in R4-2411883 and any further positions on verification of 6Rx receiver requirements at future meetings given that ΔRIB,6R values are now defined. Issue 1-2-1: Proposed ΔRIB,6R values for handheld UE and FWA Agreement: Based on the outcome of Issue 1-1-2, RAN4 to specify ΔRIB,6R values for handheld UE and FWA as follows. - Adopt the average value of ΔRIB,6R for handheld UE from company proposals (updated to include company proposals from RAN4#111) and specify it for handheld UE devices. Table: Averaged values for handheld UE based on proposals Operating Bands; ΔRIB,6R (dB). Average; Band n41; -3.4. Band n77, n78, n79, n104; -3.0. - Adopt the average value of ΔRIB,6R for FWA UE from company proposals indicating different FWA UE values from handheld UE values and specify it for FWA UE devices. Table: Averaged values for FWA UE based on proposals Operating Bands; ΔRIB,6R (dB). Average; Band n41; -3.7. Band n77, n78, n79, n104; -3.3.

It can be observed from the WF that most of issues were agreed and only one issue on verification of 6Rx receiver requirements needs further discussion.

From our perspective, we support the proposal on verification of 6Rx receiver requirements from [2], that is

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands.

According to current TS 38.101-1, the verification of 2Rx/4Rx/8Rx REFSENS and other Rx requirements were defined as follows:

For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, and additional requirements for four and eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports.

For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports, the UE shall be verified with eight Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands where the UE is equipped with eight Rx antenna ports unless UE is not supporting eight Rx ports for band(s) in band combination in which case those band(s) shall be verified with four Rx antenna ports in that band combination, otherwise, the UE shall be verified with two Rx antenna ports.

Table 2 from [2] can precisely summarize above requirements. That is for single carrier REFSENS requirements, we need to verify all the Rx capabilities without skipping. For example, if a band support 8Rx REFSENS requirements, it not only needs to meet the 8Rx REFSENS, but also 4Rx and 2Rx REFSENS. However, for other Rx requirements, it only needs to meet the highest Rx capabilities, that is 8Rx in this case, and skips both 2Rx and 4Rx requirements. We think this rule can be extended to 6Rx since it saves a lot of effort to verify 6Rx for other single carrier Rx requirements.

Table 2: diversity characteristics for 2Rx/4Rx/8Rx

|  | Single carrier REFSEN requirement | Other single carrier Rx requirements |
| --- | --- | --- |
| UE is equipped with 2Rx | 2Rx | 2Rx |
| UE is equipped with 4Rx | 2Rx, 4Rx | 4Rx, skip 2Rx |
| UE is equipped with 6Rx | 2Rx, 4Rx, 6Rx | 6Rx, skip 2Rx and 4Rx |
| UE is equipped with 8Rx | 2Rx, 4Rx, 6Rx, 8Rx | 8Rx, skip both 2Rx and 4Rx, 6Rx |

Proposal 1: Support the following rule for verification of 6Rx requirements and reflect it in TS 38.101-1:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands.

The potential changes to TS 38.101 can be:

For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, additional requirements for four and six Rx ports shall be verified in operating bands where the UE is equipped with six Rx antenna ports and additional requirements for four and eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports.

For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands where the UE is equipped with six Rx antenna ports, the UE shall be verified with eight Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands where the UE is equipped with eight Rx antenna ports unless UE is not supporting eight Rx ports for band(s) in band combination in which case those band(s) shall be verified with four Rx antenna ports in that band combination, otherwise, the UE shall be verified with two Rx antenna ports.

For 6Rx REFSENS requirements, they can be captured in TS 38.101-1:

For UE(s) equipped with 6 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,8R in Table 7.3.2-2c for the applicable operating bands.

Table 7.3.2-2c: Six antenna port reference sensitivity allowance ΔRIB,6R

| Operating band | ΔRIB,6R (dB) for handheld | ΔRIB,6R (dB) for FWA |
| --- | --- | --- |
| n41 | -3.4 | -3.7 |
| n77, n78, n79,n104 | -3.0 | -4.0 |

Conclusion

This contribution discusses 6Rx REFSENS requirements for both handheld and FWA UEs. The following proposals are made:

Proposal 1: Support the following rule for verification of 6Rx requirements:

- For single carrier REFSEN requirements, additional requirement for four Rx antenna ports and six Rx antenna ports shall be verified in operating bands;

- For Rx requirements other than single carrier REFSEN, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands.

Proposal 2: The changes for TS 38.101-1 for 6Rx REFSENS requirements are needed:

Change 1:

For the single carrier REFSENS requirements in Clause 7, the UE shall be verified with two Rx antenna ports in all supported frequency bands, additional requirements for four Rx ports shall be verified in operating bands where the UE is equipped with four Rx antenna ports, additional requirements for four and six Rx ports shall be verified in operating bands where the UE is equipped with six Rx antenna ports and additional requirements for four and eight Rx ports shall be verified in operating bands where the UE is equipped with eight Rx antenna ports.

For Rx requirements other than single carrier REFSENS in Clause 7, the UE shall be verified with four Rx antenna ports and skip two Rx antenna ports requirements in operating bands where the UE is equipped with four Rx antenna ports, the UE shall be verified with six Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands where the UE is equipped with six Rx antenna ports, the UE shall be verified with eight Rx antenna ports and skip both two and four Rx antenna ports requirements in operating bands where the UE is equipped with eight Rx antenna ports unless UE is not supporting eight Rx ports for band(s) in band combination in which case those band(s) shall be verified with four Rx antenna ports in that band combination, otherwise, the UE shall be verified with two Rx antenna ports.

Change 2:

For UE(s) equipped with 6 Rx antenna ports, reference sensitivity for 2Rx antenna ports in Table 7.3.2-1a and in Table 7.3.2-1b shall be modified by the amount given in ΔRIB,8R in Table 7.3.2-2c for the applicable operating bands.

Table 7.3.2-2c: Six antenna port reference sensitivity allowance ΔRIB,6R

| Operating band | ΔRIB,6R (dB) for handheld | ΔRIB,6R (dB) for FWA |
| --- | --- | --- |
| n41 | -3.4 | -3.7 |
| n77, n78, n79, n104 | -3.0 | -4.0 |

- References

[1] R4-2414285, WF on 6Rx UE requirements, AT&T, RAN4#112.

[2] R4-2411883, Further discussion on 6Rx Reference sensitivity requirements, ZTE Corporation, Sanechips, RAN4#112.