# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415745

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Further discussion on SRS antenna switching requirements

Agenda Item:	6.1.1.3.3

Document for:	Approval

Introduction

SRS antenna switching requirements are one of important aspects for 6Rx requirements. In the last meeting, we averaged ΔTRxSRS values based on companies’ input, however, there was no consensus on whether to introduce an additional point for band n104. The WFs are captured in [1]. In this contribution, we will further discuss the SRS antenna switching requirements.

Discussion

The WF for SRS antenna switching and ΔTRxSRS of last meeting was captured as follows:

- Sub-topic 2-1: General considerations for SRS antenna switching and ΔTRxSRS Issue 2-1-1: Whether to consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Way forward: Further discuss the following options. Option 1: Do not consider an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 Option 2: Consider additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 - Sub-topic 2-2: ΔTRxSRS values Issue 2-2-1: Proposed ∆TRxSRS values Way Forward: RAN4 agrees to adopt the average values for Bands n41, n77, and n78. For Band n79 and n104, further analysis is needed. Take Option 10 and option 11 as the starting point, not precluding any other numbers. - Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS for bands whose FUL_high is lower than the FUL_low of n79. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n41, n77, n78; 4.0; 3.5; 4.5. - Option 10: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS without considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79, n104; 5.4; 4.7; 5.9. - Option 11: Adopt the average values from companies (updated to include company proposals from RAN4#111) as summarized below and specify the values for ΔTRxSRS considering an additional breakpoint for bands whose FUL_high is lower than the FUL_low of n104. Table: Averaged values based on proposals Operating Bands; ΔTRxSRS t1r6 (dB); ΔTRxSRS t2r6 (dB); ΔTRxSRS t1r6-t2r6 (dB). Average; Band n79; 5.4; 4.7; 5.9. Band n104; 5.7; 5.0; 6.3.

For ΔTRxSRS requirements, we suggest not to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104. The insertion loss of RF switch and additional SRS trace mainly contributes to ΔTRxSRS requirements. Although the frequency of n104 is higher than n79, the IL of these two aspects is of little difference. We investigated some commercial RF switch whose operating frequency covers band n79 and n104, the IL difference is very small, 0.1~0.2 dB at most. For additional SRS trace, the IL difference for band n79 and n104 is estimated as 1.06dB. When considering the ΔTRxSRS requirements for band n79, we already consider additional margin for the IL of RF switch and additional SRS trace and large implementation margin and we think this can cover the IL difference between band n79 and n104.

For ∆TRxSRS values, they were agreed for band n77, n78, n79 for t1r6, t2r6 and t1r6-t2r6. No more discussion is needed. For band n79, n104, we prefer to use Option 10 as the ∆TRxSRS requirement since we think there is no need to introduce an additional point for band n104. For Option 11, it can be observed that the ∆TRxSRS values are pretty close, the difference is 0.3dB~0.4dB. We don’t need to introduce the breakpoint for n104 to make the spec more complex. One compromise we can think of is use more relaxed values for band n79 and n104 and not to introduce an additional breakpoint for band n104 as shown in the following table.

Table: Averaged values based on proposals

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79 | 5.4 | 4.7 | 5.9 |
|  | Band n79, n104 | 5.7 | 5.0 | 6.3 |

Proposal 1: For ΔTRxSRS requirements, no need to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 and define ΔTRxSRS requirements for 6Rx as follows:

Table: Averaged values based on proposals (Option 10 from the WF)

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79, n104 | 5.4 | 4.7 | 5.9 |

Or

Table: Averaged values based on proposals (Compromise)

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79 | 5.4 | 4.7 | 5.9 |
|  | Band n79, n104 | 5.7 | 5.0 | 6.3 |

For SRS antenna switching configurations, there was one update for t3r6 SRS antenna switching for R19 MIMO WID in RAN#105:

RAN#105 Conclusion

- Item A: SRS port grouping for TDD low-complexity 6/8RX receiver

- Item B: 2TA without CoresetPoolIdx association for asymmetric DL sTRP/UL mTRP

- Item C: 3T6R SRS antenna switching

- Item D: UE capability for 3T3R SRS antenna switching

- Item G: UE capability for non-codebook-based UL transmission

Update the Rel-19 MIMO WID accordingly.

It can be observed that RAN1 will specify 3T6R SRS antenna switching. In RAN4, we can start the work on ΔTRxSRS requirements with the addition of 3tr6.

Proposal 2: RAN4 can start the work on ΔTRxSRS requirements with the addition of 3tr6.

Conclusion

This contribution discusses ΔTRxSRS requirements for 6Rx considering SRS antenna switching configurations t1r6, t2r6 and t1r6-t2r6. The following proposal is made:

Proposal 1: For ΔTRxSRS requirements, no need to introduce an additional breakpoint for bands whose FUL_high is higher than the FUL_low of n104 and define ΔTRxSRS requirements for 6Rx as follows:

Table: Averaged values based on proposals (Option 10 from the WF)

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79, n104 | 5.4 | 4.7 | 5.9 |

Or

Table: Averaged values based on proposals (Compromise)

|  | Operating Bands | ΔTRxSRS t1r6 (dB) | ΔTRxSRS t2r6 (dB) | ΔTRxSRS t1r6-t2r6 (dB) |
| --- | --- | --- | --- | --- |
| Average | Band n79 | 5.4 | 4.7 | 5.9 |
|  | Band n79, n104 | 5.7 | 5.0 | 6.3 |

Proposal 2: RAN4 can start the work on ΔTRxSRS requirements with the addition of 3tr6.

- References

[1] R4-2410751, WF on requirements for 6Rx, AT&T, RAN4#111.

[2] R4-2414285, WF on 6Rx UE requirements, AT&T, RAN4#112.