# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415746

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on system parameters for less than 5MHz in FR1-NTN bands

Agenda Item:	6.8.3.1

Document for:	Approval

Introduction

In the last meeting, we started the discussion on system parameters for less than 5MHz in FR1-NTN bands. For most of the requirements, they were defined the same way with 3MHz in FR1 TN bands. In this contribution, we further discuss the remaining issues of system parameters for less than 5MHz for NTN.

Discussion

According to the WF [1], the remaining issues for less than 5MHz for NTN are captured as follows:

System parameters

Issue 1-1-6: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

- FFS

Proposal： Optional and release independent from R19.

In FR1 NR TN, 3MHz was defined as an optional feature and release independent from Rel-18. We can still follow the same logic for FR1-NTN.

Table 5.1-3: NR channel bandwidths

| Feature | Duplex-mode | Release independent from | Requirements to be fulfilled (see TS 38.307 of the release in which the channel bandwidth was introduced) |
| --- | --- | --- | --- |
| NR channel bandwidths, all unless otherwise stated | FDD, TDD, SDL, SUL | Rel-15 | Table B.4.1-1, Table B.4.3-1 |
| Channel bandwidth of 3 MHz | FDD | Rel-18 | Table B.4.1-1 |

Usually, a new bandwidth is introduced in later releases and it is optional from the release firstly introduced. In FR1-NTN, we suggest to introduce 3MHz as an optional feature and release independent from Rel-19. Support of 3MHz requires additional baseband capability, which may need modification on modem and RF to support this feature. In this sense, we prefer 3MHz as an optional feature.

Proposal 1: Channel bandwidth 3MHz for FR1-NTN is an optional feature and release independent from Rel-19.

Issue 1-2-6: Signalling of SS raster

- Proposals

  - Option 1: Use the Rel-18 UE capabilities support 3MHz-ChannelBW-Symmetric-r18 and support 3MHz-ChannelBW-Asymmetric-r18 for NTN communications (Huawei)

  - Option 2: Others

- Recommended WF

  - TBD

In TN 3MHz, three UE capabilities were introduced, that is support12PRB-CORESET0-r18, support3MHz-ChannelBW-Symmetric-r18, support3MHz-ChannelBW-Asymmetric-r18. We haven’t fully discussed these UE capabilities. In TN 3MHz, UE can still support the reception of 12 PRB with 15PRB transmission bandwidth. In NTN 3MHz, the UE capability for this issue is till open. For asymmetric channel bandwidth, some satellite commented they didn’t need asymmetric channel bandwidth in band n254. There was no clear demand for asymmetric channel bandwidth for FR1-NTN bands. From our perspective, the UE capabilities discussion can defer after all the requirements are clear.

Proposal 2: Defer the UE capability discussion for 3MHz in FR1-NTN after all the requirements are clear.

Issue 1-3-1: Whether 3MHz channel bandwidth applies to Redcap/eRedcap NTN

- FFS

From our perspective, RedCap or eRedCap NTN supporting 3MHz channel bandwidth is out of scope of this WID. In the history of 3MHz for TN, it was not even supported for RedCap or eRedCap. The WID upscoping of Redcap/eRedcap NTN 3MHz needs confirmation in RAN Plenary.

Proposal 3: Whether Redcap/eRedcap NTN supports 3MHz channel bandwidth needs confirmation in RAN Plenary.

Conclusion

This contribution gives our initial views on system parameters for FR1-NTN supporting less than 5MHz channel bandwidth. The following proposal is made:

Proposal 1: Channel bandwidth 3MHz for FR1-NTN is an optional feature and release independent from Rel-19.

Proposal 2: Defer the UE capability discussion for 3MHz in FR1-NTN after all the requirements are clear.

Proposal 3: Whether Redcap/eRedcap NTN supports 3MHz channel bandwidth needs confirmation in RAN Plenary.

- References

[1] R4-2413528, WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4#112.