# 3GPP TSG-RAN WG4 Meeting #112bis	R4-2415748

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on the subband configurations and guardbands for gNB SBFD

Agenda Item:	6.20.1

Document for:	Approval

Introduction

In the last meeting, we further discussed system parameters for gNB SBFD. The WF for gNB SBFD was captured in [1]. In this contribution, we focus on subband configurations and guard band issues for gNB SBFD.

Discussion

In the last meeting, we discussed operating bands and channel bandwidth support for SBFD operation, however, there was no agreement on this issue. The consensus is that, all TDD bands for FR1 and FR2-1 are potentially to be used for SBFD operations with some restrictions. One open issue is the restrictions put on the supported channel bandwidth for SBFD. The agreements are captured as follows:

Issue 1-2-1: Band/channel bandwidth support for SBFD - Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = [20] - For next meeting, we will further discuss the potential to increase the value of X beyond X = [20]. Companies are encouraged to bring t-docs to discuss the benefits or drawback to increase this value. - FFS for some specific band if this principle rules out this band, which is required for SBFD by operators. - FFS how to implement in the spec: - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1. - Option 2: Add SBFD entry in Table 5.2-1 in Clause 5.2 in TS 38.104 for the TDD bands with channel bandwidth >= X MHz.

From our perspective, we prefer the supported channel bandwidth shall be no less than X=20 MHz. The benefit of using this number is that it rules out few TDD bands with rather small channel bandwidth (e.g. n34, n51, n53, n54, n101), which are expectedly there is no performance gain to deploy SBFD. Also, it allows the flexibility of most of TDD bands to deploy SBFD.  We suggest at least no less than 20MHz supported channel bandwidth can be a starting point.

One concern of using X=20MHz, is that if it can still achieve the required RSIC to suppress the self-interference within SBFD capable BS. After all, we only considered 100MHz channel bandwidth in the SBFD feasibility study. When we use a smaller X MHz channel bandwidth, the RSIC may vary and we need further check if the required RSIC can still be achievable especially for FR1 WA and MR BS since the conclusion of these two types are not that optimistic. I think this requires further work if we choose X=20 MHz.

If the X MHz channel bandwidths restriction rules out some TDD bands, we think it is OK. If they are required by the operator, firstly, we need to verify the benefits of deploying SBFD in TDD bands with channel bandwidth less than X MHz. If there is benefit, we add band request from the operator and further modify the value of X MHz. However, with X=20MHz, it is not likely for operator to request TDD bands with smaller than 20MHz to deploy SBFD.

As for how to implement the restriction in the spec, we prefer Option 1 since it is straightforward and easy to implement.

        - Option 1: To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to TDD bands with channel bandwidth >= X MHz, given in Table 5.2-1.

Proposal 1：Channel bandwidth no less than 20MHz for SBFD can be a starting and further check if RSIC can be achievable for 20 MHz channel bandwidth especially for FR1 WA and MR BS.

Issue 1-2-4: DL/UL subband configurations support for SBFD - Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB - FFS how to implement BS RF requirements based on the above options for different levels of flexibility. - BS transmission configuration in BS RF requirements (for information only): Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1 SCS (kHz); 3 MHz; 5 MHz; 10 MHz; 15 MHz; 20 MHz; 25 MHz; 30 MHz; 35 MHz; 40 MHz; 45 MHz; 50 MHz; 60 MHz; 70 MHz; 80 MHz; 90 MHz; 100 MHz. NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB; NRB. 15; 15; 25; 52; 79; 106; 133; 160; 188; 216; 242; 270; N/A; N/A; N/A; N/A; N/A. 30; N/A; 11; 24; 38; 51; 65; 78; 92; 106; 119; 133; 162; 189; 217; 245; 273. 60; N/A; N/A; 11; 18; 24; 31; 38; 44; 51; 58; 65; 79; 93; 107; 121; 135. Table 5.3.2-2: Transmission bandwidth configuration NRB for FR2-1 SCS (kHz); 50 MHz; 100 MHz; 200 MHz; 400 MHz. NRB; NRB; NRB; NRB. 60; 66; 132; 264; N/A. 120; 32; 66; 132; 264.

Of all these options for subband configurations, we think Option 2b is a good compromise. With Option 1, it only considers 100MHz BS channel bandwidth with DUD and DU configuration. From our standpoint, it restricts the supported band width to be only 100MHz. It rules most of TDD bandwidth without 100MHz BS channel bandwidth. We think it is too restricted. With Option 3 the highest flexibility, the test burden would be high which is not necessary. With middle flexibility Option 2, we think Option 2b can be a good starting point and it also meets the existing BS conformance testing custom.

Proposal 2：Option 2b (UL subband shall be selected from BS transmission channel bandwidth) can be used as a starting point for subband configuration.

Issue 1-2-5: Restriction on guardband size for SBFD - Agreement: - Restriction on guardband size for SBFD: - Option 1: Definition based on existing guardbands requirement in TS38.104 (i.e., minimum guardband) - Option 2: Typical values of guardband defined for requirement, but the guardband should not be restricted to those in RAN4 specifications. - Option 3: No restriction (manufacturer declaration-based), but with restriction on maximum allowed number of RBs for guardband: - Option 3a: No restriction (manufacturer declaration-based), and no restriction on maximum/minimum number of RBs for guandband:

For guardband issue, it is related to the SBFD configuration. If we go with Option 2b as the SBFD configuration, that is to select UL subband from the existing BS transmission channel bandwidth, DL subband and the guardband are declare based, we think it is better to put no restriction on the GB size, that is Option 3a.

Proposal 3: No restriction on maximum/minimum guardband size if UL subband size shall be selected from BS transmission channel bandwidth and DL subband size shall be declare based.

Conclusion

This contribution discusses SBFD configurations and guardbands for gNB SBFD. The following proposals are made:

Proposal 1：Channel bandwidth no less than 20MHz for SBFD can be a starting and further check if RSIC can be achievable for 20 MHz channel bandwidth especially for FR1 WA and MR BS.

Proposal 2：Option 2b (UL subband shall be selected from BS transmission channel bandwidth) can be used as a starting point for subband configuration.

Proposal 3: No restriction on maximum/minimum guardband size if UL subband size shall be selected from BS transmission channel bandwidth and DL subband size shall be declare based.

- References

[1] R4-2413514, Way Forward for [112][307] NR_duplex_evo_General, Samsung, RAN4#112.