3GPP TSG-RAN4 Meeting #112bis	R4-2415756

# Hefei, China, Oct 11 – 18, 2024

Agenda item:			6.1.1.2.3

Source:	vivo

Title:	Discussion on MPR applicability for FR2

Document for:	Discussion

- Introduction

In RAN#112 meeting, the WF[1] has been agreed, and the remaining issue is as follows:

### Sub-topic 3-1: Scope and cases considered for FR2 MPR enhancements #### Issue 3-1-1: Whether new CA MPR for CABW < 400MHz should be considered in the WI - Agreement in main session: - Hold on discussions on whether new MPR requirement is defined for CABW < 400MHz unless the WID can be updated accordingly. - Only MPR applicability needs to be discussed in this WID, and defining new MPR requirement is out of scope. #### Issue 3-1-2: whether FR2 MPR enhancement for the single UL case is also applicable for intra-band DL non-contiguous CA with single UL - Agreement in AH: - FR2 MPR enhancement for the single UL case is not applicable for intra-band DL non-contiguous CA with single UL unless the WID is revised #### Issue 3-1-3: Power classes considered for FR2 MPR enhancement - Agreement in main session: - All FR2 power classes could be considered for the MPR enhancement #### Issue 3-1-4: sub-FR2 frequency ranges - Agreement in main session: - MPR reduction applies to both FR2-1 and FR2-2. - Sub-topic 3-2: Applicable MPR #### Issue 3-2-1: Applicable MPR for FR2 single carrier UL with DL intra band CA - Proposals - Proposal 1: For the case of DL intra band CA with single UL CC (activated or configured), the reference MPR (originally being Table 6.2A.2.4-1) is changed as the MPR requirement for single carrier. (Huawei) - WF - FFS in next meeting. #### Issue 3-2-2: Applicable MPR for FR2 UL CA with DL intra band CA - WF - In general, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW) - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused - No need to specify new column for <400MHz CABW in the MPR table. FFS specific spec change to accommodate 200MHz BW granularity - FFS whether to take the minimum values of MPR between the intra-band CA case and the single carrier for FR2-2 - Sub-topic 3-3: UE capability #### Issue 3-3-1: Whether an optional UE capability is considered - Proposals - Proposal 1: no need to introduce a new UE capability for such MPR improvement as Network does not need to enable the MPR improvement originating from the application of MPR requirements. (CATT) - Proposal 2: The new capability shall be defined as optional and per UE, and only applicable to FR2. In addition, it could be release independent. (ZTE) - Proposal 3: RAN4 to consider as an optional capability. (Qualcomm) - Proposal 4: Subject to UE capability indication on the support of Rel-19 MPR applicability for FR2 CA. (Huawei) - WF - FFS in next meeting. #### Issue 3-3-2: Whether UE capability, if needed, is agnostic to specific LO implementations - Proposals - Proposal 1: Consider all possible UE implementations, including UE with fast LO switching, or with dedicated LOs for Tx and Rx paths for the new UE capability. (ZTE) - Proposal 2: Capability should not be dependent on a specific implementation. (NTT DOCOMO) - WF - FFS in next meeting #### Issue 3-3-3: Proposal for CC activation-based MPR improvement - Proposals - Proposal 1: FR2 UE CA MPR is based on the activated subset of the CCs in the CA configuration that are also enabled for UL, provided the activated CCs with UL comprise a contiguous block. (Qualcomm) - Recommended WF - FFS in next meeting

- Discussion

In the last meeting, it was agreed that MPR would be based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW), however the BW change should be on the premise that UE support the capability of LO decoupling for UL and DL, e.g., fast LO switching or dedicated LOs for Tx and Rx chains.

Proposal 1：In general, with UE indication of new capability [FR2 MPR enhancements-R19] for MPR improvement, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW).

- If only 1 UL CC is activated, the MPR requirements of single carrier could be reused.

Proposal 2: A note could be added to the MPR table, the following is one example and the change should be applied to all the FR2-1 & FR2-2 intra-band C CA MPR tables with all power classes:

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-1

| Waveform Type |  | Cumulative aggregated channel bandwidth3 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.51 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.51 | ≤ 8.7 | ≤ 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: (Void) NOTE 2: Refer to clause 6.1 for 256 QAM applicability. NOTE 3: When UE support [FR2 MPR enhancements-R19]，the CABW is replaced by UL aggregated BW |  |  |  |  |  |

- Conclusion

Proposal 1：In general, with UE indication of new capability [FR2 MPR enhancements-R19] for MPR improvement, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW).

- If only 1 UL CC is activated, the MPR requirements of single carrier could be reused.

Proposal 2：A note should be added to the MPR table, the following is one example and the change should be applied to all the FR2-1 & FR2-2 intra-band C CA MPR tables with all power classes:

Table 6.2A.2.2-1: Maximum power reduction (MPRWT_C_CA) for UE power class 1 in FR2-1

| Waveform Type |  | Cumulative aggregated channel bandwidth3 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | < 400 MHz | ≥ 400 MHz and < 800 MHz | ≥ 800 MHz and ≤ 1400 MHz | > 1400 MHz and ≤ 2400 MHz |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 5.51 | ≤ 7.7 | ≤ 8.2 | ≤ 8.7 |
|  | QPSK | ≤ 6.51 | ≤ 8.7 | ≤ 9.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 9.2 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 16 QAM | ≤ 6.5 | ≤ 8.7 | ≤ 8.7 | ≤ 9.7 |
|  | 64 QAM | ≤ 9.0 | ≤ 10.7 | ≤ 11.2 | ≤ 11.7 |
|  | 256 QAM2 | ≤ 12.5 | ≤ 14.2 | ≤ 14.7 | ≤ 15.7 |
| NOTE 1: (Void) NOTE 2: Refer to clause 6.1 for 256 QAM applicability. NOTE 3: When UE support [FR2 MPR enhancements-R19]，the CABW is replaced by UL aggregated BW |  |  |  |  |  |

- References

- R4-2414441, WF on power domain enhancements, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #112.