3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415760

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on the FR2 sTxMP OTA testing

Agenda Item:	6.13.2

Document for:	Approval

- Introduction

- Agreement

  - Take below proposal as the starting point:

    - For the selection of AoA pair, one of AoA is at the beam peak direction of single CC operation without STxMP enabled (i.e., from Beam Peak Search for MOP in 6.2 of TS 38.101-2, the other AoA is at the direction decided by UE declared orientation (listed in TS38.101-2) and AoA pair selecting from set of {30deg, 60deg, 90deg, 120deg, 150deg}.

      - FFS whether a test direction other than the beam peak direction of single CC operation without STxMP enabled can be used for first AoA

    - Due to lack of absolute probe location definition of the multi-AoA UE RF test system, defined in Clause 5.2.2 of TR38.871, FFS on whether or how such test system can guarantee the same relative angular orientations between specific UE test directions and probes.

In the last meeting, the following approach is approved as starting point of the sTxMP test method:

In this contribution, provide more analysis on the applicability for this approach.

- Discussion

  - Absolute location of the probe

In R18, this issue actually discussed and captured in [2] (section 5.3.1) but doesn’t have clear conclusion in the end. In MIMO OTA, P0 was defined to avoid the ambiguity [3]:

In order to avoid positioning ambiguities, the turntable implementing the rotation in q shall match the intended DUT q for P0 Orientation 1 without the re-positioning approach, as defined in Annex A.3 in [2], applied. With the re-positioning approach applied, the relative orientation between the DUT and the probes for P0 Orientation 2 shall be the same the relative orientation between DUT and probes as for P0 Orientation 1.

In our understanding, the definition above also works for multi-Rx/Tx test system.

Proposal 1: Same P0 definition in MIMO OTA can be used in multi-Rx/Tx test system.

  - EIRPmax

In [4], we provide the simulation results to validate the agreed test method for min peak EIRP, but for EIRPmax, whether this method still works need more verification. Unlike min peak EIRP, the probes do not need to differentiate beams when testing EIRPmax, and each probe need to measure the aggregated power of both beams. Table I shows the simulation results for EIRPmax when panels are set in the same side and opposite side:

Table I simulation results for EIRPmax

|  | AoA offset | AoA pair of aggregated EIRPmax across all tested beam pair | EIRPmax | beam peak direction when only one panel is activated |
| --- | --- | --- | --- | --- |
| Panels in same side | 30° | [270,105] [270,75] | 29.22 | [270,105] |
|  | 60° | [270,105] [270,45] | 29.22 |  |
|  | 90° | [270,105] [270,15] | 29.22 |  |
|  | 120° | [270,105] [90,15] | 29.22 |  |
|  | 150° | [270,105] [90,45] | 29.22 |  |
| Panels in opposite side | 30° | [90,105] [90,135] | 26.97 | [90,105] |
|  | 60° | [90,95] [90,155] | 26.94 |  |
|  | 90° | [90,95] [270,175] | 26.94 |  |
|  | 120° | [90,95] [270,145] | 26.94 |  |
|  | 150° | [90,95] [270,115] | 26.94 |  |

In the simulation, each panel has 7 analog beams, and the procedure to find EIRPmax actually is to find the beam pair that can lead to the highest aggregated EIRP. When panels are located in the same side, the results shows that when AoA offset changes, the EIRPmax keeps the same. This is because the beam pair to maximize the aggregated EIRP is not changed. As a comparison, we can observe that the EIRPmax beam pair changed when panel in the opposite side.

Based on the simulation results, we can find that the AoA pair for EIRP always contain the beam peak when only the single panel is activated, which actually is same as the beam peak direction from beam peak search for MOP. However, AoA offset to reach EIRPmax may not same as the AoA offset in min peak EIRP which is declared by UE. For example, when panels are in the opposite side, the AoA offset to get EIRPmax is 30° but for min peak EIRP this case should be 150°.

Observation 1: The AoA pair that can reach EIRP contain the beam peak direction of single CC operation, but the required AoA offset may be different from the min peak EIRP test.

To make the test more convincible, a possible solution is that test all candidate AoA offset during EIRPmax verification.

Proposal 2: For EIRPmax test, one of AoA is at the beam peak direction of single CC operation without STxMP enabled, the other AoA is at the direction decided by UE declared orientation (listed in TS38.101-2) and all AoA pair from set of {30deg, 60deg, 90deg, 120deg, 150deg}.

- Conclusion

In this contribution, we provide our view on NTN HPUE.

Proposal 1: Same P0 definition in MIMO OTA can be used in multi-Rx/Tx test system.

Observation 1: The AoA pair that can reach EIRP contain the beam peak direction of single CC operation, but the required AoA offset may be different from the min peak EIRP test.

Proposal 2: For EIRPmax test, one of AoA is at the beam peak direction of single CC operation without STxMP enabled, the other AoA is at the direction decided by UE declared orientation (listed in TS38.101-2) and all AoA pair from set of {30deg, 60deg, 90deg, 120deg, 150deg}.

- References

- R4-2413534 WF on Rel-19 NR_FR2_OTA, RAN4#112, Qualcomm

- 3GPP TR 38.871

- 3GPP TR 38.827