3GPP TSG-RAN WG4 Meeting # 112 	R4-2415761

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Co-existence evaluation for 14800 to 15350 MHz frequency range

Agenda Item:	6.2.4.1

Document for:	Approval

- Introduction

In [1], the co-existence assumption for 14800 to 15350 MHz was agreed and in the last meeting, some UE parameters are updated. In this contribution, we provide our evaluation results and analysis.

- Discussion

  - Comparison for different BS antenna

In [1], different BS antenna elements are assumed, and the Table I show the results under different assumption. The ISD is 450m and the UE power is 23dBm.

Table I Co-existence results under different BS antenna assumption

| Urban macro |  | ACIR (UL average throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS antenna | UE antenna | 10 | 15 | 20 | 25 | 30 |
| 64*24 | FR1 like | 0.8 | 0.38 | 0.18 | 0.08 | 0.04 |
|  | FR2 like | 0.46 | 0.23 | 0.11 | 0.05 | 0.02 |
| 64*32 | FR1 like | 0.56 | 0.29 | 0.15 | 0.06 | 0.03 |
|  | FR2 like | 0.36 | 0.19 | 0.07 | 0.03 | 0.01 |

|  |  | ACIR (UL 5% cell edge throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS antenna | UE antenna | 10 | 15 | 20 | 25 | 30 |
| 64*24 | FR1 like | 5.77 | 2.82 | 0.93 | 0.14 | 0 |
|  | FR2 like | 2.84 | 1.07 | 0.22 | 0.13 | 0 |
| 64*32 | FR1 like | 5.73 | 1.01 | 0.56 | 0.28 | 0 |
|  | FR2 like | 3.13 | 1.97 | 0.02 | 0 | 0 |

|  |  | ACIR (DL average throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS antenna | UE antenna | 10 | 15 | 20 | 25 | 30 |
| 64*24 | FR1 like | 3.11 | 1.79 | 0.91 | 0.46 | 0.24 |
|  | FR2 like | 1.91 | 1.12 | 0.57 | 0.29 | 0.15 |
| 64*32 | FR1 like | 2.56 | 1.32 | 0.71 | 0.39 | 0.19 |
|  | FR2 like | 1.49 | 0.81 | 0.46 | 0.22 | 0.09 |

|  |  | ACIR (UL 5% cell edge throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS antenna | UE antenna | 10 | 15 | 20 | 25 | 30 |
| 64*24 | FR1 like | 17.26 | 10.79 | 5.12 | 2.17 | 1.23 |
|  | FR2 like | 13.66 | 8.19 | 3.72 | 1.86 | 0.39 |
| 64*32 | FR1 like | 16.03 | 8.69 | 3.8 | 2.43 | 0.94 |
|  | FR2 like | 11.48 | 5.3 | 3.42 | 1.46 | 0.87 |

The results for different BS elements are similar and the interference of 64*32 is a little bit lower due to the better spatial isolation, so 64*24 can be the worst case used in co-existence study. Similar observation can also find in Indoor scenario for 8*8 and 4*4

Proposal 1: For BS antenna assumption, taking 64*24 for Uma and 4*4 for Indoor as the worst case.

  - Comparison for different UE Tx power

For UE Tx power, both 23 dBm and 26 dBm are candidate option. The simulation results are listed in the Table below. The BS antenna assumption is 64*24 and the ISD is 450m

Table II Co-existence results under different UE Tx power

| Urban macro |  | ACIR (UL average throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE antenna | UE Tx power | 10 | 15 | 20 | 25 | 30 |
| FR1 like | 23 | 0.8 | 0.38 | 0.18 | 0.08 | 0.04 |
|  | 26 | 0.72 | 0.33 | 0.16 | 0.07 | 0.03 |
| FR2 like | 23 | 0.46 | 0.23 | 0.11 | 0.05 | 0.02 |
|  | 26 | 0.4 | 0.22 | 0.11 | 0.05 | 0.03 |

| Urban macro |  | ACIR (UL 5% cell edge throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| UE antenna | UE Tx power | 10 | 15 | 20 | 25 | 30 |
| FR1 like | 23 | 5.77 | 2.82 | 0.93 | 0.14 | 0 |
|  | 26 | 5.5 | 2.34 | 0.3 | 0.15 | 0 |
| FR2 like | 23 | 2.84 | 1.07 | 0.22 | 0.13 | 0 |
|  | 26 | 1.34 | 0.76 | 0.44 | 0 | 0 |

Unlike the co-existence study in HPUE, the Tx power assumption here is applied to all UE, so the throughput loss for higher Tx power actually will be lower due to the better signal strength, so from co-existence perspective, 23dBm can be the worst assumption.

Proposal 2: For UE Tx power, using 23 dBm as the worst assumption.

  - Comparison for different ISD for Urban Macro

For Urban macro, two different ISD assumptions are listed, 350m and 450m.

Table III Co-existence results under different ISD

| Urban macro |  | ACIR (UL average throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ISD | UE antenna | 10 | 15 | 20 | 25 | 30 |
| 450m | FR1 like | 0.8 | 0.38 | 0.18 | 0.08 | 0.04 |
|  | FR2 like | 0.46 | 0.23 | 0.11 | 0.05 | 0.02 |
| 350m | FR1 like | 0.59 | 0.29 | 0.14 | 0.07 | 0.02 |
|  | FR2 like | 0.36 | 0.17 | 0.09 | 0.04 | 0.01 |

|  |  | ACIR (UL 5% cell edge throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ISD | UE antenna | 10 | 15 | 20 | 25 | 30 |
| 450m | FR1 like | 5.77 | 2.82 | 0.93 | 0.14 | 0 |
|  | FR2 like | 2.84 | 1.07 | 0.22 | 0.13 | 0 |
| 350m | FR1 like | 5.17 | 2.26 | 1.33 | 0.76 | 0 |
|  | FR2 like | 4.91 | 1.64 | 1.07 | 0.08 | 0 |

|  |  | ACIR (DL average throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ISD | UE antenna | 10 | 15 | 20 | 25 | 30 |
| 450m | FR1 like | 3.11 | 1.79 | 0.91 | 0.46 | 0.24 |
|  | FR2 like | 1.91 | 1.12 | 0.57 | 0.29 | 0.15 |
| 350m | FR1 like | 2.52 | 1.41 | 0.77 | 0.37 | 0.19 |
|  | FR2 like | 1.59 | 0.86 | 0.49 | 0.24 | 0.11 |

|  |  | ACIR (DL 5% cell edge throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ISD | UE antenna | 10 | 15 | 20 | 25 | 30 |
| 450m | FR1 like | 17.26 | 10.79 | 5.12 | 2.17 | 1.23 |
|  | FR2 like | 13.66 | 8.19 | 3.72 | 1.86 | 0.39 |
| 350m | FR1 like | 17.71 | 10.83 | 6.21 | 2.67 | 1.49 |
|  | FR2 like | 14.84 | 8.27 | 5.81 | 2.96 | 1.52 |

Based on the results above, the bottleneck of ACIR across all cases is DL cell edge and, in this case, the 350m is worse than 450m due to the higher interference from adjacent cell.

Proposal 3: For ISD assumption in Uma, 350m is used as worst case.

  - Coordinated vs un-coordinated for Urban Macro

For Urban macro, both coordinated and un-coordinated are assumed.

Table IV Co-existence results under coordinated vs un-coordinated

| Urban macro |  | ACIR (UL average throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| layout | UE antenna | 10 | 15 | 20 | 25 | 30 |
| coordinated | FR1 like | 0.80 | 0.38 | 0.18 | 0.08 | 0.04 |
|  | FR2 like | 0.46 | 0.23 | 0.11 | 0.05 | 0.02 |
| un-coordinated | FR1 like | 0.59 | 0.30 | 0.14 | 0.007 | 0.03 |
|  | FR2 like | 0.36 | 0.17 | 0.09 | 0.04 | 0.01 |

|  |  | ACIR (UL 5% cell edge throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| layout | UE antenna | 10 | 15 | 20 | 25 | 30 |
| coordinated | FR1 like | 5.77 | 2.82 | 0.93 | 0.14 | 0 |
|  | FR2 like | 2.84 | 1.07 | 0.22 | 0.13 | 0 |
| un-coordinated | FR1 like | 5.17 | 2.26 | 1.32 | 0.76 | 0 |
|  | FR2 like | 4.91 | 1.07 | 1.64 | 0.08 | 0 |

|  |  | ACIR (DL average throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| layout | UE antenna | 10 | 15 | 20 | 25 | 30 |
| coordinated | FR1 like | 3.11 | 1.79 | 0.91 | 0.46 | 0.24 |
|  | FR2 like | 1.91 | 1.12 | 0.57 | 0.29 | 0.15 |
| un-coordinated | FR1 like | 2.52 | 1.41 | 0.77 | 0.37 | 0.19 |
|  | FR2 like | 1.59 | 0.86 | 0.49 | 0.24 | 0.11 |

|  |  | ACIR (DL 5% cell edge throughput loss) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| layout | UE antenna | 10 | 15 | 20 | 25 | 30 |
| coordinated | FR1 like | 17.26 | 10.79 | 5.12 | 2.17 | 1.23 |
|  | FR2 like | 13.66 | 8.19 | 3.72 | 1.86 | 0.39 |
| un-coordinated | FR1 like | 17.71 | 10.13 | 6.21 | 2.67 | 1.49 |
|  | FR2 like | 14.84 | 8.27 | 4.81 | 1.97 | 0.52 |

Based on the results above, we can find that for FR1-like UE, the un-coordinated layout is a little bit worse and for FR2 like coordinated is worse, so from co-existence perspective, it is better to differentiate it based on the UE type.

Proposal 4: For FR1-like UE, coordinated layout is assumed while for FR2-like UE, un-coordinated is assumed.

- Conclusion

In this contribution, we provide our preference and analysis on the co-existence assumptions.

Proposal 1: For BS antenna assumption, taking 64*24 for Uma and 4*4 for Indoor as the worst case.

Proposal 2: For UE Tx power, using 23 dBm as the worst assumption.

Proposal 3: For ISD assumption in Uma, 350m is used as worst case.

Proposal 4: For FR1-like UE, coordinated layout is assumed while for FR2-like UE, un-coordinated is assumed.

- References

- R4-2410592 TP to TR 38.922: System level simulation methodology and assumptions for coexistence study for 14800 - 15350 MHz frequency range Nokia