3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415768

Hefei, Anhui, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on the CW cancellation capability

Agenda Item:	6.21.3.4

Document for:	Approval

- Introduction

In the RAN#105, the following conclusion is achieved:

Conclusion 6

RAN guidance to RAN4:

RAN confirms that RAN4 studies and documents the methods and feasible values of CW interference cancellation according to their agreements.

In this contribution, we share our views on the CW cancellation.

- Discussion

  - Spatial domain

For A1/B scenario, the receiver and transmitter are separated, which means spatial domain isolation for the CW interference can be obtained through this deployment. The amount of spatial isolation can be achieved by pathloss isolation.

For the D1T1-A1/B scenario, the pathloss isolation can reach 61 dB, which assumes the distance between CW emitter and receiver is 20 meters for InH-DH-NLOS path loss model. Meanwhile, for the D2T2-A1/B scenario, the pathloss isolation is 54 dB when the CW emitter and receiver distance is 10 meters and the pathloss model is InH-Office-NLOS.

Observation 1：For A1/B scenario, the pathloss can provide spatial domain suppression due to the separation of CW transmitter and D2R receiver.

For A2 scenario, the D2R receiver and CW transmitter are located at same node, so the pathloss can’t provide suppression in this case, and the spatial domain suppression capability mainly from Tx/Rx antenna isolation. For D1T1 we assume the antenna isolation is 15 dB, and for D2T2 we assume 10 dB which is a typical value for NR UE.

Observation 2：For A2 scenario, the spatial domain suppression mainly from Tx/Rx antenna isolation.

| Scenario | Part1 Spatial domain suppression |
| --- | --- |
| D1T1-A1/B | 61 dB |
| D1T1-A2 | 15 dB |
| D2T2-A1/B | 54 dB |
| D2T2-A2 | 10 dB |

  - Analog domain

This part is the suppression before the LNA. A typical self-interference cancellation architecture is show in below:

Figure 1 A typical self-interference cancellation architecture [2]

The basic rationale is to generate a CW that have -180° phase difference compared to the incoming interference and combine them together. According to [2][3], the RF-IC suppression capability between 30dB~ 60dB can be achieved generally, and we assume 45dB in this contribution.

For A1/B scenario, since the CW transmitter is separated from D2R receiver, the method above actually can’t used at receiver side, so we assume 0 dB suppression capability for these cases.

Observation 3: The suppression in analog domain can be achieved by generating suitable CW inside the receiver for cancellation.

| Scenario | Part2 Analog domain suppression |
| --- | --- |
| D1T1-A1/B | 0 dB |
| D1T1-A2 | 45 dB |
| D2T2-A1/B | 0 dB |
| D2T2-A2 | 45 dB |

  - Digital domain

From our side, it is not clear how digital domain process can provide benefit on the interference suppression. Since if there is no blocking or non-linear distortion, the single tone CW interference is still single tone and converted to DC in baseband, which has no impact to detection performance as long as the ratio between desired signal and residual interference is within ADC dynamic range. If blocking happens, it is not clear how to model the distortion and how to overcome it in digital baseband.

Observation 4: The digital domain interference suppression is not modelled due to

- If there is no blocking or non-linear distortion, the single tone CW interference is still single tone and converted to DC in baseband, which has no impact to detection performance as long as the ratio between desired signal and residual interference is within ADC dynamic range.

- If blocking happens, it is not clear how to model the non-linear distortion and how to overcome it in digital baseband.

Proposal 1: The rationale that how digital domain interference suppression works should be clearly identified before the discussion of the specific suppression value.

  - IM3 product

After LNA, the CW and D2R will generate intermodulation product, and the IM3 product will overlap with D2R signal in frequency domain, and the BB filter actually cannot help on this, as shown in below:

Figure 2 Spectrum for D2R and IM3 product

Observation 5: The IM3 produced by CW and D2R will overlap with D2R which will bring receiver performance degradation.

Since this degradation is also related to the CW interference power, an equivalent CW cancellation capability can be defined to shown the impact, which is the difference between CW power at Tx side and the IM3 power at Rx side.

Proposal 2: The CW cancellation capability is defined as below:

CW cancellation capability = CW power @Tx – IM3 power @Rx

The calculation procedure is described as below

- Remaining CW interference 2K1=CW transmit power 1E1+CW Tx antenna gain 1E2-CW cancellation - where CW cancellation = Part1 spatial domain suppression + Part2 analog domain suppression - D2R receiver power2K1_1=CW transmit power 1E1+CW Tx antenna gain 1E2-CW_AIoT pathloss-AIoT_reader pathloss – return loss – 2* cable loss[1N] - RxIM3_1=2*Remaining CW interference 2K1+D2R receiver power2K1_1-2*IIP3 - RxIM3_2=2*D2R receiver power2K1_1+Remaining CW interference 2K1-2*IIP3 - IM3 modelling is widely used in 3GPP [8] - Receiver sensitivity loss [2K2] =lin2dBdB2linNoise Power [2F]dB2linNoise Power [2F]+dB2linRx IM3_1+Rx IM3_2 => Total CW cancellation Value[2K] = CW transmit power 1E1+CW Tx antenna gain 1E2 –( Rx IM3_1+Rx IM3_2 )

Based on the calculation above, the CW cancellation capability for each scenario is summarize below:

| Scenarios | D2T2-A2 | D2T2-B | D2T2-A2 | D2T2-B | D1T1-A2 | D1T1-B | D1T1-A2 | D1T1-B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | InH-LOS | InH-LOS | InH-LOS | InH-LOS | InF-DH-NLOS | InF-DH-NLOS | InF-DH-NLOS | InF-DH-NLOS |
| CW case |  |  |  |  |  |  |  |  |
| Device type | Device1 |  | Device2a |  | Device1 |  | Device2a |  |
| Total CW cancellation capability | 151.15 | 150.07 | 141.15 | 140.07 | 159.17 | 161.52 | 149.17 | 151.52 |

It is noted that the LOS model used for D2T2 and NLOS model used for D1T1, this is because in RAN1 only NLOS is assumed, as the agreements below:

Agreement

For D1T1,

- InF-DH NLOS model defined in TR38.901 is used for D2R and R2D links as pathloss model in coverage evaluation.

For D2T2,

- InF-DL and InH-Office model defined in TR38.901is used as pathloss model in coverage evaluation,

  - NLOS for D2R and R2D links if InF-DL is used

  - LOS for D2R and R2D links if InH-Office is used

We propose such method should be captured in TR and more details can be find in the Annex.

Proposal 3: The CW cancellation capability based on IM3 analysis should be captured in the TR.

- Conclusion

In this contribution, we provide our analysis on the CW cancelation capability.

Observation 1：For A1/B scenario, the pathloss can provide spatial domain suppression due to the separation of CW transmitter and D2R receiver.

Observation 2：For A2 scenario, the spatial domain suppression mainly from Tx/Rx antenna isolation.

Observation 3: The suppression in analog domain can be achieved by generating suitable CW inside the receiver for cancellation.

Observation 4: The digital domain interference suppression is not modelled due to

- If there is no blocking or non-linear distortion, the single tone CW interference is still single tone and converted to DC in baseband, which has no impact to detection performance as long as the ratio between desired signal and residual interference is within ADC dynamic range.

- If blocking happens, it is not clear how to model the non-linear distortion and how to overcome it in digital baseband.

Proposal 1: The rationale that how digital domain interference suppression works should be clearly identified before the discussion of the specific suppression value.

Observation 5: The IM3 produced by CW and D2R will overlap with D2R which will bring receiver performance degradation.

Proposal 2: The CW cancellation capability is defined as below:

CW cancellation capability = CW power @Tx – IM3 power @Rx

Proposal 3: The CW cancellation capability based on IM3 analysis should be captured in the TR.

- References

- R4-2414304 WF on Co-existence study for ambient IoT, CMCC, RAN4#112

- R4-2414309 WF on LP-WUS UE RF requirements, vivo, RAN4#112

- Annex

| system configuration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0A | Scenarios | D2T2-A2 | D2T2-B | D2T2-A2 | D2T2-B | D1T1-A2 | D1T1-B | D1T1-A2 | D1T1-B |
|  |  | InH-LOS | InH-LOS | InH-LOS | InH-LOS | InF-DH-NLOS | InF-DH-NLOS | InF-DH-NLOS | InF-DH-NLOS |
| 0A1 | CW case |  |  |  |  |  |  |  |  |
| 0B | Device type | Device1 |  | Device2a |  | Device1 |  | Device2a |  |
| 0C | Center frequency(GHz) | 0.9 |  |  |  | 0.9 |  |  |  |
| Transmitter |  |  |  |  |  |  |  |  |  |
| 1E | CW Tx power(dBm) | 23 |  |  |  | 33 |  |  |  |
| 1F | D2R bandwidth(Hz) | 160000 |  |  |  | 160000 |  |  |  |
| 1G | Tx antenna gain(dBi) | 0 |  |  |  | 2 |  |  |  |
| 1N (cable, connector loss) |  | 1 |  |  |  | 3 |  |  |  |
| Receiver |  |  |  |  |  |  |  |  |  |
| 2B | bandwidth used for the evaluated channel(Hz) | 160000 |  |  |  | 160000 |  |  |  |
| 2C | Receiver antenna gain(dBi) | 0 |  |  |  | 2 |  |  |  |
| 2D | Receiver Nosie Figure(dB) | 7 |  |  |  | 5 |  |  |  |
| 2E | Thermal Noise(dBm/Hz) | -174 |  |  |  | -174 |  |  |  |
| 2F | Noise Power(dBm) | -114.9588002 |  |  |  | -116.9588002 |  |  |  |
| 2G | Tx/Rx spatial isolation capability | 10 | 0 | 10 | 0 | 15 | 0 | 15 | 0 |
| 2F | RX cable, connector loss | 1 |  |  |  | 3 |  |  |  |
| 2J | Rx RF-IC suppression capability; For outside topology, this item is pathloss suppression interference | 45 | 54.46 | 45 | 54.46 | 45 | 61.18 | 45 | 61.18 |
|  | CW cancellation (dB) | 55 | 54.46 | 55 | 54.46 | 60 | 61.18 | 60 | 61.18 |
| 2K1 | Remaining CW interference(dB) | -34 | -33.46 | -34 | -33.46 | -29 | -30.18 | -29 | -30.18 |
| 2K2 | D2R power (dBm), for D1T1 scenario, assuming 10m distance between reader and tag; for D2T2 scenario, assuming 5m distance between reader and tag | -72.15 | -72.15 | -62.15 | -62.15 | -88.17 | -88.17 | -78.17 | -78.17 |
| 2K3 | Rx IIP3 capability(dBm) | -6 | -6 | -6 | -6 | -10 | -10 | -10 | -10 |
| 2K4_1 | Rx IM3#1 = 2K1+2*2K2-2*2K3 | -166.31 | -165.77 | -146.31 | -145.77 | -185.34 | -186.52 | -165.34 | -166.52 |
| 2K4_2 | Rx IM3#2 = 2K2+2*2K1-2*2K3 | -128.15 | -127.08 | -118.15 | -117.08 | -126.17 | -128.52 | -116.17 | -118.52 |
| 2K4 | Rx IM3 = 2K4_1+2K4_2 | -128.15 | -127.07 | -118.15 | -117.07 | -126.17 | -128.52 | -116.17 | -118.52 |
| 2K5 | Total CW cancellation capability=1E-2K4 | 151.15 | 150.07 | 141.15 | 140.07 | 159.17 | 161.52 | 149.17 | 151.52 |