3GPP TSG-RAN WG4 Meeting #112bis	R4-2415769
Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.12.1

Source:	vivo

Title:	Discussions and reply LS on LTE TRPTRS test method for UE over 72mm wide

Document for:	Approval

1	Introduction

ETSI sent an LS to RAN4 on adding TRP TRS test method for UE>72mm into LTE spec [1].

2. Actions:

MSG TFES would respectfully ask 3GPP RAN4 and RAN5 to include TRP/TRS phantom definition and positioning guideline for UE above 72mm to LTE specifications by March 2025. In case this is not feasible, MSG TFES would like to ask for confirmation whether there are technical concerns on directly applying the NR specifications on the phantom definition and positioning guidelines also for LTE.

This contribution provides views on how to update the LTE OTA specification to meet ETSI requests, and a draft reply LS is prepared in the Annex part of this contribution. The corresponding draft CR is also prepared in [2].

2	Discussion

RAN4 has successfully concluded the NR TRP TRS talk mode and browsing mode requirements for Size 1 UE (wide, width >72mm and ≤92mm) in Rel-18 [3]. The latest version of TS 38.161 captures the performance requirements and corresponding permitted test methods [4].

In TS 38.161, the phantom definition (Head phantom, PDA Grip hand, Wide Grip hand, Forearm) is described in Annex B.2, and the UE positioning guidelines (Browsing mode, Talk mode, Wrist-worn mode) is specified in Annex B.3.

To meet ETSI requests on updating LTE specifications to include Wide Grip hand definition and positioning guidelines, a simple way is to update the spec with reference directly to TS 38.161 Annex part.

Given there is no LTE TRP TRS requirements in TS 37.144, therefore, it is straight forward that there is no need to update this RAN4 core requirement specification. However, TR 37.902 is the LTE TRP TRS test method, which is the basis for RAN5 to specify test specifications, thus this specification should be updated to include Wide grip hand related test method.

Proposal 1: update LTE TRP TRS test specification TR 37.902 to include TRP/TRS phantom definition and positioning guideline for UE above 72mm by reference to NR TS 38.161.

The corresponding draft CR is prepared in [2]. After endorsement of the CR content, catA CRs will also be provided next meeting.

The draft RAN4 Reply LS is also prepared in the Annex part of this document. One thing should be decided is that whether the reply LS should be sent by WG or RAN-P. Given potentially RAN5 also needs to update the test specification TS 37.544 accordingly. Alternatively, the reply LS to ETSI can also be sent by RAN plenary meeting in December RAN#106, after updating the specifications in RAN4 and RAN5.

Proposal 2: After concluding the spec update, a reply LS to ETSI is required. Either RAN4 LS or LS from RAN-P in Dec 2024.

3 Conclusion

In this paper, we provide our views on LS from ETSI on LTE TRP TRS test method for UE >72mm.

Proposal 1: update LTE TRP TRS test specification TR 37.902 to include TRP/TRS phantom definition and positioning guideline for UE above 72mm by reference to NR TS 38.161.

The corresponding draft CR is prepared in [2]. After endorsement of the CR content, cat A CRs will also be provided next meeting.

The draft RAN4 Reply LS is also prepared in the Annex part of this document.

Proposal 2: After concluding the spec update, a reply LS to ETSI is required. Either RAN4 LS or LS from RAN-P in Dec 2024.

4	References

- R4-2414918, LS to 3GPP on LTE TRP/TRS test method for UE over 72mm wide, ETSI TC MSG/TFES, RAN4#112bis, 2024.

- R4-2415770, Draft CR on adding test method for UE over 72mm wide, vivo, RAN4#112bis

- RP-241829, SR for NR_FR1_TRP_TRS enhancement, vivo, RAN#105, Sep 2024.

- TS 38.161 V18.2.0

- TS 37.902 v12.1.0

Annex (draft reply LS)

3GPP TSG-RAN WG4 Meeting #112bis	R4-241xxxx
Hefei, Anhui, China, 14th – 18th October, 2024

Title:	draft Reply LS on 3GPP LTE TRP/TRS test method for UE over 72mm wide

Response to:		R4-2414918/TFES(24)081028r2

Release:

Work Item:

Source:	3GPP RAN4 (or RAN)

To:	ETSI TC MSG/TFES

Cc:	3GPP RAN5

Contact person:	Ruixin Wang

ruixin.wang@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

3GPP RAN4 (RAN) would like to thank ETSI TC MSG for their LS in R4-2414918/TFES(24)081028r2 on LTE TRP/TRS test method for UE over 72mm wide.

3GPP RAN4 (and RAN5) has discussed and finalize the spec update with adding corresponding phantom definition and UE positioning for LTE device width > 72mm into 3GPP LTE specifications TR 37.902 (and TS 37.544).

3GPP would like to keep collaboration with ETSI TC MSG on OTA test method and requirements.

2	Actions

To GCF CAG:

ACTION: 	3GPP respectfully asks ETSI TC MSG to take the above feedback into account when developing LTE OTA antenna performance standard.

3	Dates of next TSG-RAN WG4 meetings

TSG-RAN WG4 Meeting #113                           	 18-22 November, 2024    		 Orlando, US

TSG-RAN WG4 Meeting #114                           	 17-21 February, 2025    		       Athens, GR