3GPP TSG-RAN WG4 Meeting #112bis	R4-2415773

Hefei China, 14th – 28th Oct, 2024

Agenda item:	4.3.1

Source:	vivo, CAICT

Title:	Discussions on spectrum analyzer for wideband TRP measurement

Document for:	Approval

1	Introduction

Spectrum analyzers are widely utilized in 2G, 3G, and 4G OTA TRP testing, according the latest 3GPP technical report [1], the NR System Simulator is the typical power measurement device for TRP testing, spectrum analyzer is not precluded however there is currently no detailed configuration to support using spectrum analyzer to test NR FR1 TRP because of the wide bandwidth of NR FR1.

Current high-end spectrum analyzers support a maximum test bandwidth of over 80MHz, which adequately meet the power-related test requirements for 5G NR FR1 OTA with faster test speed and relatively low uncertainty contributions. CTIA Test Plan [2] and CCSA Industry standard [3] has outlined the detail test parameters for using spectrum analyzers to measure NR FR1 TRP.

This contribution presents the detailed NR FR1 TRP test parameters for using spectrum analyzers, and provide text proposal in [7]

2	Discussion

Spectrum analyzer can directly control the relevant parameters to measure the different signal types, there are typically two test modes for spectrum analyzer: Total channel power and integrated channel power.

  - Total channel power

If the test signal is smaller than the resolution bandwidth (RBW) of the spectrum analyzer, then we need to set the spectrum analyzer as zero-span sweep, the root mean square (RMS) or sample detector can be used. For a Gaussian RBW filter shape, the RBW must be at least three times of the channel bandwidth to obtain broadband power readings within 0.1 dB of the actual channel power. Newer spectrum analyzers often offer rectangular or flat-top filter options, which typically converge to the correct value when the RBW setting is something greater than 120% of the channel bandwidth. Setting the appropriate trigger method and scan time to capture stable uplink signals, therefore, broadband power measurement could be performed using a Gaussian RBW filter or a Flat-top RBW filter with the VBW set to the maximum value supported by the spectrum analyzer.

  - Integrated channel power

If the bandwidth of the test signal is greater than the resolution bandwidth (RBW) of the spectrum analyzer, the in-band power integration method can be used to test the signal power. The RBW setting should not be greater than 30kHz, and the video bandwidth (VBW) setting will not affect the measurement accuracy. The integration bandwidth setting requirement is equal to the bandwidth being tested.

For integrated channel power measurement, it is necessary to determine the resolution bandwidth correction factor of the spectrum analyzer and use this factor to correct the TRP test results.

The method for measuring the spectrum analyzer's resolution bandwidth correction factor is FFS

  - NR FR1 TRP Test Parameters

NR FR1 uses CP-OFDM in the Uplink and Downlink as well as DFT-s-OFDM in the Uplink, the occupied bandwidth is configurable by the number of specified resource blocks (RBs) independent of the actual channel bandwidth, allowing multiple UEs to share one channel. NR FR1 supports scalable sub-carrier spacing (SCS) of 15 kHz, 30 kHz, and 60 kHz as well as wider channel bandwidths (CBWs) compared to LTE. The relationship between SCS, CBW, and maximum transmission bandwidth configuration NRB is outlined in 3GPP TS 38.101-1 [6], specifically Section 5.3.2. The sub-channel bandwidth (fspan) should be calculated as follow:

Where the quantity (10/9) is the adjustment factor to ensure that the span is adequate to capture the power contribution from the band edges in the final integrated channel power measurement.

For NR FR1 TRP measurement using spectrum analyzers, the power measurement may be either the total power or integrated channel power modes using the paraments in Table 1 below centered on the occupied bandwidth define by the RB allocation. The required frequency offsets specified in Table 1 could be determined: generally as: [(RBStart)+(RBAllocation-Total RBs)/2] *12 *SCS(kHz).

Table 1 Broadband Power Mode Measurement Requirements for NR FR1

| Channel BW (MHz) | SCS [kHz] | NRB | RB Allocation | RB Start | Frequency Offset (MHz) | fspan (MHz)1 | fGaussian (MHz) | fflat top (MHz)1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 15 | 25 | 12 | 6 | -0.09 | 2.4 | 7 | 3 |
| 10 | 15 | 52 | 25 | 12 | -0.27 | 5 | 14 | 6 |
| 15 | 15 | 79 | 36 | 18 | -0.63 | 7.2 | 20 | 8 |
| 20 | 15 | 106 | 50 | 25 | -0.54 | 10 | 27 | 11 |
| 30 | 15 | 160 | TBD | TBD | TBD | TBD | TBD | TBD |
| 100 | 30 | 273 | 135 | 67 | -0.72 | 54 | 146 | 59 |
| fRBW |  |  |  | 30 kHz |  |  |  |  |
| Tdwell or Tsweep |  |  |  | 100 ms |  |  |  |  |
| Tshort dwell or Tshort sweep |  |  |  | 20 ms |  |  |  |  |
| Pflat |  |  |  | 70% |  |  |  |  |

To support the usage of spectrum analyzer for NR FR1 TRP measurements, it is suggested to capture the detailed configurations into TR 38.870.

Proposal 1: Adding detailed configuration parameters of spectrum analyzer for TRP measurement into TR 38.870. The draft CR is provided in [7].

- Conclusion

This contribution makes the following proposals:

Proposal 1: Adding detailed configuration parameters of spectrum analyzer for TRP measurement into TR 38.870. The draft CR is provided in [7].

4	References

- 3GPP TR 38.870: “Enhanced Over-the-Air (OTA) test methods for NR FR1 Total Radiated Power (TRP) and Total Radiated Sensitivity (TRS)”

- CTIA Certification™: “CTIA Certification Test Plan for Wireless Device Over-the-Air Performance, CTIA 01.73 Supporting Procedures

- CCSA YD/T 1484.1-2023 Measurement method for radiated RF power and receiver performance of wireless device Part 1:General requirement

- 3GPP TS 37.544: “Universal Terrestrial Radio Access (UTRA) and Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) Over The Air (OTA) performance; Conformance testing

- CTIA OTA_2024_004_016 V3 CR on Sub-channel Bandwidth Calculation in 01.73 SUN Siyang (CTTL China Telecommunication Technology Labs), Elvis Yen (Dell), Peter Liao (SGS), Benny Dong (vivo), Jun Luo (ETS), and Dayan Gao (BV)

- 3GPP TS 38.101-1 “Technical specification NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

- R4-2415771, draft CR to TR 38.870 on spectrum analyzer configurations, vivo, RAN meeting #112bis