3GPP TSG-RAN WG4 Meeting #112bis                                                    R4-2415777  
Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	 6.12.3.1

Source:	 vivo

Title:	Working procedure for Rel-19 2Tx lab alignment and UE performance measurement campaign

Document for:	Approval

- Introduction

This contribution provides the working procedure based on agreements in WF[1] and Rel-18 working procedure [2], to provide a guidance to organize the Rel-19 2Tx lab alignment activity and UE performance measurement campaign.

- Discussion

To organize Rel-19 OTA work, framework in Rel-18 can be resued as much as possible, e.g., working procedure, UE information template, UE data template, as baseline.

Several aspects should be discussed and concluded this meeting:

- Volunter test labs for 2Tx lab alignment activity. Volunter test labs for measurement campaign (from Rel-17, Rel-18 aligned AC test labs).

- Whether only TRP is needed for 2Tx testing. If TRS is not needed, then by default the TRS requirements for the same band with 1Tx and 2Tx would be the same.

- RAN4 should discuss whether there is MU different for test system under 1Tx and 2Tx UE measurement.

- RAN4 should decide the detailed time schedule for DUT collection from interested companies, and measurement campaign.

- A reasonable band list should be discussed and decided. The band selection should consider at least industry interests, commercialization and number of available UEs.

- How to specify PDA requirements (size 2 UE)

- Working procedure for Rel-19 AC 2Tx Lab Alignment Campaign (for approval)

Working procedure for Rel-19 AC 2Tx Lab Alignment Campaign

- The purpose of 2Tx Lab Alignment Campaign is to ensure there is no unexpected lab deviation and establish full trust and confidence on the results of UE with 2Tx configuration.

- Test labs are invited to participate to the lab alignment and test campaign, the following conditions should be fulfilled:

  - Participating lab should be accredited under ISO 17025 (ISO 17025 accredited labs) and have any of 3GPP TS 37.544, CCSA YD/T 1484.6, and CTIA OTA Test Plan listed on its accreditation scope.

  - Participating lab should have anechoic chamber(s) ready to support testing based on 3GPP TS 38.161.

  - Participating lab should have sufficient test resource to provide the on-time measurement results without delay.

- Test methodology:

  - Test plan: 3GPP TR 38.870 v18.2.0;

- Test cases for Lab Alignment Campaign:

  - Test bands: n41, n78; two typical bands non-coherent UL-MIMO

  - Number of test cases per band: only TRP; BHHL and BHHR at low/mid/high channel,  HL and HR at low/mid/high channel;

  - Use scenario: Head and Hand phantom (Talk mode) and Hand only (Browsing mode)

  - Hand Phantom: Wide Grip hand

  - Operation mode: NR Standalone (SA)

  - Number of Tx chain: UE with 2Tx (non-coherent UL MIMO)

- Lab Alignment Device (LAD): in total 2 devices

  - 2 New devices (LAD 5 and LAD 6) for band n41 and n78 non-coherent UL-MIMO;

  - UE information will be further added

- Test results submitting:

  - Using the same worksheet template in [Rel-18 R4-2313892 as starting point, update is needed] to submit the measurement results

  - The measurement results of LADs should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - Results shall not be shared between labs before submitting to RAN4 meetings or sharing in the RAN4 reflector. Comparison and lab alignment analysis should only be done in RAN4 meetings/discussions

  - Sharing of the measurements results of a specific device from test labs is permitted only towards the 3GPP member that provided such device (i.e., device’s owner) before submitting to RAN4

- Lab alignment criteria:

  - The pass/fail criteria are defined as the maximum deviation between the measurement result and the reference value

  - Confirm the reference value derived based on the per-band per-PC averaging approach (linear average with dBm) of lab alignment data pool from ≥3 labs submitted before end of [RAN4#TBD] as baseline.

  - Apparent outliers will not be considered in averaging process. The value deviates over 1.5*MU from all the other lab’s results should be identified as apparent outlier.

  - Pass/fail limit for lab alignment should be defined as 0.75*MU as baseline. MU value is the expanded MU for 2Tx condition ( need further check whether same as 1Tx).

  - The summation form for TRP and TRS lab alignment should keep consistent during the calculation process of TRP TRS lab alignment from each company i.e. sin weights approach. Only traditional approach (15-degrees TRP and 30-degrees TRS) should be used during lab alignment activity to reduce unnecessary uncertainty.

  - The timeline of 2Tx AC alignment activity is in [TBD]. How to treat late submission results and confirm the alignment: TBD

- Test lab procedures:

  - LAD delivery scheme

    - Decide LAD delivery scheme after all the test lab and LAD information being confirmed (after the confirmation of volunteered labs and LADs).

  - LAD measurement time in each test lab: finalize LAD measurement within [5] workdays, and deliver to next lab ASAP with LAD delivery In/Out information shared in reflector.

  - Encourage test labs to share resulting combined MU based on their own systems.

Table 1: Rel-19 2Tx LAD information

| Rel-19 2Tx Lab Alignment Devices (LADs) |  |  |  |
| --- | --- | --- | --- |
| DUT | Provider/Owner | Contact | Note |
| LAD5 (new device for Rel-19 2Tx) | vivo | Ruixin Wang ruixin. wang@vivo.com | For n41 and n78 testing. |
| LAD6 (new device for Rel-19 2Tx) | vivo | Ruixin Wang ruixin. wang@vivo.com | For n41 and n78 testing. |
| Note: To keep the test burden, two devices per band. |  |  |  |

- Working procedure for Rel-19 Measurement Campaign to define requirements (for approval)

Working procedure for TRP TRS Performance Test Campaign to define requirements

- The purpose of Test Campaign is to collect devices results for the permitted labs after lab-alignment activity for the definition of the FR1 TRP TRS requirements.

- Test cases for TRP TRS Performance Test Campaign:

  - Test bands: discuss and decide a reasonable scope; in the WID: 1Tx at new bands n3, n5, n7, n8, n20, n38, n77, n79; 2Tx at bands n41 and n78;

  - Use scenarios: Both Browsing mode (Hand phantom only), i.e., Hand Left and Hand Right; and Talk mode (head and hand phantom), i.e., BHHL and BHHR

  - Hand Phantom: Wide Grip Hand as first priority, PDA hand is not precluded if sufficient devices are available

  - Operation mode: SA with 1Tx  as phase 1. SA with 2Tx as phase 2 after 2Tx lab alignment activity.

- Commercial Device (Smartphone) selection criteria for TRP TRS Performance Test Campaign:

  - DUT size: Size 1(width >72mm and ≤92mm)

  - DUT capability: support for Bands those listed in the WID is preferred, but devices supporting only a subset of the above bands can equally be used in the measurement campaign for such supported bands

  - The following selection criteria can also be considered:

- Year of production: [2022-2025]

- Brand variety

- Price range (to capture different price segment, including High/Mid/Low-end products)

- Popularity

- Number of bands supported

  - Power Class: PC2 and PC3. For bands support both PC2 and PC3, focus on PC2 measurements, but both PC2 and PC3 requirements are needed. For PC3 bands, only PC3 requirements.

- Devices provisioning:

  - Any 3GPP member can work with test labs to provide devices to the aligned test labs (each test lab shall provide a single measurement result set for repeated UE model into RAN4 measurement campaign data pool)

    - FFS for the same UE model with supporting different set of bands

  - Logistical aspects for devices provisioning to the labs are TBD

- Test results submitting:

  - UE information disclosure: the following rules should be considered

    - UE model information based on template [Rel-18 R4-2408104 as starting point, update is needed] can be shared to MCC. RAN4 Secretary will cover the role of the trusted and neutral third party for the whole procedure.

    - laboratories use the same template to submit the device information. The UE information should NOT BE CORRELATED with the order in the measurement data submitted by the same lab for the respective list of devices in c, i.e., the UE mode order in the list should be randomly disrupted.

      - Information of the devices that are going to be measured shall be shared with the RAN4 Secretary as soon as available (i.e., before the measurement activity on such devices starts)

      - The RAN4 Secretary updates the summary of statistical information  and publishes it to 3GPP RAN4 (i.e., living document) in order to monitor the achievement of the thresholds in a timely manner and take the proper actions if these are not met

    - Statistics of UE information can be shared to RAN4 reflector after TBD meeting by RAN4 secretary after post-processing

    - Consider following aspects as thresholds related to the devices pool for Rel-19

      - Minimum number of devices for each band, each device size, each power class: TBD

      - Minimum number of device models: TBD

      - Minimum number of devices' vendors: TBD

      - Percentage of devices from second-half 2022 to 2025: TBD

      - Percentage of the devices that are certified by PTCRB/GCF/NAL-CTA/CE/FCC: TBD

  - Laboratories use the same worksheet template in [Rel-18 R4-2315844 as starting point, update is needed] to submit the measurement results for Rel-19 3GPP TRP TRS performance data pool.

  - The measurement results should be submitted to RAN4 by anonymous approach (the UE model should not be disclosed)

  - The allowed maximum number of submitted devices from each lab is [15] (depends on how many test labs will join the activity) with a minimum of 3 devices.

  - Only the results from aligned labs will be considered for specifying requirements

  - The progress in each lab are encouraged to share on the RAN4 reflector (for example - how many devices have been measured and on which bands)

  - TRP and TIS Quantities based on Clenshaw-Curtis quadrature and traditional sin(theta) weighting are both allowed during Performance campaign test. This information should be provided from each test lab when submitting measurement results.

- Specify TRP TRS requirements:

  - Minimum number of devices for defining requirements for each band, each device size, each power class (requirement will not be specified if measurement results is less than): [30]

  - Performance part of the work will proceed in a contribution-driven manner. Start with one type of device width requirement which is most efficient to collect enough results in Rel-19.

  - Method of limits derivation: per-band Data driven approach

  - The value at [80%] percentile of the CDF curve could be selected as the starting point for minimum requirement discussion

  - For a band supporting both PC2 and PC3, specify PC3 requirement based on finalized PC2 requirements, for TDD bands 3dB offset, for FDD bands TBD.

- Test lab procedures for 1Tx

  - Tx Antenna switching: for 1Tx configuration, test lab should make sure the testing follows the TAS OFF procedure, i.e., lock the UE antenna to primary antenna yielding best TRP. Assistants from OEM may be needed.

  - Time-averaging algorithm (TAA): if supported by UE, test lab should make sure TAA should be disabled. Assistants from OEM or chipset vendor may be needed. TAA OFF can be based on UE declaration.

  - For UE support PC2 at one band, PC3 testing is not needed.

  - Newly defined Coarser measurement grid in TR 38.870 can be used for TRP TRS measurement

- Test lab procedures for 2Tx

  - TBD.

- Rel-19 Volunteer test labs (for collecting feedback)

Volunteer test labs for 2Tx lab alignment activity:

| Rel-19 Volunteer lab for 2Tx lab alignment activity |  |  |  |
| --- | --- | --- | --- |
| Volunteer lab | Location | Contact | Note |
| TBA |  |  |  |

Proposal: Approve the working procedure for Rel-19 2Tx lab alignment and measurement campaign in Section 3 and Section 4 of this contribution.

- Conclusion

In this contribution, we propose the working procedure for Rel-19 TRP TRS performance part work:

Proposal: Approve the working procedure for Rel-19 2Tx lab alignment and measurement campaign in Section 3 and Section 4 of this contribution.

# References

- R4-2315842, Working procedure for Rel-18 TRP TRS lab alignment and harmonization campaign, vivo, RAN4 #108bis

- R4-2413603, WF for [112][333] TRP_TRS_MIMO_OTA, vivo, RAN4 #112

- R4-2315844, (Template) Measurement results for TRP TRS Performance Test Campaign, vivo, RAN4#108bis

- R4-2313892, [Template] Measurement results for 3GPP Rel-18 TRP TRS AC lab alignment activity, vivo, RAN4#108

- R4-2408104, Updated Rel-18_TRP TRS data collection template, vivo, RAN4 #111