3GPP TSG-RAN WG4 Meeting #112bis		R4-2415781

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	 6.23.2.3

Source:	 vivo

Title:	Discussions on LP-WUS IBB and OBB requirements

Document for:	 Approval

- Introduction

This contribution shares further views on LP-WUS IBB and OBB requirements.

- LP-WUS UE RF

In RAN4#111 meeting, it was agreed that RAN4 should further check whether same interference level of IBB and OBB as MR could be assumed for LP-WUR [1].

Issue 3-1-1: IBB and OBB assumption

Agreement:

  - RAN4 further check and discuss whether same interference level of IBB and OBB as MR is assumed for LP-WUR.

    - With assumption that Rx dynamic range for LR may not be as good as MR

Issue 3-1-2: IBB and OBB requirements

Agreement:

  - RAN4 further discuss detailed test condition for IBB and OBB, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level.

    - FFS IBB and OBB requirements

Issue 3-1-4: Intermodulation requirements

Agreement:

  - RAN4 further discuss detailed test condition for Intermodulation requirements, e.g., location of LP-WUS in-band, MR channel bandwidth, whether guard RBs are allocated, power level.

    - FFS Intermodulation requirements requirement

Based on the guard RB analysis of ACS, RAN4 should firstly decide whether same interference level can be assumed for LP-WUR. Besides, the phase noise performance of LP-WUR may have impacts on receiver performance [2]. Therefore, it is suggested that RAN4 perform simulation evaluation to specify reasonable IBB/OBB requirements for LP-WUS/WUR.

The following parameters in table below for IBB/OBB evaluation:

| Attributes | Assumptions |  |  |
| --- | --- | --- | --- |
| Case name (waveform) | OOK-1 waveform |  | OOK-4 waveform |
| Center frequency | 900MHz, 2.6GHz and 3.5GHz |  |  |
| Channel structure | Total 8/16 bits |  |  |
| Chip rate | M=1 | M=1/2/4 |  |
| WUS duration | FFS |  |  |
| Coding | 1/2 rate Manchester coding |  |  |
| Time error | 0 |  |  |
| Residual Frequency error | 0/10/20 ppm |  |  |
| SCS | 30kHz |  |  |
| UE Channel BW | 20MHz (51 RB)-case 1 10MHz (24 RB)-case 2 5MHz (11 RB)-case 3 |  |  |
| WUS RB | - Fixed 11RB ~ 3.96MHz for 10MHz and 20MHz cases |  |  |
| Position within channel | - For 10/20MHz CBW, Center for ASCS, edge for ACS [assume no ASCS impact] - For 5MHz CBW, fixed center of channel |  |  |
| Guardband of NR channel, both wanted cell and interfer cell (ACS) | - For wanted signal: 505kHz for 5MHz, 665kHz for 10MH, 805kHz for 20Mhz - For interference cell2 5MHz: fixed 505kHz |  |  |
| Guard RB | - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB |  |  |
| Filter | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |  |  |
| ASCS | PDSCH mapped on RBs not used for LP-WUS and Guard RB; EPRE of PDSCH /EPRE of LP-WUS = 0 dB Same PSD with WUS signal |  |  |
| ACS | PDSCH mapped on interference RBs (11RB for 5MHz CBW), one side; EPRE of PDSCH /EPRE of in-band LP-WUS = [20~33] dB NOTE: decide the interference level depending on SNR |  |  |
| Wanted signal level | For ACS, REFSENS + 14 dB for LP-WUS For IBB and OBB, FFS |  |  |
| Sampling rate | 7.68MHz |  |  |
| ADC bit width | 4/8 bits ADC for ASCS/ACS Encourage companies to provide simulation results with both options for comparison |  |  |
| Phase noise | Not modelled (check and discuss this meeting) |  |  |
| Non-linearities | Not modelled |  |  |
| Power boosting | EPRE ratio: 0dB/3dB for OOK-1/OOK-4 NOTE: 3dB is optional for simulation |  |  |
| Channel Model | Option 1: TDL-C 300 Option 2: AWGN Note: encourage companies to provide simulation results with both options |  |  |
| Performance metric | - 1% MDR/BLER as baseline and 5% MDR/BLER as optional - The following false alarm rate can be considered - 1% - 5% Providing the information whether the false alarm rate is considered or not |  |  |

Proposal 1: The IBB/OBB requirements for LP-WUS should be re-evaluated. Simulation assumptions of ACS can be reused as much as possible.

Proposal 2: The detailed test parameters, e.g., interference signal level, wanted signal level, should be re-defined.

For the above requirements, RAN4 should assume that the guard RBs for ACS and ASCS are always configured.

Proposal 3: The concluded guard RBs for ACS/ASCS should be configured for the above requirements testing.

- Conclusion

In this contribution, we share our views on other LP-WUR RF requirements:

Proposal 1: The IBB/OBB requirements for LP-WUS should be re-evaluated. Simulation assumptions of ACS can be reused as much as possible.

Proposal 2: The detailed test parameters, e.g., interference signal level, wanted signal level, should be re-defined.

Proposal 3: The concluded guard RBs for ACS/ASCS should be configured for the above requirements testing.

# References

- R4-2410569, WF for [111][136] NR_LPWUS_UERF, vivo, RAN4#111

- R4-2415780, Further discussions on LP-WUS REFSENS, ASCS and ACS, vivo, RAN4#112bis