3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415787

Hefei, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Discussion on SRS IL imbalance reporting

Agenda Item:	6.1.1.3.4

Document for:	Approval

Introduction

In RAN#103, a new WID [1] was approved on UE RF enhancements for NR FR1/FR2 and EN-DC.

In RAN4#110bis, the 6Rx related issue has been discussed. The topic summary is in [2] and a WF is agreed in [3]. A discussion paper in [4] is also provided for 6Rx related topics. Among this WF, the SRS IL imbalance is started. This has been discussed for a long time in REl-18 but no conclusion has been reached.

In RAN4#111, a dedicated agenda is set up for the SRS IL imbalance issue. A WF is agreed in [9] and summary is in [10], in which the common understandings were proposed to be discussed as the base for tentative solutions.

In RAN4#112, the common understandings were discussed. The summary is in [13] and WF in [14] which is as following:

Issue 4-1-1: Whether to solve SRS IL imbalance issue in Rel-19

Way Forward: RAN4 to further discuss the following options.

-	Option 1: RAN4 should not continue the discussion on how to solve the SRS IL imbalance issue.

-	Option 2: Continue to pursue a solution to the SRS IL imbalance issue dependent on the outcome of Issue 4-1-2 in the approved WF in R4-2410751.

Issue 4-1-2: Initial Considerations for SRS IL imbalance issue

Way forward: Given the different views amongst companies, RAN4 to further discuss the set of initial considerations which will allow companies to have a common understanding for the study including existing UE behavior for SRS transmissions in case of SRS IL.

-	Companies are encouraged to bring analysis on the existing UE behavior and achievable power imbalance for SRS transmissions based on current specification and UE implementations in case of SRS IL.

-	Companies are encouraged to analyse the impact of SRS IL imbalance on NW performance degradation.

Issue 4-2-1: Candidate solutions for the SRS IL imbalance issue

Way forward: Companies encouraged to provide compromised solutions for consideration with minimal impact to the specification and to indicate the specific impacts to RAN1, RAN2, and RAN4 specifications and performance gain.

Unfortunately, not too much progress were made up till now, and the latest WF are almost identical to previous one except the last issue. Even for the last issue, which says “Companies encouraged to provide compromised solutions”, still it has a lot of preconditions such as “minimal impact” and “performance gain”.

It is noted that the discussion in [11] and [15] already well covered the issues in the latest WF. In this paper, the main contents of [15] is re-submitted, with certain update, mainly on the latest issue for the last proposal.

Discussion

Background in Rel-18 and early Rel-19

In Rel-18, the SRS IL imbalance issue is raised in the scope of 8Rx. After some discussion, a preliminary LS [5] was sent to RAN1 to ask RAN1 to consider the reporting of IL imbalance. However, no conclusion has been reached in RAN1, and no reply LS was able to be sent back. Some background discussion can reference the RAN1 FL summary [6]. In the meantime, RAN4 discussion also did not make any concrete progress. There are strong concerns about the need to introduce such reporting, and quite diverse proposals were made. After a draft LS [7] was noted, this work was not pursued in Rel-18 anymore.

In Rel-19 early stage before RAN4#111, although SRS IL imbalance was not formally written into WID [1], this is still regarded as a topic and now has a dedicated agenda. Some draft proposals were already presented and documented in [2][3], and most of them are resubmissions of Rel-18 views.

Impact of Dynamic antenna switching

To make more progress, more constructive inputs were suggested. We also would like to discuss some general principles and considerations first for this topic and try to converge on some general guidelines.

Firstly, many previous contributions have repeatedly emphasized the already widespread implementation of dynamic antenna switching in current UE. By doing this, optimum antenna(s) would be selected to adapt to different UE positions and conditions. Though is targeted for Tx, the Rx would also be impacted sometimes for FDD, and this is purely by UE implementation without reporting. Under this condition, the IL corresponding to any antenna port would be changed among a set of cases, thus making any pure static reporting meaningless.

Observation 1: Dynamic antenna switching is widely used, and the impact to the perceivable IL has to be considered.

About Compensation

A basic understanding of UE behaviour for compensation is also needed. Among the different proposals, [8] has emphasized that the RAN1 equation has some implication that UE has to do some compensation to achieve balanced power, which we think is problematic. All the tolerances related parameters and concepts are defined in RAN4 not in RAN1, and RAN1 spec would not explicitly consider tolerances. For the power control part, RAN1 use Pcmax as a “bridge” between the schemes of two working groups. Our understanding is that the tolerance would be implicitly also considered in RAN1, but this does not preclude the introduction of them in RAN4.

Based on this analysis, there is no “mandatory compensation” by UE in current stage, and it is currently purely an implementation.

Observation 2: Currently there is no “mandatory compensation” requirement or behaviour defined or implied for UE.

Tx-Rx imbalance

The Tx-Rx imbalance matters for SRS antenna switching. To be specific, the very basic idea of SRS antenna switching is using UL channel matrix obtained by sounding for DL. This means that even UL imbalance is perfectly compensated between different branches, the DL channel matrix is still not the same. This would dimmish the possible performance gain introduced by IL compensation. Some evaluation was done in previous release [12] to demonstrate the possible performance degradation.

To a certain degree, the Rx part has its own “compensation” of imbalance. Currently there are ways to achieve same RSRP measurement by adjusting ADC, to achieve the effect of the removing different Rx IL from measurement perspective. This is independent from the Tx part, though also might be impacted by dynamic antenna switching in some cases mainly for FDD since some links are reused for Tx/Rx. For SRS antenna switching which is mainly used for TDD, the impact of dynamic antenna switching may not have much impact for Rx since the links can be independent in different time slot.

However, even with “compensation” in both UL/DL, some more factors may need to be considered. E.g. Antenna performance. To be specific, the there are following conditions:

- For the same antenna, Tx/Rx performance can be different;

- The difference can related to many factors frequency band, signal strength, different antenna etc;

- The difference is not small, and can be several dB for low frequencies

Though not usually considered, these OTA performance would introduce more uncertainty factors in the actual network and bring difficulties for optimization.

Observation 3:  The Tx-Rx imbalance related points:

- Tx-Rx imbalance matters for SRS antenna switching;

- Rx part do have its own “compensation” of imbalance;

- More factors such as antenna performance may also have impact on Tx-Rx imbalance;

- Bring more uncertainty factors in the network.

Based on the previous discussion, a general proposal is suggested:

Proposal 1: Discuss some general understandings may be helpful to make progress, e.g.:

- Widely used Dynamic antenna switching have to be considered.

- Currently there is no “mandatory compensation” requirement or behaviour for UE.

- Tx-Rx imbalance still need further consideration..

Various other considerations

The current WID include only RAN4, and no RAN1/2 scope identified yet. In addition, there were no RAN1 spec listed as would be impacted yet. RAN2 spec 38.306 and 38.331 were listed as impacted TS if capabilities need to be specified.

Based on this condition, our understanding is that possible RAN1 impact is not ready to be accepted by RAN4. Although we may have precedence to still add work to RAN1 in a similar condition, any such attempt should be sent to RAN or RAN1, and RAN4 cannot have a conclusion by ourselves.

Observation 4: Current WID scope and impacted TS/TR doesn’t involve RAN1.

Proposal 2: RAN4 should avoid RAN1 impact as much as possible, and should not conclude a scheme with RAN1 impact without confirmation from RAN/RAN1.

Even in case reporting was considered. The partial self-compensation + partial reporting seems to be complex. Unless more evidence can be provided, the UE self-compensation and UE reporting IL loss need not be used at the same time. Combine them together may significantly increase the complexity, thus making reporting even less attractive, if some reporting can be introduced.

Proposal 3: Do not consider UE self-compensation and UE reporting IL loss at the same time, if reporting is considered.

Possible simplified solution and WF (Update)

There were a lot of discussions in the last two meetings. However, basically no more progress compared to the ending of Rel-18. There were even no real discussion since companies have significant differences even on the basic understandings.

Observation 4(Update): Basically, no progress was made for the last two meetings.

There was also other simplified solution was raised, such as reporting a single digit to show whether compensation is used or not. However, this over simplified solution has very limited potential of performance gain, since very limited choice is provided to the network to do any optimization work.

Based on this condition, stopping discussions can be considered if no progress can be made.

Proposal 4(Update): If no more agreements can be made on the scheme, this IL loss reporting discussion can be stopped.

Conclusion

In this paper, some considerations for 6Rx SRS IL reporting were discussed, and the following observations and proposals were provided:

Observation 1: Dynamic antenna switching is widely used, and the impact to the perceivable IL has to be considered.

Observation 2: Currently there is no “mandatory compensation” requirement or behaviour defined or implied for UE.

Observation 3: The Tx-Rx imbalance related points:

- Tx-Rx imbalance matters for SRS antenna switching;

- Rx part do have its own “compensation” of imbalance;

- More factors such as antenna performance may also have impact on Tx-Rx imbalance;

- Bring more uncertainty factors in the network.

Based on the previous discussion, a general proposal is suggested:

Proposal 1: Discuss some general understandings may be helpful to make progress, e.g.:

- Widely used Dynamic antenna switching have to be considered.

- Currently there is no “mandatory compensation” requirement or behaviour for UE.

- Tx-Rx imbalance still need further consideration..

Observation 4: Current WID scope and impacted TS/TR doesn’t involve RAN1.

Proposal 2: RAN4 should avoid RAN1 impact as much as possible, and should not conclude a scheme with RAN1 impact without confirmation from RAN/RAN1.

Proposal 3: Do not consider UE self-compensation and UE reporting IL loss at the same time, if reporting is considered.

Observation 4(Update): Basically, no progress was made for the last two meetings.

Proposal 4(Update): If no more agreements can be made on the scheme, this IL loss reporting discussion can be stopped.

- References

[1] RP-240828, New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4, Huawei, RAN#103

[2] R4-2405282, Topic summary for [110bis][129] NR_ENDC_RF_Ph4, Moderator (Huawei, HiSilicon), RAN4 #110bis

[3] R4-2406585, WF on requirements for 6Rx, AT&T, RAN4 #110bis

[4] R4-2404666, Discussion on 6Rx for handheld and FWA UE, vivo, RAN4 #110bis

[5] R4-2303519, [Draft] LS on the UE SRS IL imbalance issue, RAN4#106

[6] R1-2306200, FL summary on UE SRS IL imbalance issue, Moderator (Huawei), RAN1#112bis-e

[7] R4-2321793, [Draft] LS on the UE SRS IL imbalance issue, RAN4#109

[8] R4-2405933, Initial discussion on SRS insertion loss imbalance reporting and 6Rx requirements, Ericsson, RAN4 #110bis

[9] R4-2410751, WF on requirements for 6Rx, AT&T, RAN4 #110bis

[10] R4-2408937, Topic summary for [111][126] NR_ENDC_RF_Ph4_part3, Moderator(AT&T), RAN4 #110bis

[11] R4-2408122, Discussion on SRS IL imbalance reporting, vivo, RAN4#111;

[12] R4-2305082, Discussion on 8Rx UE RF requirements, vivo, RAN4#106bis

[13] R4-2412820, Topic summary for [112][118] NR_ENDC_RF_Ph4_part3, Moderator (AT&T), RAN4#112

[14] R4-2414285, WF on 6Rx UE requirements, AT&T, RAN4#112

[15] R4-2412094, Discussion on SRS IL imbalance reporting, vivo, RAN4#112