3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415788

Hefei, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Feasibility study and analysis for NR NTN HPUE Tx Requirements

Agenda Item:	6.8.2.2.1

Document for:	Approval

Introduction

In RAN4#112, the HPUE for NTN [1] was started. A summary can be referenced in [2] and a WF [3] has been agreed. A paper [4] was submitted including an initial feasibility study and analysis for both NR-based and Iot-based NTN HPUE Tx requirements.

In this paper, further feasibility analysis based on some measurements for NR NTN were collected, and a number of proposals were made based on last meeting’s progress.

Verification and feasibility Analysis

General

An analysis has been provided in [4], in which the following proposal has been provided:

Proposal 1: For NR-NTN, including handheld and non-handheld:

    - Power Class 2 is feasible;

    - Power Class 1.5 is feasible [with possible restrictions on components]

    - Power Class 1 is FFS

In the WF [3], the following WF is provided

Issue 3-2-1: NR-NTN HPUE feasibility for different power classes

- Agreement of Issue 3-2-1 to Issue 3-2-3:

  - For PC2, it is feasible to support both handheld and non-handheld devices.

  - FFS other power classes for applicable device type.

Among the restrictions of the power classes that can be achieved, Power amplifier and duplexer are two main constrains for NR NTN UE to reach higher power.

Observation 1: Power amplifier and duplexer are two main constrains for NR NTN UE to reach higher power.

In order to have more insight on the state-of-the-art technology, we made some testing on a commercial PA that is designed for satellite communication, and also a survey on duplexer. It should be noted those are all based on handheld UE type, which is designed for mobile phones.

PA Testing & Analysis

PA introduction

In order to have an understanding of the state-of-the-art PA performance for satellite communication for handheld UE, we select one commercial PAM designed for satellite communication. This is already used in some mobile phones for TianTong which is operating by China Telecom as [4] introduced. It is noted that this is a different PA compared to what is evaluated for NB-Iot based Iot NTN in [7], although some design targets are similar.

Here are some basic information of the PA and the testing scenarios

- PAM designed for Satellite voice, SMS, and data services application with optimized PAE and harmonic suppression.

- Package Size: Approximate 5 x 5 x 0.8 mm

- Multi-band PA cover:1980M~2010MHz,1608M~1624MHz and 1668M~1675MHz

- Verification using typical PA configurations (VCC etc.)

Testing settings and results

The testing signals are wideband NR signals UL Freq as illustrated. The channel BW used is 20MHz. The detailed the signals are:

- NR FDD DFT-S-OFDM QPSK 20M RB 25@12 SCS 30KHz

- NR FDD DFT-S-OFDM 16QAM 20M RB 50@0 SCS 30KHz

- NR FDD DFT-S-OFDM 64QAM 20M RB 50@0 SCS 30KHz

It is noted that MPR is not considered yet. The EURTA ACLR and UTRA ACLR is measured.

Table 1: ACLR Results (NR FDD DFT-S-OFDM QPSK 20M RB 25@12 SCS 30KHz)

Table 2: ACLR Results (NR FDD DFT-S-OFDM 16QAM 20M RB 50@0 SCS 30KHz)

Table 3. ACLR Results (NR FDD DFT-S-OFDM 64QAM 20M RB 50@0 SCS 30KHz)

From the results, it can be seen that the Pout is ranging from 31~33dBm, and The ACLR conditions are:

- For QPSK, EUTRA ACLR generally larger than 38dB;

- For 16QAM, EUTRA ACLR generally larger than 33dB;

- For 64QAM, EUTRA ACLR generally larger than 32dB;

Analysis

The measured Pout is the output power of the PA and did not consider the insertion loss of front end yet. In the production, we would assume a 5dB IL needed to leave more room for implementation.

Based on this value, the Pout of PA of 31~33dBm corresponds to an expected 26 ~ 28dBm output in the UE antenna port. And this roughly corresponds to the need of PC2 with margin.

Though ACLR is still depending on co-existence study, we can still use the current TN value as reference.

In addition, although for 16QAM/64QAM, the margin for ACLR of 32~33dB seems not sufficient, there is a 2/2.5dB MPR still not counted yet. If we count this MPR, the expected output power in the UE antenna port would go as high as 28~30dBm with the ACLR here, and this should leave sufficient margin for ACLR.

For PC1.5 2Tx, the requirement of any single RF chain is basically the same to PC2, thus PC1.5 should also have no feasibility issue in the PA side for 2Tx case.

Observation 2a: PC2 1Tx and PC1.5 2Tx would have sufficient margin based on the measurement of a satellite PA.

As for higher power classes, it seems that the margin is not that good. E.g. even with 28~30dB output power in UE antenna connector which also considered MPR, this value clearly still not good enough for PC1, and even PC1.5 1Tx if considered, though the gap seems not that large.

One interesting possible architecture is 2Tx PC1, which needs 31 - 3 = 28dBm nominal output power in one RF chain. Although this seems already being satisfied by the 28~30 dBm, the measured ACLR of 32~33dB do not have sufficient margin (only 1~2dB here), thus even 2Tx PC1 seem not ok.

Observation 2b: PC1.5 1Tx and PC1 2Tx do not have enough margin for the measured PA, while PC1 1Tx cannot be achieved yet by current verification.

Admittedly, this is still a rough analysis, based on only one example PA and a few singles, thus the results are not complete. However, this will still have its meanings.

Duplexer Analysis

The duplexer is another main restrict for NR UE RF chain to reach higher power class for FDD band. For NR NTN, duplexer is needed since it always has to guarantee full duplex operation.

For commercial duplexer that designed for handheld UE, currently there are already products for PC3 which have sufficient margins that are almost ready for PC2 use cases. Here we listed two commercial SAW duplexer for n256 and n255 respectively, which data sheet can be reference in [5] and [6]. The main information regarding the input power is:

Table 4. General information of a commercial n256 duplexer for smart phone

Table 5. General information of a commercial n256 duplexer for smart phone

It can be seen that the input power for NR(DFT-s-OFDM) is 30dBm, and NR(CP-OFDM) is 28.5dBm. If a 5dB IL is assumed, then 31 dBm is needed for the input power of duplexer (PA output power) for a 26dBm output in the antenna connector which is for PC2. It can be seen that the duplexer is almost ready to work in the PC2 condition.

With some further offline discussion, it is found that it should not be that difficult to cover this 1dB gap, with the current tolerance and duplexer implementation.

Similar to PA case, for PC1.5 2Tx, the per RF chain implementation is the same to PC2 1Tx, thus duplexer is also almost ready for PC1.5 2Tx for handheld UE.

Observation 3a: The duplexer is almost ready for PC2 1Tx and PC1.5 2Tx architecture for n256/n255 handheld UE.

However, anything beyond 1Tx PC2, such as 2Tx PC1 and 1Tx PC1.5 seems beyond what duplex vendor can do currently for handheld UE.

Observation 3b: PC1 seems beyond what current duplexer vendor can handle even with 2Tx.

Feasibility Summary

In all, with current verification, it seems that for handheld UE:

- PC2 1Tx and PC1.5 2Tx would be feasible based on the current analysis for PA and duplexer;

- PC1.5 1Tx and PC1 2Tx may be achieved by the PA but the margins are not sufficient; and this cannot be achieved by duplexer currently;

- PC1 1Tx may not be achieved by the PA, and the duplexer cannot be achieved.

A brief table is as following:

Observation 4a: A summary of current restriction for handheld UE, and it can be seen duplexer poses more restriction compared to PA:

Table 6. Current restriction summary for handheld UE for NR NTN

|  | PA | Duplexer | Conclusion based on PA&Duplexer |
| --- | --- | --- | --- |
| PC2 1Tx, PC1.5 2Tx | Feasible with sufficient margin | (Almost) feasible | Feasible |
| PC1.5 1Tx, PC1 2Tx | May feasible but insufficient margin | Not that feasible | Not that feasible, can also be FFS. |
| PC1 1Tx | May not feasible | Not feasible | Not feasible. |

In addition, it should be noted that PC1.5 is useful for NR NTN, since it is currently the highest power class can be achieved.

For non-handheld UE, theoretically any PC is possible since we all have precedence currently in the spec, including PC1. However, there may still need discussion on the requirements that needed. However, we still need to discuss whether PC1.5 1Tx and PC1 2Tx would be needed to be considered in the requirements.

Observation 4b: For non-handheld UE, all the current power classes could be feasible, based on similar assumption with previous discussion in TN.

Based on previous analysis, the following proposals were provided.

Proposal 1: Confirm the feasibility of PC2 1Tx and PC1.5 2Tx for handheld and non-handheld UE for NR NTN.

Proposal 2: Confirm the feasibility of PC1.5 2Tx and PC1 1Tx for non-handheld UE. Discuss whether PC1.5 1Tx and PC1 2Tx are needed or not.

In addition, considering a higher power is beneficial, and PC1.5 is the highest power class achievable by handheld UE for NR-NTN (PC1 is difficult even for 2Tx). It is proposed to keep this power class for NR-NTN, as long as it is feasible.

Proposal 3: Keep the PC1.5 with 2Tx at least for NR-NTN, even though this power class is not defined for FDD in TN yet.

HPUE Impact for Tx Requirements

Duty cycle Related

In TN network, duty cycle related solution is widely used in TDD system, in which some types of power class fall back would be enforced if certain duty cycle would be exceeded. However, for FDD HPUE, no duty cycle solutions have been defined, partly because this concept does not quite adaptive with FDD, and partly may also because duty cycle has some its internal problems.

However, for NTN system, since keeping the UE power might be more important compared to TN because of the tight link budget, the duty cycle solution is raised again by several companies. And the WF is as following:

Issue 3-3-4: SAR for handheld

Agreement:

- Using P-MPR as the starting point

- The other solutions are not precluded

  - E.g., the solution similar to TN duty cycle based on UE capability and network scheduling

In [4], originally only P-MPR is considered for simplicity. After some study, it is felt that the solution similar to TN duty cycle may also be useful.

Although the duty cycle concept is not quite aligned with FDD, the duty cycle method currently defined is actually quite adaptive. The key part actually did not use the wording “duty cycle”, but “the percentage of uplink symbols transmitted in a certain evaluation period” which actually can be adaptable to both FDD/TDD.

Observation 5: The wording of current duty cycle-based SAR solution is quite adaptive and can also be applied to FDD. This may also be useful for NR NTN.

Proposal 4: Consider applying the current duty cycle-based SAR solution to NR NTN.

Others

For other requirements, most of them would be impacted by ACLR, which would be dependent on UE co-existence study, so the discussion was postponed in this paper.

Conclusion

In this paper, further feasibility analysis based on some measurements were collected, and a number of proposals were made based on last meeting’s progress.

Feasibility - General

Observation 1: Power amplifier and duplexer are two main constrains for NR NTN UE to reach higher power.

PA testing & Analysis

Observation 2a: PC2 1Tx and PC1.5 2Tx would have sufficient margin based on the measurement of a satellite PA.

Observation 2b: PC1.5 1Tx and PC1 2Tx do not have enough margin for the measured PA, while PC1 1Tx cannot be achieved yet by current verification.

Duplexer Analysis

Observation 3a: The duplexer is almost ready for PC2 1Tx and PC1.5 2Tx architecture for n256/n255 handheld UE.

Observation 3b: PC1 seems beyond what current duplexer vendor can handle even with 2Tx.

Feasibility summary

Observation 4a: A summary of current restriction for handheld UE, and it can be seen duplexer poses more restriction compared to PA.

Table 6. Current restriction summary for handheld UE for NR NTN

|  | PA | Duplexer | Conclusion based on PA&Duplexer |
| --- | --- | --- | --- |
| PC2 1Tx, PC1.5 2Tx | Feasible with sufficient margin | (Almost) feasible | Feasible |
| PC1.5 1Tx, PC1 2Tx | May feasible but insufficient margin | Not that feasible | Not that feasible, can also be FFS. |
| PC1 1Tx | May not feasible | Not feasible | Not feasible. |

Observation 4b: For non-handheld UE, all the current power classes could be feasible, based on similar assumption with previous discussion in TN.

Based on previous analysis, the following proposals were provided.

Proposal 1: Confirm the feasibility of PC2 1Tx and PC1.5 2Tx for handheld and non-handheld UE for NR NTN.

Proposal 2: Confirm the feasibility of PC1.5 2Tx and PC1 1Tx for non-handheld UE. Discuss whether PC1.5 1Tx and PC1 2Tx are needed or not.

Proposal 3: Keep the PC1.5 with 2Tx at least for NR-NTN, even though this power class is not defined for FDD in TN yet.

Duty cycle

Observation 5: The wording of current duty cycle-based SAR solution is quite adaptive and can also be applied to FDD. This may also be useful for NR NTN.

Proposal 4: Consider applying the current duty cycle-based SAR solution to NR NTN.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412099, Feasibility study and Initial analysis for NTN HPUE RF Tx Requirements, vivo, RAN4#112

[5] https://www.murata.com/products/productdata/8825373098014/DS-SAYRZ2G00BA0E0A.pdf

[6] https://www.murata.com/en-eu/products/productdata/8820841873438/DS-SAYRZ1G54BA0C0A.pdf

[7] R4-2415790, Feasibility study and analysis for Iot NTN HPUE Tx Requirements, vivo, RAN4#112bis