3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415791

Hefei, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Analysis for Iot NTN HPUE Rx Requirements

Agenda Item:	6.8.2.3.2

Document for:	Approval

Introduction

In RAN4#112, the HPUE for NTN [1] was started. A summary can be referenced in [2] and a WF [3] has been agreed. In it, the key agreements:

Issue 4-1: General considerations

- Agreement

  - Rx RF requirements including Max input level, ACS, blocking characteristics, spurious response, intermodulation characteristics, Rx spurious emissions in existing spec can be applied to other power class.

…

Issue 4-3: RSD for IoT-NTN

- Agreement:

  - For NB-Iot based Iot-NTN, no RSD needed for HPUE

  - For eMTC based Iot-NTN with half duplex, no RSD needed for HPUE

  - For eMTC based Iot-NTN with full duplex, further discuss RSD impact

A paper [4] was submitted for including an initial study on NTN HPUE RF Rx Requirements for both NR-based and Iot-based NTN HPUE Tx requirements, in which some topics were not really discussed.

In this paper, a discussion of the impact of HPUE to RSD requirements for Iot NTN are provided.

HPUE Impact for Iot-NTN RSD

General

As already agreed, for all the half duplex mode cases, no RSD is needed. Only for the case of eMTC based Iot NTN with full duplex, RSD is needed to be discussed.

For this particular case of eMTC based Iot NTN with full duplex, actual the guidelines should be the same to NR-NTN, since the conditions are basically the same apart from the Channel BW.

Proposal 1: For full duplex eMTC based Iot-NTN, re-use the principles to specify NR-NTN RSD.

PC2

For PC2, an analysis has been provided in [6] for NR-NTN RSD analysis, and two proposals are provided:

Proposal 1: For NR-NTN power class 2,

- For n256 and n254, RSD = 0dB, for both 2Tx and 1Tx cases.

- For n255, RSD is FFS

Proposal 2: For power class 2, there is no differentiation of handheld and non-handheld UE.

It is believed that the basic principles to derive requirements are the same, and these two proposals can be re-used except for minor difference. E.g. if in the future non-zero RSD is considered, different channel bandwidth should be considered.

In addition, for the differentiation of handheld and non-handheld, it is also based on the proposal as discussed in [5]:

Proposal 2: Iot NTN also can be differentiated between handheld and non-handheld devices.

Furthermore, since it has been proposed in [5] that only 1Tx should be considered for Iot NTN, the requirements for Iot NTN should only consider 1Tx.

Proposal 2: For Iot NTN requirements, only 1Tx case be considered.

Based on the previous proposals, the following RSD value for different bands and accompanying information is as following:

Proposal 3: For PC2 full duplex eMTC based Iot-NTN,

- For n256 and n254, RSD = 0dB

- For n255, RSD is FFS

- No differentiation of handheld and non-handheld UE

PC1

It is noted that for full duplex eMTC based Iot-NTN, the PC1 analysis is also similar to NR-NTN. All the main observations and proposals can be reused from [6].

In order to reduce redundancy, only the key observations and proposals were kept here, and more detailed contents can reference from [6].

Observation 1: Currently PC1 is only for non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

Observation 2: For PC1, according to de-sense analysis in 37.880, it appears that the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands.

Proposal 4: For PC1 1Tx for full duplex eMTC based Iot-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. Whether and how to make it clear in the spec that this is for non-handheld only is FFS.

Conclusion

In this paper, the RSD requirements have been discussed for NR NTN HPUE.

General

Proposal 1: For full duplex eMTC based Iot-NTN, re-use the principles to specify NR-NTN RSD.

PC2

Proposal 2: For Iot NTN requirements, only 1Tx case be considered.

Proposal 3: For PC2 full duplex eMTC based Iot-NTN,

- For n256 and n254, RSD = 0dB

- For n255, RSD is FFS

- No differentiation of handheld and non-handheld UE

PC1

Observation 1: Currently PC1 is only for non-handheld case in LTE and NR TN, and typically share PC3 Rx requirements without any RSD.

Observation 2: For PC1, according to de-sense analysis in 37.880, it appears that the current NTN band n254/n255/n256 are easier compared to legacy PC1 bands.

Proposal 4: For PC1 1Tx for full duplex eMTC based Iot-NTN, which is only considered feasible for non-handheld, the RSD is proposed to be 0dB. Whether and how to make it clear in the spec that this is for non-handheld only is FFS.

- References

[1] RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

[2] R4-2412829, Topic summary for [112][127] NR_IoT_NTN_HPUE , Samsung, RAN4#112

[3] R4-2414452, WF on UE RF requirements for NTN HPUE, Samsung, RAN4#112

[4] R4-2412100, Initial analysis for NTN HPUE RF Rx Requirements, vivo, RAN4#112

[5] R4-2415790, Feasibility study and analysis for Iot NTN HPUE Tx Requirements, vivo, RAN4#112bis

[6] R4-2415789, Analysis for NR NTN HPUE Rx Requirements, vivo, RAN4#112bis

[7] TR 37.880, High-power UE operation for fixed-wireless/vehicle-mounted use cases in Band 12, Band 5, and Band n71