3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415792

Hefei, China, 14th – 18th October, 2024

Source: 	vivo

Title: 	Further discussion of UE RF impact for NR MIMO phase 5

Agenda Item:	6.19.2

Document for:	Approval

Introduction

In RAN#102, a new WID [1] was approved on NR MIMO Phase 5. There were already three RAN1 meetings and significant progress have been made, and this is the first meeting for RAN4 to discuss this.

In RAN4#112, an initial analysis and study [2] were made for the WI, and a WF [3] has been agreed. In RAN#105, the NR MIMO phase 5 WID has been revised to [4] to add some new scope.

In this paper, further discussion was provided based on last meeting’s WF. In addition, a brief analysis on the latest UE RF impact.

Discussion

3Tx RF Impact

General

3Tx RF impact is by far have the most RF impact, and a number of issues were discussed in the WF [3].

Power classes / UE types

Power class is a very basic assumption in UE requirements definition. Although the differentiation of different power class may introduce a lot of complex fallback behaviours, it seems a restriction of different power classes is still needed since some basic requirements such as MOP/MPR would still be based on a certain power class and designated power class. However, this kind of restriction may be kept implicitly.

Take Rel-18 4Tx as an example, FWA type and a power class of 1.5 was assumed for 4Tx, thus the MPR and Pcmax tolerance for 4Tx was only defined based on the assumption of PC1.5 and FWA in the specification. Though this does not preclude a 4Tx UE to support PC2 or PC3, there is actually no dedicated PC2 or PC3 requirements for 4Tx, and this can be taken as reference.

Observation 1: Supported power classes still need to be discussed for 3Tx codebook, since this is the basic reference for some requirements such as MOP/MPR.

Observation 2: For Rel-18 4Tx case, only PC1.5 is assumed, and MPR and Tolerance requirements are defined based on this assumption.

For 4Tx, this means that no dedicated requirements for 4Tx with PC2/PC3 were defined, since these implementations were deemed as not likely.

For 3tx codebook transmission, the following issue was kept open in the WF:

Issue 2-3: Power classes and PA configuration for 3Tx UL MIMO

- Proposals

  - Option 1: PC3 and PC2

  - Option 2: PC1.5 only

  - Option 3: Others

- Way forward

  - Companies are encouraged to provide clear view on 3Tx power class and its PA assumptions

Power Class 3:

This is the default power classed for all the cases including single carrier /UL-MIMO/CA etc. This power class is supported by all the bands, although no direct restriction on how to No matter how many Tx

It was pointed out that it is a bit too conservative for a 3Tx UE to support power class 3 if implemented, since the 23dBm PA is already the “minimum” expected in any Tx chain. In this sense, PC3 may not a typical implementation for 3Tx and a dedicated requirements definition may not be necessary.

Observation 3: Power class 3 for 3Tx is possible but not typical, thus dedicated requirements for PC3 may not be needed.

Power Class 2:

This is one possible and typical power class for 3Tx. Although one may argue that 2Tx UL-MIMO is already enough to support PC2, this is still by far the most typical High power that was deployed.

If handheld is also proposed to be considered, power class 2 is suggested to be paid more attention.

Observation 4: Power class 2 is still the most basic high power UE, even for 3Tx.

Power Class 1.5:

This is first introduced considered for FWA cases, although this restriction was eased somewhat in later stages. Currently there are consideration for PC1.5 for both 2Tx and 4Tx, so it seems that 3Tx may also consider this power class.

However, since 3Tx would consume more Tx chain, this has more challenges for handheld. If combined with PC1.5, more technical challenges would exist.

Observation 5: PC1.5 can also be considered for 3Tx, however, this may have more challenges for handheld.

Based on the analysis, the following proposal is provided;

Proposal 1: For the power classes for 3Tx UL-MIMO (not including full Tx power):

- PC3: Can be supported but not typical, dedicated requirements not needed.

- PC2: Supported and typical, particularly for handheld. Requirements needed.

- PC1.5: Can be considered for non-handheld type only. Requirements can be considered.

In addition, for full tx power, it was pointed out in [2] that the mode 0 is considered. In this sense, even only 2Tx is considered, a 26+26+26 architecture would be assumed. This would bring more challenges for UE design and implementation, and should not be considered for handheld type.

Proposal 2: For the case of full power mode 0, UE type should be restricted to non-handheld only for 3Tx PC2.

For the layer number need to be verified, the following WF was agreed:

Issue 2-6: ULFPTx mode 0

- Proposals

  - Option 1: Add both 1-layer and 2-layer for 3Tx with

  - Option 2: Add 1-layer for 3Tx with W=13100,13010,13001

- Tentative agreement

  - Take 1-layer as baseline

  - Further discuss whether 2-layer is needed

In our understanding , 2-layer verification is not that essential, since the test burden is already considerable. So we keep our proposal of option 2.

Proposal 3: Do not include 2-layer verification for ULFPTx mode 0.

New Scopes in RAN#105

In RAN#105, the NR MIMO phase 5 WID has been revised to [4] to add some new scope. In this section we provide some analysis:

SRS port grouping

This scope is targeting to have SRS port grouping. It could be used for the case that different baseband chips were used to ease the processing pressure. This is a purely baseband feature, and have no UE RF requirements.

This area has no impact on UE RF.

3T6R Related

3T6R SRS antenna switching was officially added. This is already being considered in Rel-19 UE RF WIs thus also do not have to be considered here.

It should be noted that “3T3R” here does not mean a new 3Rx UE requirements would be needed, and this is just a case that similar to 2T2R compared to 2T4R case.

This area has no impact on UE RF.

Two TAs

This has removed some previous restriction in Rel-18. This revision also have no impact to UE RF requirements.

Observation 6: New RAN1 scopes agreed in RAN#105 do not have UE RF impact need to be considered in this WI.

Proposal 4: Do not consider new RAN1 scopes agreed in RAN#105 in UE RF impact analysis.

Asymmetric DL sTRP/UL mTRP

In RAN4#112, a paper [R4-2413224] discussed a verification of mTRP case scenario 2 as following table (referenced from [R4-2413224])

| (FR2 asymmetric scenarios for Rel-19) |  | Tx capability of UL-specific TRP |  |
| --- | --- | --- | --- |
|  |  | Can transmit SSB | No transmitter |
| UE UL TCI-state | STxMP Simultaneous transmission with multiple TCI states | ‘Scenario 1’ | ‘Scenario 3’ |
|  | Single UL TRP can be same or different from DL TRP | ‘Scenario 2’ | ‘Scenario 4’ |

Currently no WF has been reached.

Issue 3-1: FR2 UL enhancement scenario

- FFS the following proposal (companies to check):

  - Option 1: UL TCI-state differs from DL TCI-state

  - Other options not precluded

For FR2, current spherical coverage and EIRP, the performance is already set once the number and position of modules are set. In this sense, the verification of of m-TRP scenarios are more likely to be functional tests.

Observation 7: The verification of m-TRP scenarios are more likely to be functional tests.

Based on this situation, it is proposed that more analysis is needed for m-TRP issue.

Proposal 5: Make more analysis on m-TRP issue.

Conclusion

In this paper, more study on UE RF impact of MIMO phase 5 is provided:

3Tx RF Impact

Observation 1: Supported power classes still need to be discussed for 3Tx codebook, since this is the basic reference for some requirements such as MOP/MPR.

Observation 2: For Rel-18 4Tx case, only PC1.5 is assumed, and MPR and Tolerance requirements are defined based on this assumption.

Observation 3: Power class 3 for 3Tx is possible but not typical, thus dedicated requirements for PC3 may not be needed.

Observation 4: Power class 2 is still the most basic high power UE, even for 3Tx.

Observation 5: PC1.5 can also be considered for 3Tx, however, this may have more challenges for handheld.

Proposal 1: For the power classes for 3Tx UL-MIMO (not including full Tx power):

- PC3: Can be supported but not typical, dedicated requirements not needed.

- PC2: Supported and typical, particularly for handheld. Requirements needed.

- PC1.5: Can be considered for non-handheld type only. Requirements can be considered.

New Scopes in RAN#105

Observation 6: New RAN1 scopes agreed in RAN#105 do not have UE RF impact need to be considered in this WI.

Proposal 4: Do not consider new RAN1 scopes agreed in RAN#105 in UE RF impact analysis.

Asymmetric DL sTRP/UL mTRP

Observation 7: The verification of m-TRP scenarios are more likely to be functional tests.

Proposal 5: Make more analysis on m-TRP issue.

- References

[1] RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator), RAN#102

[2] R4-2412095, Discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#112

[3] R4-2414310, WF on UE RF requirements for Rel-19 MIMO enhancement, Samsung, RAN4#112

[4] RP-242394, WID revision: NR MIMO Phase 5, RAN#105

[5] R4-2413224, On asymmetric connection scenarios for NR MIMO Phase 5, Qualcomm, RAN4#112