# 3GPP TSG- RAN WG4 Meeting # 112bis	R4-2415813

# Hefei, China, 14th–18th Oct, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.108 | CR | - | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR to TS36.108 Introduction of IoT-NTN S band |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | IoT_NTN_FDD_S_band-Core |  |  |  |  |  | Date: |  |  | 2024-10-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of IoT-NTN S band. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Relevant sections for IoT-NTN S band are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirements for IoT-NTN S band are not defined. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2, 5.4A.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# << Start of change >>

5.2	Operating bands

EUTRA SAN is designed to operate in the operating bands defined in Table 5.2-1.

Table 5.2-1 E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band SAN receive UE transmit |  |  | Downlink (DL) operating band SAN transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 256 | 1980 MHz | – | 2010 MHz | 2170 MHz | – | 2200 MHz | FDD |
| 255 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 254 | 1610 MHz | - | 1626.5 MHz | 2483.5 MHz | - | 2500 MHz | FDD |
| 253 | 1668 MHz | - | 1675 MHz | 1518 MHz | - | 1525 MHz | FDD |
| 252 | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |
| NOTE: Satellite bands are numbered in descending order from 256. |  |  |  |  |  |  |  |

# << Next change >>

5.4A.2	Channel raster, carrier frequency and EARFCN

The global frequency raster is defined for all frequencies. The granularity of the global frequency raster is 100 kHz, which means that the carrier centre frequency must be an integer multiple of 100 kHz. For each operating band, a subset of frequencies from the global frequency raster are applicable and forms a channel raster with a granularity ΔFRaster.

The carrier frequency in the uplink and downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143. The relation between EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in Table 5.4A.2-1 and NDL is the downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between EARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in Table 5.4.2-1 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

The applicable channel raster and EARFCNs for each operating band are specified in Table 5.4A.2-1.

For operating bands with a channel raster of 100 kHz, every EARFCN within the operating band shall be applicable for the channel raster, and the step size for the channel raster in Table 5.4A.2-1 is given as <1>. The broadcast parameter earfcn-LSB defined in TS 36.331 [9] may be used to assist the UE in synchronizing to the cell.

Table 5.4A.2-1: E-UTRA channel numbers

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| 256 | 100 | 2170 | 229076 | 229076 –<1>- 229375 | 1980 | 261844 | 261844 –<1>- 262143 |
| 255 | 100 | 1525 | 228736 | 228736 –<1>- 229075 | 1626.5 | 261504 | 261504 –<1>- 261843 |
| 254 | 100 | 2483.5 | 228571 | 228571 –<1>- 228735 | 1610 | 261339 | 261339 –<1>- 261503 |
| 253 | 100 | 1518 | 228501 | 228501 –<1>- 228570 | 1668 | 261269 | 261269 –<1>- 261338 |
| 252 | 100 | 2180 | 228301 | 228301 –<1>- 228500 | 2000 | 261069 | 261069 –<1>- 261268 |
| NOTE: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

# << End change >>