# 3GPP TSG- RAN WG4 Meeting # 112bis	R4-2415818

# Hefei, China, 14th–18th Oct, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for inter-band NR CA/DC with 3MHz CBW

# Agenda item:	6.4.2

# Document for:	Approval

# 1 Introduction

In RAN#103 meeting, a new Work Item [1] was approved on NR channel BW less than 5MHz for FR1 Phase 2 with the following RAN4 objective. In this contribution, we want to share some further views on UE RF requirements for inter-band NR CA/DC with 3MHz CBW.

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

# 2 Discussion

- Issue 2-2: UE capability signaling - Options for further discussion - Option 1: Request RAN2 to introduce UE capability signaling to enable less than 5MHz CBW operation for CA/DC - The following Rel-18 capabilities for NR less than 5MHz CBW operation need to be extended or new capabilities defined to enable CA/DC operation scenario with one or more component carriers with less than 5MHz CBW - [support12PRB-CORESET0-r18] - [support3MHz-ChannelBW-Asymmetric-r18] - [support3MHz-ChannelBW-Symmetric-r18] - [support5MHz-ChannelBW-20PRB-CORESET0-r18] - [support12PRB-CORESET0-GSCN-41637-r18] - UE capability signalling granularity is FFS - The respective UE capabilities shall be early-implementable from Rel-18 - The details including whether to update existing Rel-18 capabilities or introduce new Rel-19 capabilities are up to RAN2 - Option 2: Do not introduce new UE capability signaling to enable less than 5MHz CBW operation for CA/DC - RAN4 to define BCSs for band combination CA_n100A-n101A or combinations with other band where 3 MHz is supported as usual where 3MHz / 5MHz channel bandwidths are listed for each band. BCS5 with supportedMinBandwidthUL-r17 set to 5 MHz can be used to indicate that a UE supports CA channel BWs other than 3 MHz for a given band. - RAN2 specs need to remove “This feature is only applicable to single-carrier operation from Rel-19. RAN2 to confirm no NBC issues. - No new RAN2 UE capability signaling is required.” - Other options are not precluded

The above is WF in last meeting [2], we didn’t reach an agreement on UE capability signaling. According to [3] and TS 38.306, the following UE features were introduced to support less than 5 MHz channel bandwidth in Rel-18, which are shown in Table 1.

- support12PRB-CORESET0-r18

- support3MHz-ChannelBW-Asymmetric-r18

- support3MHz-ChannelBW-Symmetric-r18

- support5MHz-ChannelBW-20PRB-CORESET0-r18

- support12PRB-CORESET0-GSCN-41637-r18

The former three features are all per band and optional with capability signaling and the latter two features are per UE. In addition, it specified that these features are only applicable to single-carrier operation. From the UE capability signaling granularity perspective, per band features may be not feasible for CA case of 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band. Hence, per band combination UE features are needed to support less than 5MHz CBW operation for CA/DC. Whether to update existing Rel-18 capabilities or introduce new Rel-19 capabilities are up to RAN2. It’s better to send a LS to RAN2.

Proposal 1: Per band combination UE features are needed to support less than 5MHz CBW operation for CA/DC. Proposal 2: Whether to update existing Rel-18 capabilities or introduce new Rel-19 capabilities are up to RAN2 and send a LS to RAN2.

Table 1: UE features related to less than 5 MHz channel bandwidth in Rel-18

| Definitions for parameters | Per | M | FDD- TDD DIFF | FR1- FR2 DIFF |
| --- | --- | --- | --- | --- |
| support12PRB-CORESET0-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block that is located according to Table 5.4.3.1-2 in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of support-3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15kHz SCS only. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 12 PRB BWP operation. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |
| support5MHz-ChannelBW-20PRB-CORESET0-r18 Indicates whether the UE supports short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS, and the reception of 20 PRB CORESET0. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41638 of Table 5.4.3.1-3 in TS 38.101-1 [2]. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this feature supports configuration of 20 PRB BWP operation. | UE | No | FDD only | FR1 only |
| support12PRB-CORESET0-GSCN-41637-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block located at GSCN 41637. A UE supporting this feature shall also indicate support of support3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 [2]. NOTE: The UE supporting this FG supports configuration of 12 PRB BWP operation. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. | UE | No | FDD only | FR1 only |

# 3 Conclusion

In this contribution, we shared some views on power boosting and/or MPR reduction and the proposals are made as follows:

Proposal 1: Per band combination UE features are needed to support less than 5MHz CBW operation for CA/DC. Proposal 2: Whether to update existing Rel-18 capabilities or introduce new Rel-19 capabilities are up to RAN2 and send a LS to RAN2.

# 4 Reference

RP-240832, New WID: WI resulting from discussion on NR channel BW less than 5MHz for TN, vivo

R4-2414451, WF on UE RF requirements for Rel-19 NR channel BW less than 5MHz for FR1 Phase 2, Intel

R4-2312572, Updated RAN1 UE features list for Rel-18 NR after RAN1 #117, AT&T, NTT DOCOMO, INC.