# 3GPP TSG- RAN WG4 Meeting # 112bis	R4-2415824

# Hefei, China, 14th–18th Oct, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NR NTN Ku band

# Agenda item:	6.9.4

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about NR NTN Ku band from UE RF requirements perspective.

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

# 2 Discussion

  - 2.1 Regulatory references

- Issue 1-1-1: Supported VSAT Types in the NR NTN Ku band work - The current NTN VSAT types in 38.101-5 are considered in this Ku band work item - Both mechanical and electrically steered antennas can be considered - The discussion on down selecting between FR1/FR2 approaches for the NTN Ku band will not be impacted by this WF. - Issue 1-1-2: Open issues to be discussed - Companies are encourage to provide the following information for the NTN Ku band in the next meeting: - Equivalent antenna aperture for the Ku band VSAT - Aspects on whether the common Tx/Rx antenna or separate Tx/Rx antenna can be assumed for VSAT in Ku band - Parameterized array antenna model for electronic steering antenna (the table 8.1.1-1 in TR 38.921 can be referred as an example format) - The range of TRP/EIRP for the Ku band - The UL/DL budget - Regulatory requirements

The above is way forward in last meeting, we reached an agreement on NTN VSAT types, and no agreement on UE RF requirements. During Ka band discussion, some regulatory references like ETSI EN specs and ECC specs are used as the reference for the implementation and conformance of Satellite Earth Stations, both fixed and in motion. In the same way, we list the following EN specs relevant to NTN Ku band with downlink 10.70–12.75 GHz and uplink 14.0–14.5 GHz. It should be noted that these EN specs specifies some technical requirements such as off-axis EIRP emission density, on-axis spurious requirement, off-axis spurious requirement, antenna pointing accuracy and performance, and receiver antenna off-axis performance, which are similar as Ka band.

| Spec Number and Version | Title | Comment |
| --- | --- | --- |
| EN 302 340 | Satellite Earth Stations and Systems (SES); Harmonised Standard for satellite Earth Stations on board Vessels (ESVs) operating in the 11/12/14 GHz frequency bands allocated to the Fixed Satellite Service (FSS) covering the essential requirements of article 3.2 of the Directive 2014/53/EU | 11.7-12.5 Broadcasting (satellite) Apply to Earth Stations located on board Vessels (ESVs), which transmit in 14.0-14.5GHz and receive in 10.7-12.75GHz |
| EN 303 980 | Satellite Earth Stations and Systems (SES);Harmonised Standard for fixed and in-motion Earth Stations communicating with non-geostationary satellite systems (NEST) in the 11 GHz to 14 GHz frequency bands covering essential requirements of article 3.2 of Directive 2014/53/EU | Specify technical characteristics and methods of measurements for fixed and in-motion Earth Stations communicating with non-geostationary satellite systems (NEST), which transmit in 14.0-14.5GHz and receive in 10.7-12.75GHz |
| EN 303 981 | Satellite Earth Stations and Systems (SES);Fixed and in-motion Wide Band Earth Stations communicating with non-geostationary satellite systems (WBES) in the 11 GHz to 14 GHz frequency bands; Harmonised Standard for access to radio spectrum | Specify technical characteristics and methods of measurements for fixed and in-motion Earth Stations communicating with non-geostationary satellite systems (WBES), which transmit in 14.0-14.5GHz and receive in 10.7-12.75GHz |
| EN 302 977 | Satellite Earth Stations and Systems (SES);Harmonised Standard for Vehicle-Mounted Earth Stations (VMES) operating in the 14/12 GHz frequency bands covering the essential requirements of article 3.2 of the Directive 2014/53/EU | Apply to Earth Stations located on board Vehicles. VMES transmit in 14.0-14.5GHz and receive in 10.7-12.75GHz |

Observation 1: The following EN specs relevant to NTN Ku band with downlink 10.70–12.75 GHz and uplink 14.0–14.5 GHz can be used as the reference, which specify similar requirements as Ka band including off-axis EIRP emission density, on-axis/off-axis spurious requirement, antenna pointing accuracy and performance, and receiver antenna off-axis performance: EN 302 340, EN 303 980, EN 303 981, EN 302 977.

  - 2.2 Off-axis EIRP emission density limit within the operating band

In last meeting, the agreement that define as a first Ku band for Region 1 (CEPT countries): 10.7–12.75 GHz (S-to-E) and 14.0–14.5 GHz (E-to-S) was reached. For Ka band n512, which is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01, off-axis EIRP density limit requirement is derived from EN 301 459 and EN 303 978 for fixed VSAT and mobile VSAT respectively [2].

For Ku band, EN 302 977 and EN 302 340 all define off-axis EIRP emission density for Ku band in 14.0–14.5 GHz as follows. It needs to redefine off-axis EIRP density limits for Ku band in UL 14.0–14.5 GHz based on EN 302 977 and EN 302 340.

Proposal 1: It needs to redefine off-axis EIRP density limits for Ku band in UL 14.0–14.5 GHz based on EN 302 977 and EN 302 340.

  - 2.3 On-axis spurious requirement and off-axis spurious requirement

For Ka band, on-axis spurious and off-axis spurious requirements are evaluated from EN 301 978, EN 301 360 and EN 301 459 [3], which convert dBpW values in dBm (=dBpW – 90) . Look at EN 302 340, EN 303 980, EN 303 981, EN 302 977, which also define the following on-axis spurious requirement and off-axis spurious requirement for Ku band in UL 14.0–14.5 GHz.

On-axis spurious requirement

For on-axis spurious limits in “Emissions disabled” and “Carrier-off” states, EN 302 340, EN 303 980, EN 303 981 and EN 302 977 specify the same value for Ku band in UL 14.0–14.5 GHz as Ka band.

Observation 2: For on-axis spurious limits in “Emissions disabled” and “Carrier-off” states, EN 302 340, EN 303 980, EN 303 981 and EN 302 977 specify the same value for Ku band in UL 14.0–14.5 GHz as Ka band.

For on-axis spurious limits in “Carrier-on” state, EN 302 977 also give the requirement with dBW in any 100 kHz for Ku band in UL 14.0–14.5 GHz, which can be converted to the same value with dBm in 1 MHz as Ka band.

Observation 3: For on-axis spurious limits in “Carrier-on” state, EN 302 977 also give the same requirement for Ku band in UL 14.0–14.5 GHz as Ka band.

Proposal 2: It is proposed to specify the same on-axis spurious requirement for Ku band in UL 14.0–14.5 GHz as Ka band.

Off-axis spurious requirement

For radiated field strength limits at 10m from the NTN VSAT, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is same as Ka band in TS 38.101-5.

Observation 4: For radiated field strength limits at 10m from the NTN VSAT, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is same as Ka band in TS 38.101-5.

For off-axis spurious limits in “Emissions disabled” state, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is also same as Ka band in TS 38.101-5.

Observation 5: For off-axis spurious limits in “Emissions disabled” state, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is also same as Ka band in TS 38.101-5.

Proposal 3: For general and off-axis spurious limits in “Emissions disabled” state, it is proposed to specify the same requirement for Ku band in UL 14.0–14.5 GHz as Ka band.

For off-axis spurious limits in “Carrier-on” and “Carrier-off” state, EN 303 980, EN 303 981 and EN 302 977 define a little different requirements for Ku band in UL 14.0–14.5 GHz.

EN 303 980, EN 303 981

EN 302 977

Observation 6: For off-axis spurious limits in “Carrier-on” and “Carrier-off” state, EN 303 980, EN 303 981 and EN 302 977 define a little different requirements for Ku band in UL 14.0–14.5 GHz.

Proposal 4: We need to consider how to specify off-axis spurious limits in “Emissions disabled” state for Ku band in UL 14.0–14.5 GHz based on EN 303 980, EN 303 981 and EN 302 977.

  - 2.4 Antenna pointing accuracy

In EN 303 980 and EN 303 981, they only describe the purpose of antenna pointing accuracy without any specification as follows.

Observation 7: In EN 303 980 and EN 303 981, they only describe the purpose of antenna pointing accuracy without any specification.

Proposal 5: We need to consider whether to specify antenna pointing accuracy for Ku band in UL 14.0–14.5 GHz.

  - 2.5 Receiver antenna off-axis performance

For receiver antenna off-axis performance, EN 302 340, EN 303 980, EN 303 981 and EN 302 977 specify different requirements for different VSATs. Receiver antenna off-axis performance in EN 303 977 and EN 302 340 is same as the requirement for Ka band in TS 38.101-5.

EN 303 980

EN 303 981

EN 303 977 and EN 302 340

Observation 8: For receiver antenna off-axis performance, EN 302 340, EN 303 980, EN 303 981 and EN 302 977 specify different requirements for different VSATs. Receiver antenna off-axis performance in EN 303 977 and EN 302 340 is same as the requirement for Ka band in TS 38.101-5.

Proposal 6: We need to consider whether to redefine receiver antenna off-axis performance for Ku band in UL 14.0–14.5 GHz.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN Ku band and the proposals are made as follows:

Observation 1: The following EN specs relevant to NTN Ku band with downlink 10.70–12.75 GHz and uplink 14.0–14.5 GHz can be used as the reference, which specify similar requirements as Ka band including off-axis EIRP emission density, on-axis/off-axis spurious requirement, antenna pointing accuracy and performance, and receiver antenna off-axis performance: EN 302 340, EN 303 980, EN 303 981, EN 302 977.

Proposal 1: It needs to redefine off-axis EIRP density limits for Ku band in UL 14.0–14.5 GHz based on EN 302 977 and EN 302 340.

Observation 2: For on-axis spurious limits in “Emissions disabled” and “Carrier-off” states, EN 302 340, EN 303 980, EN 303 981 and EN 302 977 specify the same value for Ku band in UL 14.0–14.5 GHz as Ka band.

Observation 3: For on-axis spurious limits in “Carrier-on” state, EN 302 977 also give the same requirement for Ku band in UL 14.0–14.5 GHz as Ka band.

Proposal 2: It is proposed to specify the same on-axis spurious requirement for Ku band in UL 14.0–14.5 GHz as Ka band.

Observation 4: For radiated field strength limits at 10m from the NTN VSAT, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is same as Ka band in TS 38.101-5.

Observation 5: For off-axis spurious limits in “Emissions disabled” state, the requirements specified in EN 303 980, EN 303 981 and EN 302 977 for Ku band in UL 14.0–14.5 GHz is also same as Ka band in TS 38.101-5.

Proposal 3: For general and off-axis spurious limits in “Emissions disabled” state, it is proposed to specify the same requirement for Ku band in UL 14.0–14.5 GHz as Ka band.

Observation 6: For off-axis spurious limits in “Carrier-on” and “Carrier-off” state, EN 303 980, EN 303 981 and EN 302 977 define a little different requirements for Ku band in UL 14.0–14.5 GHz.

Proposal 4: We need to consider how to specify off-axis spurious limits in “Emissions disabled” state for Ku band in UL 14.0–14.5 GHz based on EN 303 980, EN 303 981 and EN 302 977.

Observation 7: In EN 303 980 and EN 303 981, they only describe the purpose of antenna pointing accuracy without any specification.

Proposal 5: We need to consider whether to specify antenna pointing accuracy for Ku band in UL 14.0–14.5 GHz.

Observation 8: For receiver antenna off-axis performance, EN 302 340, EN 303 980, EN 303 981 and EN 302 977 specify different requirements for different VSATs. Receiver antenna off-axis performance in EN 303 977 and EN 302 340 is same as the requirement for Ka band in TS 38.101-5.

Proposal 6: We need to consider whether to redefine receiver antenna off-axis performance for Ku band in UL 14.0–14.5 GHz.

# 4 Reference

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat et al.

- R4-2315766, NTN enhancements – UE RF requirements, Ericsson

- R4-2402331, NTN Enhancement – VSAT spurious requirement, Ericsson