3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415830

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.19.3.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO on UE initiated BM

Document for:	Discussion

# Introduction

The Rel-19 MIMO WI is approved in [1] and further updated in [2], RAN4 has initiate the discussion in lasting. In this contribution, we further provide our views regarding the RAN4 impacts for the WI.

# Discussion

Based on the discussion in last meeting, RRM impact analysis are as follows:

Issue 1: RRM core impact analysis Agreement: RRM core impact analysis: Topic; Description; RRM impact. UE-initiated/event-driven beam management; UE-initiated/event-driven beam management; YES. CSI enhancement; Type-I codebook refinement supporting up to a total of 128 CSI-RS ports; NO. Type-II codebook refinement supporting up to a total of 128 CSI-RS ports; NO. CRI-based CSI refinement for up to 128 CSI-RS ports; NO. Aperiodic standalone CJT calibration reporting; FFS. 3TX; 3-antenna-port codebook-based UL transmission; FFS. Asymmetric DL sTRP/UL mTRP; FFS.

In the following part, we provide our analysis on RRM impact for UE initiated /event triggered BM.

For Objective#1, it is to introduce UE-initiated/event-driven Beam management to reduce overhead and latency. There is clearly RAN4 RRM impact as agreed in last meeting. In this part, we further introduce the progress of RAN1 discussion.

Event:

Regarding the event, RAN1 first agreed on Event-2. In last meeting, RAN1 achieved following working assumption:

Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

In summary, following Events are supported:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

From measurement behaviour and requirements, we see no significant difference among these Events.

Observation 1: Following events are supported based on RAN1 progress:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

Observation 2: From measurement behaviour and requirements perspective, no significant difference among these Events.

Measurement RS:

RAN1 has also discussed and concluded on the definition of how to determine a current beam and a new beam. According to the agreements so far, current beam is determined is implicitly from a QCL RS of indicated TCI state (QCL RS or SSB QCL-ed with QCL RS). For new beam, at least the explicitly configuration is supported.

Observation 3: For the current beam and new beam determination, at least followings are agreed in RAN1 so far:

- Current beam: Implicitly derived from a QCL RS of indicated TCI state (QCL RS or SSB QCL-ed with QCL RS)

  - FFS one TRS case

- New beam: configured in one RS resource set associated with an CSI reporting configuration

Measurement

Agreement On UE-initiated/event-driven beam reporting, at least support L1-RSRP as a measurement quantity on SSB for intra-cell and inter-cell, and periodic CSI-RS for beam management - Notes: measurement results may be contained in the beam report and/or used as quality metric(s) to initiate/trigger the reporting. - FFS: Semi-persistent CSI-RS and aperiodic CSI-RS. - FFS: Whether/how to support L1-SINR measurement, assuming legacy RS or RS combination (e.g., CMR only, CMR+ZP/NZP-IMR) for Rel-16 SINR is reused. - FFS: Whether/how to specify filtering operation for L1-RSRP. Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification.

Regarding the determination for Event-2, the basic feature is that once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs. An enhanced feature is that beam report occurs only number of Event-2 instance is greater than configured number within a window.

Observation 4: For Event-2, following two triggering determinations are supported:

- Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

RAN1 has also discussed on the reporting procedure and content for UE-initiated/event-driven beam reporting. But from our understanding, it is quite limited impacts on RAN4 RRM impacts.

Then back to open issues in last RAN4 discussion. One of the remaining issues is whether to support/differentiate Mode A and Mode B. From our understanding, Mode A and Mode B discussed in RAN1 is about report procedure, which has less impact on RRM requirements. There is no reason in RAN4 to preclude any reporting procedure.

Issue 2-1-3: Whether/how to define new delay requirements such as measurement reporting delay/measurement period? Agreement: - RAN4 will define Event triggered measurement reporting delay requirement - Further discuss: - Whether to support both mode A and mode B - Whether to differentiate mode A and mode B - Other related aspects

Proposal 1: Mode A and Mode B has minor impact on RRM requirements (e.g. reporting uncertainty). Both Mode A and Mode B are supported, and no differentiation between Mode A and Mode B unless necessary.

Another issue is about whether to filter L1-RSRP, which is shown as follows:

Issue 2-1-8: Whether to filter L1-RSRP? <Way forward> - FFS: RRM requirements for event-triggered L1 reporting without time-window-based filtering or other filtering configured.

In current L1-RSRP requirements, there is no L1 filtering configuration, which is up to UE implementation. But one specific case the following case:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

For RRM requirements perspective, RAN4 should discuss how to define one Event-2 instance. For example, whether it means (One current beam result + one new beam result), or any of them can be considered as one Event-2 instance.

Proposal 2: RAN4 to discuss how to define one Event-2 instance for following Event-2 case:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

# Conclusions

Observation 1: Following events are supported based on RAN1 progress:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality.

Observation 2: From measurement behaviour and requirements perspective, no significant difference among these Events.

Observation 3: For the current beam and new beam determination, at least followings are agreed in RAN1 so far:

- Current beam: Implicitly derived from a QCL RS of indicated TCI state (QCL RS or SSB QCL-ed with QCL RS)

  - FFS one TRS case

- New beam: configured in one RS resource set associated with an CSI reporting configuration

Observation 4: For Event-2, following two triggering determinations are supported:

- Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

Proposal 1: Mode A and Mode B has minor impact on RRM requirements (e.g. reporting uncertainty). Both Mode A and Mode B are supported, and no differentiation between Mode A and Mode B unless necessary.

Proposal 2: RAN4 to discuss how to define one Event-2 instance for following Event-2 case:

- If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs.

  - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam

# References

[1] RP-234007 New WID: NR MIMO Phase 5

[2] RP-242394 WID revision: NR MIMO Phase 5

[3] R4-2414035 WF on RRM requirements for NR_MIMO_Ph5