3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415831

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.19.3.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 MIMO other aspect

Document for:	Discussion

# Introduction

The Rel-19 MIMO WI is approved in [1] and further updated in [2], RAN4 has initiate the discussion in lasting. In this contribution, we further provide our views regarding the RAN4 impacts for the WI.

# Discussion

Based on the discussion in last meeting, RRM impact analysis are as follows:

Issue 1: RRM core impact analysis Agreement: RRM core impact analysis: Topic; Description; RRM impact. UE-initiated/event-driven beam management; UE-initiated/event-driven beam management; YES. CSI enhancement; Type-I codebook refinement supporting up to a total of 128 CSI-RS ports; NO. Type-II codebook refinement supporting up to a total of 128 CSI-RS ports; NO. CRI-based CSI refinement for up to 128 CSI-RS ports; NO. Aperiodic standalone CJT calibration reporting; FFS. 3TX; 3-antenna-port codebook-based UL transmission; FFS. Asymmetric DL sTRP/UL mTRP; FFS.

In the following part, we provide our analysis on RRM impact for other objectives.

## Objective#3

For Obective#3, which is to support UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul. Based on RAN1 agreements so far, following uses cases are considered:

- For per-TRP delay offset reporting: - Use case 1.1: TRP selection - Use case 1.2: delay offset compensation for at least one TRP to ensure the CJT-composite delay spread doesn’t exceed a pre-defined dynamic range/threshold - For per-TRP frequency offset (FO) reporting: - Use case 2.1: TRP selection - Use case 2.2: per-TRP FO compensation at NW side - For per-TRP DL/UL Rx-Tx phase misalignment reporting: - Use case 3.1: TRP selection - Use case 3.2: per-TRP DL/UL Rx-Tx phase compensation at NW side for reciprocity (e.g. using both CSI-RS and SRS for measurement) - Use case 3.3: For TDD reciprocity, timing offset report for at least one pair of TRPs to assist TRP synchronization (i.e. to align TRP inherent timing without propagation delay)

Based on RAN4 discussion, the WF is as follows:

Issue 2-3-1: Whether RRM core requirements impacts exist by UE reporting enhancement for CJT calibration? <Way forward> - Option 1: No RRM core requirements impact. - Option 2: RAN4 to define the measurement delay or measurement behaviour for the aperiodic standalone CJT reporting. Issue 2-3-2: Whether RRM performance requirements impacts exist by UE reporting enhancement for CJT calibration? <Way forward> - Option 1: No RRM performance requirements impact. - Option 2: No accuracy requirements. Potential mapping tables are needed if any. - Option 3: Define accuracy requirements. - Option 4: Define accuracy requirements and report mapping tables.

From our understanding, it is quite similar as Rel-18 TDCP reporting. In Rel-18 MIMO discussion, RAN4 perform feasibility study on whether to define accuracy requirements for TDCP reporting. RAN4 shall discuss whether to define Core requirements for CJT calibration reporting for delay offset/frequency offset/phase offset.

Regarding whether to define core requirements, following aspects shall be considered:

- Whether the measurement RS is TRS or A-TRS: If the measurement RS is A-TRS, there is no choice since it is a one-shot measurement. Based on current RAN1 agreement, it is agreed for D/F CJTA, TRS is used. For P CJTA, there is no restriction.

- Whether there is performance improved for multiple samples measurement. At least for P CJTA, the initial phase at different time instance may not be the same, thus, the filtering may not bring performance gain.

Proposal 1: RAN4 to discuss whether D/F/P CJTA is one-shot measurement. There is no need to define core requirements if it is one-shot measurement.

## Objective#4

For objective#4, the WID is further updated as follows:

From our understanding, it is stated that there is no enhancement on SRS resource. For current SRS AS requirement, the requirements are defined based on SRS resource, regardless of xtxr capability. Thus, we see no significant difference for 3Tx SRS AS.

Observation 1: There is no enhancement on SRS resource as stated in the WID. The SRS AS requirements are defined in terms of SRS resource. Thus, no significant difference for 3Tx SRS AS RRM requirements.

## Objective#5

For objective#5, which to support sTRP/UL mTRP. The enhancement is to support two closed loop PC adjustment with pathloss offset configuration. The WID is updated to include two TA

The status of RRM discussion is as follows:

Issue 2-5-3: Whether RRM impacts exist by 2TA enhancement? <Way forward> - FFS on whether there are RRM impacts by 2TA enhancement. Issue 2-5-4: RRM core impacts of Active uplink TCI state switching delay for unified TCI? <Way forward> - Option 1: The legacy requirements can be reused. - Option 2: The legacy requirements cannot be reused. - Option 3: RAN4 to discussion how to update R17/R18 requirement for asymmetric DL sTRP/UL mTRP scenarios Issue 2-5-5: Whether to define RRM core requirements of pathloss offset update? Agreement: Whether to define RRM core requirements of pathloss offset update? - Not define RRM requirement for MAC CE decoding delay. - The agreement can be revisited depending on further RAN1 progress.

Regarding the TCI state switching, RAN1 has following conclusions:

Conclusion For the asymmetric DL sTRP/UL mTRP deployment scenario, reuse the rel-17 unified TCI/ICBM and rel-18 unified TCI framework: - When rel-17 unified TCI/ICBM is configured: - For FR1: one joint TCI state or {one DL TCI state + one UL TCI state} can be applied. - For FR2: {one DL TCI state + one UL TCI state} can be applied. - When rel-18 unified TCI is configured: - For FR1: up to two joint TCI states or {one DL TCI state + up to two UL TCI state} can be applied. - For FR2: {one DL TCI state + up to two UL TCI states} can be applied.

For Rel-17 UTCI FR1: since the UL TCI only work for PL, NW can use joint TCI with different PL offset, or separate DL + separate UL.

For Rel-17 UTCI FR2: For one particular TRP, NW can not use joint TCI, since the DL RS and UL (SRS) are different. Thus, only sperate TCI can be used since UE can only be configured with one type of TCI.

For Rel-18 UTCI FR1: Compared as Rel-17 UTCI, Rel-18 UTCI can have another layer of indication to indicate one of UTCI or both of them. Thus, UE can be configured with to joint TCI or one DL + 2 UL.

For Rel-18 UTCI FR2: Similar as Rel-17 UTCI, UE can only be configured with sperate TCI. But now it can be configured with 2 UL TCI, since Rel-18 UTCI can support indication one of them or both.

Based on the analysis above, for FR2, not matter whether it is Rel-17 UTCI or Rel-18 UTCI, SRS has to be the QCL source for beam. Based on current RRM requirements principles, there is no RRM requirement for SRS based TCI states switching, since it relies on beam correspondence and no RRM behavior/performance to be verified.

Observation 2: For FR2, not matter whether it is Rel-17 UTCI or Rel-18 UTCI, SRS has to be the QCL source for beam.

Proposal 2: No RRM requirement for TCI state switching for asymmetric DL sTRP/UL mTRP for FR2.

For FR1, we believe current requirement can apply, but the only difference is whether to define RRM requirements for TCI state switching for PL offset change. From our understanding and also based on discussion in last RAN4 meeting, it is more about MAC CE decoding delay and RF requirements. Thus, we propose not to define RRM requirements for PL offset change.

Proposal 3: No RRM requirements (e.g. TCI state switch, PL switching) for PL offset changes.

For 2 TA case, as stated in the updated WID, it is to reuse Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured. In Rel-18 MIMO CR, it seems existing description can also fit. But, RAN4 can wait for more RAN1 progress to see whether there is any updating needed.

For multi-DCI based multi-TRP operation with two TAs, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [TS 38.331] having the same TAGas the uplink signal.

Proposal 4: For two TA in asymmetric DL sTRP/UL mTRP, RAN4 to wait for more progress to check whether there is necessary updating on RRM requirements

# Conclusions

Proposal 1: RAN4 to discuss whether D/F/P CJTA is one-shot measurement. There is no need to define core requirements if it is one-shot measurement.

Observation 1: There is no enhancement on SRS resource as stated in the WID. The SRS AS requirements are defined in terms of SRS resource. Thus, no significant difference for 3Tx SRS AS RRM requirements.

Observation 2: For FR2, not matter whether it is Rel-17 UTCI or Rel-18 UTCI, SRS has to be the QCL source for beam.

Proposal 2: No RRM requirement for TCI state switching for asymmetric DL sTRP/UL mTRP for FR2.

Proposal 3: No RRM requirements (e.g. TCI state switch, PL switching) for PL offset changes.

Proposal 4: For two TA in asymmetric DL sTRP/UL mTRP, RAN4 to wait for more progress to check whether there is necessary updating on RRM requirements

# References

[1] RP-234007 New WID: NR MIMO Phase 5

[2] RP-242394 WID revision: NR MIMO Phase 5

[3] R4-2414035 WF on RRM requirements for NR_MIMO_Ph5