3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415833

Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.22.2.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on RRM impact for Rel-19 NES on adpatation

Document for:	Discussion

# Introduction

For Rel-19 NES, RAN 4 initiate the discussion in last meeting with agreements and issues captured in [1]. In this paper, we provide our views on remaining issues with further progress in other WGs considered.

# Discussion

## Adaptation of common signal/channel transmission

For adaption of PRACH and paging, RAN4 achieved following agreements in last meeting.

- Sub-topic 3-2: Impact on Paging Adaptation requirements Issue 3-2-1: Paging adaptation Agreement: - Deprioritize Paging adaptation until the RAN1 concrete conclusion is available. - Sub-topic 3-3: Impact on PRACH Adaptation requirements Issue 3-3-1: PRACH adaptation Agreement: - Deprioritize PRACH adaptation until the RAN1 concrete conclusion is available.

Based on the progress in RAN1/2 so far, the conclusion about paging adaptation is still not concrete enough. For PRACH adaptation discussed in RAN1, the enhancement is about additional PRACH configuration, which could be dynamically changed by DCI indication. From RRM requirement perspective, there is no particular requirements on Paging/PRACH performance. We see no significant RRM impacts for Paging/PRACH adaptation.

Observation 1: There is no significant RRM requirements impacts for PRACH/Paging adaptation.

For SSB adaptation, RAN4 achieved following agreements in last meeting

- Sub-topic 3-1: SSB Adaptation requirements Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode - Agreement - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode. - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

It could be observed from existing RAN1 progress that the discussion is mainly about the clarification about scenarios.

Observation 2: Based on RAN1 agreements so far, it could be observed that:

- At least adaptation of SSB periodicity is supported.

- For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported.

- For Rel-19 NES-capable UE’s SCell, followings are supported:

  - NCD SSB on sync raster

  - NCD SSB off sync raster

However, some detailed design is still needed to investigate the RRM impacts, e.g. the triggering method of SSB adaptation, or whether there are other adaptation mechanisms. It is still beneficial to proceed RAN4 discussion to identify the potential RAN4 impacts.

For all L1/L3 measurement, it could be further divided into following two case:

- Case 1: SSB based measurement based on SSB periodicity

- Case 2: SSB based measurement based on SMTC

For instance, RLM/BFD/CBD/L1-RSRP are performed based SSB periodicity (with necessary relaxation. E.g. when DRX is configured). For case 2, for instance, L3 measurement defined in section 9, UE perform L3 measurement based on SMTC configured to the UE. Thus, since the SSB adaption is not that dynamic, and it could happen for a neighbour cell, it is reasonable that UE just perform measurement based on configured SMTC. In particular, if there is a cell, of the which is SSB periodicity is changed from {20.40,80}, it is straightforward that NW configured a SMTC with periodicity as 80 ms, thus there is no impact on measurement behaviour.

For L3 measurement, one special case is the L3 measurement during SCell activation. In Rel-18 RRM enhancement, RAN4 introduced “shortMeasInterval-r18” that UE use SSB periodicity instead of SMTC periodicity if it is only configured in MO. For SSB adaptation, it is very likely that the SSB periodicity is changed (e.g. from 40 ms to 20 ms) before SCell activation, and it is straightforward that UE just uses the new SSB periodicity for SCell activation. Because for this case, it is not clear how to configure the SMTC in servingCellConfig.

Proposal 1: After SSB adaptation, for SSB based measurement based on SSB periodicity, UE shall perform measurement based on “new SSB configurations” (e.g. new SSB periodicity)

Proposal 2: After SSB adaptation, for SSB based measurement based on SMTC:

- For L3 measurement in clause 9, UE is still expected to perform measurement based on configured SMTC.

- For L1 and L3 measurement during SCell activation, UE shall perform SCell activation based on “new SSB configurations” (e.g. new SSB periodicity).

The above analysis is about after SSB adaptation. More complicated the case is the transaction of the SSB adaptation, or the impact to RRM requirements when SSB adaptation happens. From our understanding, it is not that frequently happens, RAN4 could further discuss, whether and how to define requirements when SSB adaption happens during RRM procedures.

Proposal 3: RAN4 to discuss whether and how to define requirements when SSB adaption happens during RRM procedures.

# Conclusions

Observation 1: There is no significant RRM requirements impacts for PRACH/Paging adaptation.

Observation 2: Based on RAN1 agreements so far, it could be observed that:

- At least adaptation of SSB periodicity is supported.

- For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported.

- For Rel-19 NES-capable UE’s SCell, followings are supported:

  - NCD SSB on sync raster

  - NCD SSB off sync raster

Proposal 1: After SSB adaptation, for SSB based measurement based on SSB periodicity, UE shall perform measurement based on “new SSB configurations” (e.g. new SSB periodicity)

Proposal 2: After SSB adaptation, for SSB based measurement based on SMTC:

- For L3 measurement in clause 9, UE is still expected to perform measurement based on configured SMTC.

- For L1 and L3 measurement during SCell activation, UE shall perform SCell activation based on “new SSB configurations” (e.g. new SSB periodicity).

Proposal 3: RAN4 to discuss whether and how to define requirements when SSB adaption happens during RRM procedures.

# References

[1] R4-2413885 WF on Netw_Energy_NR_enh

[2] RP-242354 Revised WID: Enhancements of network energy savings for NR

[3] R1-2407565 LS on timeline for On-demand SSB operation on SCell