3GPP TSG-RAN WG4 Meeting # 112bis	R4-2415834

# Hefei, Anhui, China, 14th – 18th October, 2024

Agenda item:	6.15.2.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The enhancement on FR2-1 L3 measurement by optimizing Rx beam sweeping factor was further discussed in last meeting with agreements and issues captured in [1]. In this paper, we further provide our views on remaining issues.

# Discussion

Based on the discussion in last meeting, one controversial issue is what is the searcher assumption to enable this feature. The status is summarized as follows:

Issue 1-2-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier [Way forward]: FFS: - Option A/1a: Two searchers are occupied by this single carrier to the requirements of enhanced BSF for single carrier. Discuss CA case later. (HW, CATT, Apple, OPPO, LGE, Xiaomi) - Option B: Consider one searcher for single carrier. (MTK, ZTE, OPPO, QC, Nokia, Intel, vivo, Ericsson, Apple, Xiaomi, CATT) Issue 1-2-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC [Way forward]: FFS: - Option 1 (CATT, vivo): No need to add additional processing time due to multiple SSBs within one SMTC. - Option 2 (Apple): allow additional processing time for UE supporting multiple-Rx simultaneous reception for L3 delay enhancement if there is only one searcher available for the processing - Option 3 (Ericsson): Extra SSB post-processing time in several ms, as side effect of BSF enhancement, may be acceptable, but it depends on how much we can gain with BSF enhancement - Option 4 (Intel): - Consider UE baseband processing capabilities when specifying the L3 delay reduction for simultaneous receptions on multiple FR2 SSB-s. - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Option 5 (Nokia): - If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms

At the very beginning of the discussion, we supported to take 2 searchers as the basic assumption thus we could avoid complicated discussion about UE implementation to support this feature, e.g. re-design of CSSF. And we believe it should be the very baseline implementation as all companies agreed with the agreement in the first meeting that we consider one target carrier and one serving carrier as starting point.

Observation 1: Two searcher assumption shall be the baseline implementation/solution to enable this feature, due to which RAN4 agreed on the applicable scenario at beginning with one target carrier and one serving carrier as starting point.

However, companies propose another solution that UE can use one searcher with post processing time to support this feature. The benefit is apparent that it is easy to be supported in CA case, where PCell will occupy one searcher all the time. However, as argued in last meeting, the exact meaning of “one searcher with post processing” is not clear to us. The fact is that “searcher” is the logic term being used in discussion. Actually, there are different components to support cell search in UE implementations (e.g. buffer, processer). Whether the searcher is equivalent to processor or all relevant components are not aligned among companies.

Observation 2: From implementation perspective, there is no aligned understanding about the definition of “searchers” being used in discussion.

Thus, even we agree on either “two searchers” or “one searcher with post processing”, it does not give clear guidance on how to implement the feature. The essential part of the discussion is about whether and how to support this feature in CA mode instead of what is the searcher assumptions. Thus, we suggest to skip the discussion about the searcher assumption, and whether and how to support this feature in CA can be up to UE capability discussion. In details, the capacity of “Cell search” maybe different with different serving cells configurations. We think more flexibility on UE capability indication can make it easier for UE to support this feature.

Observation 3: The essential part is whether and how to support this feature in CA case.

Proposal 1: RAN4 to skip the discussion about the searcher assumption to support FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. And whether UE can support this feature can be considered with UE capability discussion.

Another bunch of issues are related to the applicability conditions of the features, which are shown as follows:

Issue 1-2-5: FFS: Power consumption issue (including conditions to trigger UE using FBS for L3 measurement) - Option 1 (Apple, LGE): For power saving purpose, there is a need to have a mechanism to activate/de-activate L3 fast beam sweeping. The R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline, while other conditions are not precluded. - Option 2 (HW): - RAN4 shall firstly identify the promising scenario(s) for L3 measurement delay reduction enabled by multi-Rx with clear/significant benefits, which could help to converge the discussion. - One possible scenario to be considered is when there is strong demand of mobility performance (e.g. UE at cell edge or the link is about to break). - It shall not be assumed that UE supporting this feature shall activate multiple panels all the time for all L3 measurement. RAN4 shall discuss the entering and/or exiting conditions for enhanced L3 measurement enabled by multi-Rx taking the targeting scenario into account. - Option 3 (Ericsson/Nokia): NW indicates UE enabling/disabling FBS through L3 or lower layers signaling. - Option 4 (vivo): Power consumption issue is important and BSF reduction of L3 measurement will not trigger UE to activate multi-Rx - Option 5 (Intel): Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced. Whether UE has prior knowledge or cell centre/edge conditions do not affect reduction in BSF but they are addressed in legacy side conditions. - Option 6a (Nokia): Mobility Event triggering BSF reduction - Option 6b (Nokia): FBS is triggered by conditional Handover configuration - Option 6c (Nokia): BSF reduction is always enabled, but used for reduced measurement delay in cell edge and used for reduced scheduling restrictions in cell center (e.g. by extending T_SMTC). - Option 7 (QC): RAN4 to adopt the following framework for the fast UE Rx beam sweeping based L3 measurement and mobility requirements: - NW provides the following criteria for fast beam sweeping application, and the signaling details are FFS - Cell edge condition: Threshold value of absolute L3 SSB-RSRP of SpCell - High speed condition: Threshold value of L3 SSB-RSRP variation on SpCell over a time period T - When the condition of not cell-edge (and not high-speed, if configured) is met, the UE is allowed to fallback Rx beam sweeping factor to the existing N value - Note: the existing criteria defined for the relaxed idle/inactive mode measurement and/or RLM/BFD evaluation can be reused or served as a baseline - Report configuration for the status of fast beam sweeping factor application, and the signaling details are FFS - A TTT-like time window or N310-like timer, which starts running or counting upon the first satisfaction of the condition “not cell-edge (and not high-speed)” is observed by the UE, can be configured to avoid frequent status transitions and reports - Note: the existing report defined for RLM/BFD relaxation status can be reused or served as a baseline - Besides, other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed. - FFS on the application delay of UE Rx beam sweeping factor switch. - Option 8 (MTK): Activating multi-Rx for L3 measurements (intra/inter-frequency) may or may not be always necessary, depending on UE current conditions: - UE location (cell centre or cell edge) - UE mobility (stationary or moving) - Both above - Option 8a (MTK): - On UE mobility status, RAN4 should consider low/medium speed mobility of the UEs as one of the conditions to activate multi-Rx for L3 measurement delay reduction. - RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 9 (ZTE): Due to L3 measurement is long-term operation, power consumption issue could be considered, which may lead to some interaction signalling. But which would not be the applicability condition of applying fast beam sweeping. [Way forward]: discussion can be mainly focus on 3 directions: (1)“additional triggering for this R19 L3 measurement with FBS” (use option 8 for discussion) or (2)“L3 measurement with FBS can be activated/deactivation following on R18 mechanism” (use option 1 for discussion) or (3)“up to network indication” (use option 3 for discussion). Discuss option 1/3/8 together, and then add details from other options if needed. Issue 1-2-8: FFS: cell-centre UE or cell-edge UE [Way forward]: this issue can be discussed in issue 1-2-5. No more duplicated discussion in issue 1-2-8.

As analysed in last meeting, from our understanding, one promising scenario is to speed up the L3 measurement delay when there is strong demand of mobility. For instance, UE is in the edge of the cell or the link is about to broken, it is worthy to enable multi-RX for L3 measurement with the additional efforts and power consumption.

As mentioned above, for instance, if UE is at cell edge, it is most likely that handover is about to happen and gNB is expecting timely L3 report to trigger the handover and to provide the neighbour cell information as target Cell. If the L3 measurement delay could be reduced, UE may better experience in terms of smooth handover. Another example is that when the quality of the link is becoming worse, UE shall try to find target Cells for potential handover as soon as possible. It could be observed that the situation mentioned above is some temporary state, and it is acceptable that UE only enable multiple panel for L3 measurement instantly which may not last for too long time.

Observation 4: UE may only enable multiple panel for L3 measurement instantly which may not last for too long time, when the additional efforts and power consumption maybe acceptable.

Thus, we proposed to consider entering and exiting conditions to guarantee that UE is not forced to activating multiple panels for L3 measurement for long time, which will cause sever power consumption. As mentioned about, one promising scenario is when UE is at cell edge or the link to the existing serving cell is about to break, UE shall active multiple panels to measure neighbour cells for as candidate target cells for HO/SCell addition. Thus, if UE has completed measurement and reported qualified results to NW, which means NW already has the information about candidate Cells for potential HO/SCell addition, thus the urgency of fast L3 measurement is diminished. We think it is reasonable that UE can switch back to normal measurement behaviour (i.e. single panel activated).

Proposal 2:  For scenario 1/2/3/4 (L3 measurement in connected mode), define at least following applicability conditions to avoid endless multi-panel L3 measurement which will cause sever power consumption:

- UE is only required to perform multi-panel L3 measurement when the serving cell is deteriorating (e.g. the link to the serving cell is about to break).

- UE can switch back to normal measurement (i.e. single panel activated) when UE has reported qualified results.

For Scenario 5/7 (HO and RRC re-establishment/RRC release with redirection), which is different from L3 measurement in connected mode, there are not long-term RRM activities. There is no power consumption issues for this one-time procedure.

Proposal 3: No need to define additional conditions for power consumption issue for Scenario 5/7.

# Conclusions

Observation 1: Two searcher assumption shall be the baseline implementation/solution to enable this feature, due to which RAN4 agreed on the applicable scenario at beginning with one target carrier and one serving carrier as starting point.

Observation 2: From implementation perspective, there is no aligned understanding about the definition of “searchers” being used in discussion.

Observation 3: The essential part is whether and how to support this feature in CA case.

Proposal 1: RAN4 to skip the discussion about the searcher assumption to support FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. And whether UE can support this feature can be considered with UE capability discussion.

Observation 4: UE may only enable multiple panel for L3 measurement instantly which may not last for too long time, when the additional efforts and power consumption maybe acceptable.

Proposal 2:  For scenario 1/2/3/4 (L3 measurement in connected mode), define at least following applicability conditions to avoid endless multi-panel L3 measurement which will cause sever power consumption:

- UE is only required to perform multi-panel L3 measurement when the serving cell is deteriorating (e.g. the link to the serving cell is about to break).

- UE can switch back to normal measurement (i.e. single panel activated) when UE has reported qualified results.

Proposal 3: No need to define additional conditions for power consumption issue for Scenario 5/7.

# References

[1] R4-2410259 WF for [111][205] NR_RRM_enh3_part1