3GPP TSG-RAN WG4 Meeting # 112bis		                                          R4-2415838

Hefei, China, 14th - 18th October, 2024

Title: 					Discussion on FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC

Source: 	Huawei, HiSilicon

Agenda item:	6.15.2.2

Document for:	Discussion

Introduction

The RRM Phase 5 FR2-1 L3 measurement delay by optimising CSSF outside gap in CA/DC discussion, as outlined in WF[1], was one of the topics during RAN4 #112. Key contributions from various companies are detailed in WF[1]. In this paper, we present our perspectives and contributions regarding FR2-1 L3 measurement delay, focusing on optimising the CSSF outside gap in CA/DC scenarios.

Discussion

UE measurement procedure

In the last meeting, there was an agreement in the WF[1] captured in the following. This conclusion provided three aspects, each of these aspects is one vital component of the CSSF outside gap spec.

Agreement in online session: - The following aspects in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be discussed, and further prioritization among the 3 aspects can be discussed in future meeting: - Aspect 1: SSB based Intra-frequency measurement without MG, including: - TPSS/SSS_sync_intra and TSSB_measurement_period_intra - CSSFintra for intra-frequency measurement without gap which is defined since Rel-15 - Aspect 2: SSB based Inter-frequency measurement without MG, including: - TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - CSSFinter for inter-frequency measurement without gap. - Aspect 3: Inter-RAT SSB measurement without MG, including: - CSSFinterRAT for inter-RAT measurement without gap if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement. - MG related features to be considered in aspect 1/2/3 including: - R16 Inter-frequency measurement without gap where SSB is completely contained in active BWP - R17 NCSG measurement with ‘nogap-noncsg’ - R18 NeedForGaps measurement with ‘no-gap-no-interruption’ or with “no-gap-with-interruption”

For the enhancement of CSSF, all the aforementioned aspects are integral and equally crucial components. There are two points for this is detailed as follows:

- Prioritising any single part would necessitate the creation of individual specification content, which could complicate future maintenance and expansion.

- Given that the solution has not yet been fully discussed or finalised, isolating and prioritising one aspect may not align with potential solutions that could arise.

Proposal 1: The optimisation of CSSF for L3 measurement delay reduction in CA/DC should take into account all three aspects.

Searcher assumptions

In the last meeting, we discussed the assumption of the searcher number. Companies have different views and understandings on this sub-topic, and two main directions are finally recorded in the WF[1].

[Way forward]: FFS: - Option 1 (CATT): Not to preclude the solutions based on 3 searchers assumption in current stage. - Option 1a (CMCC, HW): it is proposed to consider both the CSSF optimization based on 2 searchers and the CSSF optimization based on 3 searchers. - Option 1b (QC): RAN4 to consider introducing a new UE optional capability regarding the number of cell search/L3-measurement engines for CSSF enhancement, with the details to be finalized (FFS). - Option 2 (Apple, CTC, Nokia, Ericsson): RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF. - Option 2a (Apple): If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. - Option 2b (Nokia) RAN4 to confirm if one of the searchers is assumed for PCC/PSCC measurement and the other is assumed for the measurements on all SCCs.

There are two kinds of searcher assumptions available as options: the baseline assumption, as stated in the WID, is 2 searchers, while the other is 3 searchers. We continue to propose the use of 3 searchers for the FR2-1 L3 measurement enhancement under specific conditions. The reason and technology considerations will be shown in the following. Based on the analysis, it is evident that in certain legacy scenarios, the processing capability of the UE already matches that of 3 searchers. Here is an example for explanation.

When one UE performs measurements within the Gap in one FR and another UE conducts measurements outside the Gap in a different FR (Please avoid delving into the specifics of the assumptions; this scenario is intended to illustrate not all MOs are outside the Gap at one moment), it becomes apparent that three searchers are operating simultaneously. Subsequently, if all MOs are outside the Gap, one searcher remains idle without any MOs assigned. All of this aligns with the searcher definitions as outlined in the legacy specification.

Figure 1. Measurement tasks in different situations.

Note: The Gap Searcher refers to the UE's measurement capability during the gap.

If we utilise the Gap Searcher for measurements while it is idle, the UE can employ its three searchers to conduct measurements outside the Gap. To highlight that no additional searchers were added for measurement purposes, which indicates that the processing capability of the UE remains unchanged.

Obversion 1: The UE can support three searchers in certain situations without any enhancement to its processing capability.

Figure 2. Measurement outside gap improvement.

Proposal 2: RAN4 to consider both 2-searcher and 3-searcher assumptions.

Proposal 3: The assumption of three searchers can be applied under specific conditions. The details of the conditions can be FFS.

Proposal 4: RAN4 RRM can define a new UE capability to indicate whether the UE supports up to three searchers.

Candidate Solutions

In this section, the approach for utilising the CSSF outside the gap will be presented from two perspectives: the 2-searcher solution and the 3-searcher solution.

2-searcher solution

The following content has been extracted from the last meeting’s WF[1].

[Way forward]: FFS: - Option 1 (Apple, CTC. Ericsson, Intel, Samsung, ZTE): UE only needs to measure one serving CC per band if multiple serving CCs are in the same band - Option 2 (CATT, Apple, CTC, ZTE): UE can change the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions - Option 4 (CATT, HW): - For UE supporting per FR gap, when all MOs are to be measured outside gap, the searcher used for within gap can be leveraged for outside gap. CSSFoutsidegap can be optimized, as these MOs can share totally three searchers. (HW, CATT) - We are open to further discuss the applicable condition of the three searcher solution and whether a new capability is needed. (HW)

The first assumption is the use of two searchers. We propose a method to optimise the CSSF outside the gap, the PCell shares its searcher with one or multiple SCells. Essentially, this approach enables the searcher occupied by the PCell to handle a portion of the Scell MOs to reduce the measurement delay for the SCell slightly. While this solution aligns conceptually with Option 2 in the Way forward, it represents a fixed PCell MO configuration, rather than a dynamic adjustment method of the PCell searcher occupancy.

The following table is an example of the solution which PCell share its occupied searcher with one SCell. We used the CSSF spec in version Rel-15 to discuss and explain the improvement.

Proposal 5: To facilitate the discussion of this issue, we recommend utilising the Rel-15 version of the CSSF as a baseline for solution discussion. Once the solution is settled, we can then explore the incorporation of the features introduced in Rel-16, Rel-17 and Rel-18 into CSSF table.

Table 1. Example 1 of CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR2 only intra band CA | N/A | N/A | 1 + 1 | N/A | Number of configured FR2 SCell(s) - 1 |

The primary objective of this solution is to speed up the measurement of SCells by reducing the measurement delay associated with one SCell. However, it is important to note that this will increase the measurement time for the PCell. Considering how crucial PCell measurements are, the implementation of this solution will be released upon specific conditions, which we can discuss further.

Table 4. Example 2 of CSSFoutside_gap,i scaling factor for SA mode

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | Number of configured SCell(s) – 1 | 1 + 1 | N/A | Number of configured SCell(s) – 1 |

In the FR1 + FR2 scenario, the 2-searcher solution effectively reduces the measurement delay for all SCells by one SCell measurement delay. The FR2 SCell measurement delay will have a decrease in this example scenario.

Proposal 6: To facilitate the reduction of FR2-1 L3 measurement delay, we propose that the PCell share its occupied searcher with one SCell MO. The specific conditions under which this solution can be implemented are open for further discussion.

3-searcher solution

Secondly, we will address the solution of 3-searcher assumption and its performance.

Table 3. Example 1 of CSSFoutside_gap,i scaling factor for SA mode 3 searchers solution

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR2 only intra band CA | N/A | N/A | 1 | N/A | Number of configured FR2 SCell(s) / 2 Note 1 |
| Note 1: When the number of configured FR2 SCells is odd and larger than 1 this value is equal to (Number of configured FR2 SCells – 1) / 2 + 1. If only one SCell is configured this value is equal to 1. |  |  |  |  |  |

The measurement delay for the SCell can be reduced by half when utilising 3-searcher solution (how UE got these three searchers is already explained above). The PCell occupies one searcher, while the SCell utilises the remaining searcher as well as the searcher that was originally designated for gap measurements, the ‘Gap Searcher’ mentioned above.

Table 4. Example 2 of CSSFoutside_gap,i scaling factor for SA mode 3-searcher solution

| Scenario | CSSFoutside_gap,i for FR1 PCC | CSSFoutside_gap,i for FR1 SCC | CSSFoutside_gap,i for FR2 PCC | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is required | CSSFoutside_gap,i for FR2 SCC where neighbour cell measurement is not required |
| --- | --- | --- | --- | --- | --- |
| FR1 +FR2 CA (FR2 PCell) Note 1 | N/A | Number of configured SCell(s) / 2 | 1 | N/A | Number of configured SCell(s) / 2 Note 1 |
| Note 1: When the number of configured SCells is odd and larger than 1 this value is equal to (Number of configured SCells - 1) / 2 + 1. If only one SCell is configured this value is equal to 1. |  |  |  |  |  |

Example two illustrates a scenario involving combined CA with FR1 and FR2. In this case, when all MOs are outside the gap, the measurement delay for the FR2 SCell is reduced by 50%. The PCell utilises one searcher, while the remaining searcher and the third searcher which is referred to as the 'Gap Searcher' are shared between the FR1 and FR2 SCells.

For the 3-searcher scenario, the condition is when all MOs are situated outside the gap. In this instance, the third searcher, referred to as the ‘Gap Searcher’ can be activated, as previously discussed.

Proposal 7: The 3-searcher solution can be utilised under the following conditions

- When all MOs need to be measured outside the gap.

- Other conditions can be FFS.

Feature capability

Since the solutions are still under discussion, the new capability will need to be defined at a later stage.

Proposal 8: Discussion on the new UE capability is supposed to be addressed later.

Conclusions

Proposal 1: The optimisation of CSSF for L3 measurement delay reduction in CA/DC should take into account all three aspects.

Obversion 1: The UE can support three searchers in certain situations without any enhancement to its processing capability.

Proposal 2: RAN4 to consider both 2-searcher and 3-searcher assumptions.

Proposal 3: The assumption of three searchers can be applied under specific conditions. The details of the conditions can be FFS.

Proposal 4: RAN4 RRM can define a new UE capability to indicate whether the UE supports up to three searchers.

Proposal 5: To facilitate the discussion of this issue, we recommend utilising the Rel-15 version of the CSSF as a baseline for solution discussion. Once the solution is settled, we can then explore the incorporation of the features introduced in Rel-16, Rel-17 and Rel-18 into CSSF table.

Proposal 6: To facilitate the reduction of FR2-1 L3 measurement delay, we propose that the PCell share its occupied searcher with one SCell MO. The specific conditions under which this solution can be implemented are open for further discussion.

Proposal 7: The 3-searcher solution can be utilised under the following conditions

- When all MOs need to be measured outside the gap.

- Other conditions can be FFS.

Proposal 8: Discussion on the new UE capability is supposed to be addressed later.

References

- RP-2414026 WF for NR_RRM_Ph5_Part1.

- RP-242380 NR Radio Resource Management (RRM) Phase 5.